Software Radio PIM & PSM

THALES Contribution to

Joint Submission

( To Be Included as a replacement for Annexes C & D )

Table of contents

1 - Object	7

2 - Radio System Modeling Elements	8

2.1 - Actors	8

2.2 - Component Breakdown	9

2.3 - Radio Set states	9

3 - Radio Set Use Cases	12

3.1 - Transmit / Receive	12

3.1.1	Transmit Voice (non-packet)	13

3.1.2	Receive Voice (non-packet)	14

3.1.3	Transmit Data Stream (non-packet)	16

3.1.4	Receive Stream Data (non-packet)	18

3.1.5	Transmit Packet	20

3.1.6	Receive Packet	21

3.2 - Scanning	22

3.3 - Relaying	22

3.3.1	Repeating	22

3.3.2	Bridging	23

3.3.3	Networking	23

3.3.4	Internetworking	24

3.4 - Control	24

3.4.1	Radio Control	25

3.4.1.1	Startup	25

3.4.1.1.1	Power-up and Initialize	25

3.4.1.1.2	System Reset	27

3.4.1.2	Shutdown	28

3.4.1.2.1	Loss of Power	28

3.4.1.2.2	Commanded Shutdown	29

3.4.1.3	Radio Silence	30

3.4.1.4	Radio Identification	31

3.4.1.5	Radio time	31

3.4.1.6	Control Communication Channel	31

3.4.1.6.1	Change Comm Channel Transmission Mode	32

3.4.1.6.2	Change Comm Channel Administrative Mode	33

3.4.1.6.3	Configure Comm Channel	33

3.4.1.6.4	Change Comm Channel Security Mode	34

3.4.1.6.5	List Comm Channels	34

3.4.1.6.6	Instantiate Comm Channel	35

3.4.1.6.7	Uninstantiate Comm Channel	35

3.4.2	Waveform Control	36

3.4.2.1	Change Frequency	36

3.4.2.2	Change QoS	36

3.4.2.3	Power Control	36

3.4.2.4	Antenna Management	36

3.4.2.5	Bandwidth management	37

3.4.3	Spectrum Control	37

3.4.3.1	Cosite	37

3.5 - Administration	37

3.5.1	Run Diagnostics	37

3.5.1.1	Enable Background BIT	38

3.5.1.2	Initiate System BIT	38

3.5.1.3	Initiate Channel BIT	38

3.5.1.4	View Fault Log	39

3.5.2	Manage Time	39

3.5.2.1	Provide System Synchronization	39

3.5.2.2	Provide Time to Waveforms	40

3.5.2.3	Synchronize to External Time Source	40

3.5.3	Performance Management	41

3.5.3.1	Create PM Status Report	41

3.5.3.2	Start PM Status Reporting	42

3.5.3.3	Stop PM Status Reporting	42

3.5.3.4	Delete PM Status Report	43

3.5.3.5	Create PM Threshold Report	43

3.5.3.6	Start PM Threshold Reporting	43

3.5.3.7	Stop PM Threshold Reporting	44

3.5.3.8	Delete PM Threshold Report	44

3.5.3.9	Create PM On Change Report	45

3.5.3.10	Start PM On Change Reporting	45

3.5.3.11	Stop PM On Change Reporting	46

3.5.3.12	Delete PM On Change Report	46

3.5.3.13	Create Intermediate Calculation	46

3.5.3.14	Start Intermediate Calculation	47

3.5.3.15	Stop Intermediate Calculation	47

3.5.3.16	Delete Intermediate Calculation	48

3.5.4	Monitor Radio System	49

3.5.4.1	Provide General Statistics	49

3.5.4.2	Provide OE Statistics	50

3.5.4.3	Handle Special Monitoring Activity	51

3.5.4.4	Provide Waveform Status	52

3.5.4.5	Alarms	52

3.5.4.5.1	Change Alarm Operational Mode	53

3.5.4.5.2	List Alarms	53

3.5.4.5.3	Modify Alarm	53

3.5.4.5.4	Trigger Alarm	54

3.5.4.6	Log Historical Data	54

3.5.4.7	View Historical Data	55

3.5.4.8	Register to Receive Radio Events	55

3.5.4.9	Notify Radio Event	56

3.5.4.10	Unregister to Receive Radio Events	56

3.5.5	Manage Configuration	57

3.5.5.1	Manage Physical Configuration	57

3.5.5.1.1	Install New Hardware Driver(s)	57

3.5.5.1.2	List Hardware	59

3.5.5.1.3	Change Comm Hardware Administrative State	60

3.5.5.1.4	Add Hardware	61

3.5.5.1.5	Set Hardware Configuration	61

3.5.5.1.6	Get Hardware Configuration	62

3.5.5.1.7	Remove Hardware	62

3.5.5.1.8	Perform Hardware BIT	63

3.5.5.1.9	Enable Hardware	63

3.5.5.1.10	Disable Hardware	63

3.5.5.1.11	Determine Co-Site Potential	64

3.5.5.2	Manage Software Configuration	65

3.5.5.2.1	Initiate Local Software Download	66

3.5.5.2.2	Initiate Solicited OTAP Software Download	68

3.5.5.2.3	Initiate Unsolicited OTAP SW Download	70

3.5.5.2.4	Transmit OTAP	71

3.5.5.2.5	Application Management	71

3.5.5.2.5.1	Uninstall Application	71

3.5.5.2.5.2	Remove Application	72

3.5.5.2.5.3	List Installed Applications	73

3.5.5.2.5.4	List Device Drivers	74

3.5.5.3	Configure Radio	75

3.5.5.3.1	Get Radio Configuration	75

3.5.5.3.2	Set Radio Configuration	76

3.5.5.3.3	Communications Channel Control	77

3.5.5.3.3.1	Establish Waveform Communication Channel	77

3.5.5.3.3.2	Release Waveform Communication Channel	79

3.5.5.3.4	Configure Waveform	80

3.5.5.3.4.1	Set Waveform Parameters	81

3.5.5.3.4.2	Start Waveform	82

3.5.5.3.4.3	Stop Waveform	83

3.5.5.3.5	Configure Retransmission	84

3.5.5.3.5.1	Configure Repeater	84

3.5.5.3.5.2	Configure Bridge	84

3.5.5.3.5.3	Configure Router	85

3.5.5.3.6	Configure Gateway	85

3.5.5.3.7	Determine Cosite Potential	85

3.5.5.3.8	Deployment Plan	86

3.5.5.3.8.1	List Deployment Plans	86

3.5.5.3.8.2	Enable Deployment Plan	87

3.5.5.3.8.3	Disable Deployment Plan	87

3.5.5.3.8.4	Add Deployment Plan	88

3.5.5.3.8.5	Remove Deployment Plan	88

3.6 - Manage Security	89

3.6.1	Manage Operator Profile	90

3.6.1.1	Define Operator	90

3.6.1.2	Modify Operator	91

3.6.1.3	Delete Operator	92

3.6.1.4	Lock Operator	93

3.6.1.5	Unlock Operator	94

3.6.2	Manage Security Threats	95

3.6.2.1	Detect Physical Tamper	96

3.6.3	Manage Keys	97

3.6.3.1	Load Keys	97

3.6.3.2	Initiate Software Zeroize of the Radio Set	98

3.6.3.3	Initiate Software Zeroize of a Specific Waveform	99

3.6.3.4	Initiate Information Fill	100

3.6.3.5	Over the Air Rekey	101

3.6.3.6	Over the Air Zeroize	102

3.6.3.7	Over the Air Transfer of Keys	104

3.6.3.8	Initiate Algorithm Load	105

3.6.4	Security Policy	106

3.6.5	Access Control	107

3.6.6	Authentication/Integrity/Confidentiality	107

3.6.7	Security Audit	107

3.7 - Fault Management	107

3.7.1	Gather POST Results	107

3.7.2	Get Radio Health	108

3.7.3	Start Continuous BIT	108

3.7.4	Monitor Continuous BIT	108

3.7.5	Report Continuous BIT Fault	109

3.7.6	Stop Continuous BIT	109

3.7.7	Stop Specific Continuous BIT	110

3.7.8	Restart Specific Continuous BIT	110

3.7.9	Run Specific Commanded BIT	110

3.8 - Support	111

4 - Waveform Use Cases	112

4.1 - FM3TR Waveform	112

4.2 - WCDMA Waveform	112

4.3 - HF 4285 Military Waveform	112

4.3.1	HF 4285 : Reception	112

4.3.2	HF 4285 : Transmission	113

5 - Open questions	115

Table of FIGURES

Figure 1: Radio System Actors	8

Figure 2: Radio System Components Breakdown	9

Figure 3: Radio Set states	10

Figure 4: Use cases	12

Figure 5: Transmit/Receive	12

Figure 6: Transmit voice	14

Figure 7: Receive voice	16

Figure 8: Transmit data	18

Figure 9: Receive data	19

Figure 10: Transmit a packet	21

Figure 11: Receive a packet	22

Figure 12: Relaying	22

Figure 13: Control	24

Figure 14: Radio Control	25

Figure 15: Startup	25

Figure 16: The startup of the OE is successful	27

Figure 17: The reconfiguration after reset is successful	28

Figure 18: Shutdown	28

Figure 19: Shutdown command	30

Figure 20: Control Communication Channel	31

Figure 21: Communication Channel’s Usage State	31

Figure 22: Communication Channel’s Administrative State	32

Figure 23: Administration	37

Figure 24: Run Diagnostics	37

Figure 25: Manage Time	39

Figure 26: Performance Management	41

Figure 27: Monitor Radio System	49

Figure 28: Access general statistics	50

Figure 29: Obtain OE statistics	51

Figure 30: Alarms	52

Figure 31: Manage Configuration	57

Figure 32: Manage Physical Configuration	57

Figure 33: Installation of a new hardware device	59

Figure 34: Confirmation of configuration change	65

Figure 35: Manage Software Configuration	65

Figure 36: Installation of an application software	67

Figure 37: Installation of OE	68

Figure 38: Install software using initiate OTAP	70

Figure 39: Uninstall an existing software application	73

Figure 40: List installed applications	74

Figure 41: Configure Radio	75

Figure 42: Communications Channel Control	77

Figure 43: Nominal activation of a channel	79

Figure 44: Release a waveform communication channel	80

Figure 45: Configure Waveform	80

Figure 46: Configure a waveform	82

Figure 47: Start of a waveform channel	83

Figure 48: Stop of a waveform channel	83

Figure 49: Configure Retransmission	84

Figure 50: Continue with desired operation	86

Figure 51: Deployment Plan	86

Figure 52: Manage Security	89

Figure 53: Manage Operator Profile	90

Figure 54: A new operator is successfully defined	91

Figure 55: Modification of a user identification	92

Figure 56: Deletion of a user identification	93

Figure 57: Lockout an operator	94

Figure 58: Unlock an operator	95

Figure 59: Manage Security Threats	95

Figure 60: Detection of unauthorized access removal	96

Figure 61: Manage Key Fill	97

Figure 62: Successful key load	98

Figure 63: Successful zeroize	99

Figure 64: Successful zeroize	100

Figure 65: Successful OTAR	102

Figure 66: Successful OTAZ	104

Figure 67: Successful OTAT of keys	105

Figure 68: Initiate algorithm load	106

Figure 69: Fault Management	107

Figure 70: Support	111

Figure 71: HF 4285 Receive data	113

Figure 72: HF 4285 Transmit data	114

# Object

This document contains scenarios that illustrate Radio Set system usage.

These scenarios are issued from several sources:

- MSRC/SRD : srd_1-1_app_a.pdf,

- SDR Forum,

- Submitters contributions.

# Radio System Modeling Elements

Elements presented here introduce some terminology used within scenarios.

## Actors

The following figure summarize main Radio System actors:

Figure 1: Radio System Actors

| Radio System Actor | An actor that figures any other Radio System actor. |
| --- | --- |
| Subscriber | An actor that uses Radio System to transmit and receive. |
| Voice Subscriber | An actor that uses Radio System to transmit and receive voice. |
| Data Subscriber | An actor that uses Radio System to transmit and receive data. This encompasses Streaming and Packetized Data (such as IP – Internet Protocol datagrams). |
| Operator | An actor on the Radio System that handles operation and local configuration management. Note that practically, the Operator is a role that is often assumed by the same entity than the one, which assumes the Subscriber one. |
| Administrator | An actor on the Radio System that handles control, performance and configuration management. |
| Administration System | An actor on the Radio System that support Administrator missions. |
| Security Officer | An actor on the Radio System that handles security management. |
| Intruder | An unauthorized subscriber on the Radio System that tries to overcome security measures to use, spy or damage the Radio System. |
| Maintainer | An actor on the Radio System that handles maintenance. |
| Developper | An actor on the Radio System that figure the system designer. |
| External Radio Set | An actor on the Radio System that figures another Radio Set in connection with current one. |
| Time Providing System | An actor on the Radio System that provides Time and Location services such as GPS. |

Table 1: Radio System Actors

## Component Breakdown

The following figure summarize main Radio System components:

Figure 2: Radio System Components Breakdown

## Radio Set states

The following figure illustrates main Radio Set functional states:

Figure 3: Radio Set states

| State | Allowed Transition | Description |
| --- | --- | --- |
| EMPTY |  | Radio Set do not contain any information. |
|  | burning | Radio Set is loaded with firmware and initial keys. |
| POWER_OFF |  | Radio Set memory contain firmware and initial keys. |
|  | powerOn | Activate all h/w components |
| INITIALIZING |  | Within this transient state, Radio Set setup standalone configuration and sequence various initialisation tasks such as Hardware BIT, Boot, OE Setup, |
|  | saStartup | Radio Set has detected that basic initialization tasks succeeded and that the standalone configuration is ready to run. |
|  | maintenanceCondition | Request from Operator or generated by Radio Set on major and critical conditions (other than security). |
|  | criticalSecurityCondition | Request from Operator (emergency reset), SecurityOfficer (OTAZ), or generated by Radio Set when detecting critical security condition. |
| STANDALONE |  | Within this state, Radio Set perform radio-network related initialisations.. |
|  | netStartup | Radio Set has detected that network BIT succeeded and that radio network configuration is ready to run. |
|  | maintenanceCondition | See above |
|  | criticalSecurityCondition | See above |
| READY |  | Within this state, Radio Set is operative and connected to one or several networks. |
|  | maintenanceCondition | See above |
|  | criticalSecurityCondition | See above |
|  | shutdown | Tear down until power off. |
| MAINTENANCE |  | Within this state, Radio Set is disconnected from any network and allow: - To perform BITs, - To configure radio parameters, - To configure software, - To process security-sensitive operations such as key fill. |
|  | maintenanceCondition | See above |
|  | criticalSecurityCondition | See above |
| EMERGENCY |  | Within this state, Radio Set erases (overwrite) all sensitive information. |
|  | ErasingCompletion | Radio Set has detected that erasure is now complete. |
| ERASED |  | Within this state, all Radio Set memory is erased. Radio Set is unusable. |

# Radio Set Use Cases

Figure 4: Use cases

## Transmit / Receive

Figure 5: Transmit/Receive

### Transmit Voice (non-packet)

Use Case Identifier: 1 - Transmit Voice (non-packet)

Description: The Voice Subscriber wishes to transmit voice over a loaded and configured waveform.

External Actors: Voice Subscriber, External Radio Set

Related Use Cases: Set Waveform Parameters

Precondition: The Operator must be authorized to use the Radio Set. The Operator must have configured the waveform(s) using Set Waveform Parameters. Triggering event:

Main Flow: 1a The Operator is presented with a list of active waveforms by the Radio Set for which Operator has privilege according to Privilege Rules. 2 The Operator selects the waveform channel(s) to be connected to the Voice Subscriber. 3a The Radio Set routes audio from the Voice Subscriber to the selected waveform channel(s) 4 The Operator requests transmission enable. 5a The Radio Set enables transmission(s) 6 The Radio Set indicates to the Operator that the transmission(s) is enabled. 7 The Radio Set transmits the audio to the External Radio Set according to Transmit Rules. 8 The Operator requests transmission disable. 9 The Radio Set disables the transmission(s). 10 The Radio Set indicates to the Operator that the transmission(s) is disabled.

Resulting event: Postcondition: The voice has been transmitted. The Operator can continue to use the same waveform(s) or select a new one.

Alternatives: 1b The Radio Set is single-channel so the list of active waveforms is not presented. 1 [continue at 4] 3b The Radio Set is unable to route audio from the Voice Subscriber to the selected waveform channel(s). 1 The Radio Set informs the Operator that it is unable to route the audio. 5b The Radio Set cannot enable the transmission(s) 1 The Radio Set notifies the Operator of an exception. 2 [continue at 10] The Radio Set detects the occurrence of a fault (out of lock, over temperature, low output power.) 1 The Radio Set disables transmission. 2 The Radio Set notifies the Operator of the fault. 3 [continue at 10] The Radio Set detects an alarm (e.g. Crypto Alarm) 1 The Radio Set notifies the Operator of an alarm.

Nonfunctional requirements: Privilege Rules : Operator privilege is domain specific and will be global or restricted based on the platform. Transmit Rules : SA data transmission will precede audio transmission if selected Set Waveform Parameters. The transmission will be secure if selected by Set Waveform Parameters.

Comments:

Figure 6: Transmit voice

### Receive Voice (non-packet)

Use Case Identifier: 2 - Receive Voice (non-packet)

Description: The Voice Subscriber wishes to receive voice data via a loaded and configured waveform.

External Actors: Operator, Voice Subscriber, External Radio Set

Related Use Cases: Set Waveform Parameters

Precondition: The Operator must be authorized to use the Radio Set. The Operator must have configured the waveform(s) using Set Waveform Parameters. Triggering event:

Main Flow: 1a The Operator is presented with a list of active waveforms by the Radio Set for which the Operator has privilege according to the Privilege Rules. 2 The Operator selects the waveform channel to be connected to the Voice Subscriber interface. 3 The Radio Set detects the transmission from the External Radio Set and provides activity indication. 4a The Radio Set routes the audio to the Voice Subscriber based on the Receive Rules. 5a The Radio Set presents the audio to the Voice Subscriber. 6 The Radio Set detects the end of External Radio Set transmission. 7 The Radio Set squelches the output associated with the previous activity and no longer indicates activity.

Resulting event: TBD Postcondition: The voice data has been received. The Operator is done receiving the voice data via the selected waveform, the Radio Set is returned to the state it was in before the scenario.

Alternatives: 1b The Radio Set is single-channel so no list is presented. 1 [continue at 3] 4b The Radio Set is unable to route the received data to the Voice Subscriber. 1 The Radio Set informs the Operator that it is unable to route data to the Voice Subscriber. 5b The Radio Set is unable to present to audio to the Voice Subscriber. 1 The Radio Set informs the Operator that it is unable to present audio to the Voice Subscriber. The Radio Set detects the occurrence of a fault (out of lock, over temperature, low output power.) 1 The Radio Set disables reception. 2 The Radio Set notifies the Operator of a fault. 3 [continue at 11]

Nonfunctional requirements: Privilege Rules : Operator privilege is domain specific and will be global or restricted based on the platform. Receive Rules : SA will be supported if selected by Set Waveform Parameters. The transmission will be decrypted if CT is selected by Set Waveform Parameters. PT transmissions will bypass decryption if CT is selected by Set Waveform Parameters.

Comments:

Figure 7: Receive voice

### Transmit Data Stream (non-packet)

Use Case Identifier: 3 - Transmit Data Stream (non-packet)

Description: The Operator wishes to transmit a data stream (e.g. FM data mode: where the data is not in a packet format) over a loaded and configured waveform.

External Actors: Operator, Data Subscriber

Related Use Cases: Set Waveform Parameters

Precondition: The Operator must be authorized to use the Radio Set. The Operator must have configured the waveform(s) using Set Waveform Parameters. Triggering event:

Main Flow: 1a The Operator is presented with a list of active waveforms by the Radio Set for which Operator has privilege according to Privilege Rules. 2 The Operator selects the waveform channel(s) to be connected to the Data Subscriber. 3a The Radio Set routes data from the Data Subscriber to the selected waveform channel(s). 4 The Data Subscriber requests transmission enable. 5a The Radio Set enables transmission(s). 6 The Radio Set indicates to the Data Subscriber and the Operator that the transmission(s) are enabled. 7 The Radio Set transmits the data to the External Radio Set according to Transmit Rules. 8 The Data Subscriber requests transmission disable. 9 The Radio Set disables the transmission(s). 10 The Radio Set indicates to the Data Subscriber and the Operator the transmission(s) are disabled.

Resulting event: Postcondition: The data has been transmitted. The Operator can continue to use the same waveform(s) or select a new one.

Alternatives: 1b The Radio Set is single-channel so no list is presented 1 [continue at 4] 3b The Radio Set is unable to route the Data Subscriber interface to the selected waveform channel(s). 1 The Radio Set informs the Operator that it is unable to route the interface. 5b The Radio Set cannot enable the transmission(s). 1 [continue at 10] The Radio Set detects the occurrence of a fault (out of lock, over temperature, low output power.) 1 The Radio Set disables transmission. 2 The Radio Set notifies the Operator of the fault. 3 [continue at 10] The Radio Set detects an alarm (e.g. Crypto Alarm) 1 The Radio Set notifies the Operator of an alarm. Verify this is added to all.

Nonfunctional requirements: Privilege Rules : Data Subscriber privilege is domain specific and will be global or restricted based on the platform. Transmit Rules : SA data transmission will precede data transmission if selected by Set Waveform Parameters. The transmission will be encrypted if selected by Set Waveform Parameters.

Comments:

Figure 8: Transmit data

### Receive Stream Data (non-packet)

Use Case Identifier: 4 - Receive Stream Data (non-packet)

Description: The Data Subscriber wishes to receive a data stream (e.g. FM data mode: where the data is not in a packet format ) via a loaded and configured waveform.

External Actors: Operator, Data Subscriber

Related Use Cases: Set Waveform Parameters

Precondition: The Operator must have configured the waveform(s) using Set Waveform Parameters. Triggering event:

Main Flow: 1a The Operator is presented with a list of active waveforms by the Radio Set for which he/she has privilege. 2 The Operator selects the waveform channel to be connected to the Data Subscriber interface. 3 The Radio Set detects data and sends the data to the Data Subscriber. 4 The Data Subscriber receives entire data stream and the Radio Set returns to the state it was in previously

Resulting event: Postcondition: When the Operator is done receiving a data stream via the selected waveform, the Radio Set returns to the state it was in previously.

Alternatives: 1b The Radio Set is single-channel so no list is presented. 1 [continue at 3] 3b The Radio Set is unable to route the received data to the Data Subscriber. 1 The Radio Set informs the Operator that it is unable to route data to the Data Subscriber interface.

Nonfunctional requirements:

Comments:

Figure 9: Receive data

### Transmit Packet

Use Case Identifier: 5 - Transmit Packet

Description: The Radio Set transmits a packet received from the Data Subscriber. The packet may contain data, digitized voice or digitized video.

External Actors: Data Subscriber, Radio Set, External Radio Set

Related Use Cases: Set Waveform Parameters

Precondition: The Data Subscriber must be authorized to use the Radio Set. The Operator must have configured the waveform using Set Waveform Parameters which may include QoS parameters. Triggering event:

Main Flow: 1 The Data Subscriber sends a network packet to the Radio Set according to Network Packet Rules. 2a The Radio Set resolves the packet address and routes the packet to the appropriate waveform channel(s). 3 The Radio Set transmits the packet to the External Radio Set(s) over the addressed waveform channel(s). according to Transmit Rules

Resulting event: Postcondition: The Radio Set returns to the state it was in before the scenario.

Alternatives: 2b The Radio Set does not have a route for the network address in the packet. 1 The Radio Set drops the packet. The Radio Set detects the occurrence of a fault (out of lock, over temperature, low output power.) 1 The Radio Set disables transmission. 2 The Radio Set notifies the Operator of the fault. 3 [continue at 10] The Radio Set detects an alarm (e.g. Crypto Alarm) 1 The Radio Set notifies the Operator of an alarm.

Nonfunctional requirements: Network Packet Rules : The packet contains a point to point, broadcast or multi-cast address. Transmit Rules : The transmission will be encrypted if selected by Set Waveform Parameters.

Comments:

Figure 10: Transmit a packet

### Receive Packet

Use Case Identifier: 6 - Receive Packet

Description: The Subscriber wishes to receive packet information (voice, data, or video).

External Actors: Operator, External Radio Set, Subscriber

Related Use Cases: Set Waveform Parameters

Precondition: The Subscriber must be authorized to use the Radio Set. The Operator must have configured the waveform(s) using Set Waveform Parameters which may include QoS parameters. Triggering event:

Main Flow: 1 The External Radio Set sends a transmission that is received by the Radio Set. 2 The Radio Set converts the transmission to packet(s). 3a The Radio Set resolves the packet addressing and routes the packet(s) to the appropriate Subscriber(s). 4 The Operator receives entire packet transmission and the Radio Set returns to the state it was in previously

Resulting event: Postcondition: When the Operator is done receiving data, the Radio Set returns to the state it was in before the scenario.

Alternatives: 3b Radio Set determines that the message is addressed to another Operator/Radio Set in the routing table. 1 Radio Set determines the next hop to forward message to reach the final destination address. 2 Radio Set sends the packet to the next hop in the route to deliver the message to the final destination address.

Nonfunctional requirements:

Comments:

Figure 11: Receive a packet

## Scanning

TBD

## Relaying

Figure 12: Relaying

### Repeating

Use Case Identifier: 7 - Repeating

Description: The Radio Set forwards information received on a waveform channel towards the same waveform channel.

External Actors: Radio Set, External Radio Set

Related Use Cases: Set Waveform Parameters

Precondition: The Operator must have configured the waveform(s) using Set Waveform Parameters with ability to forward received transmission. Triggering event:

Main Flow: 1 An External Radio Set sends a transmission that is received by the Radio Set. 2 The Radio Set re-transmit the information received towards another External Radio Set(s).

Resulting event: Postcondition: When the information has been forwarded, the Radio Set returns to the state it was in before the scenario.

Alternatives:

Nonfunctional requirements:

Comments:

### Bridging

Use Case Identifier: 8 - Bridging

Description: The Radio Set forwards information received on a waveform channel another waveform channel.

External Actors: Radio Set, External Radio Set

Related Use Cases: Set Waveform Parameters

Precondition: The Operator must have configured the waveform(s) using Set Waveform Parameters with ability to forward received transmission. Triggering event:

Main Flow: 1 An External Radio Set sends a transmission that is received by the Radio Set. 2 The Radio Set re-transmit the information received towards another External Radio Set(s).

Resulting event: Postcondition: When the information has been forwarded, the Radio Set returns to the state it was in before the scenario.

Alternatives:

Nonfunctional requirements:

Comments:

### Networking

Use Case Identifier: 9 - Networking

Description: The Radio Set forwards packet information (voice, data, or video) within a single network protocol.

External Actors: Operator, External Radio Set

Related Use Cases: Set Waveform Parameters

Precondition: The Operator must have configured the network’s routing tables. Triggering event:

Main Flow: 1 The Radio Set receive a transmission from a channel. 2 The Radio Set converts the transmission to packet(s). 3 The Radio Set resolves the packet addressing, 4 The Radio Set finds the next hop address to route the packet(s) to the appropriate Subscriber(s). 5. The Radio Set finds an appropriate channel to transmit packet(s), 6 The Radio Set convert packet(s) to transmission, 7 The Radio Set transmit to the next hop through the appropriate channel.

Resulting event: Postcondition: When the data has been forwarded, the Radio Set returns to the state it was in before the scenario.

Alternatives: 5b The Radio Set report an error if no appropriate channel exists.

Nonfunctional requirements:

Comments: Routing may be ruled by QoS protocol.

### Internetworking

Use Case Identifier: 10 - Inter Networking

Description: The Radio Set forwards packet information (voice, data, or video) between 2 networks protocols.

External Actors: Operator, External Radio Set

Related Use Cases: Set Waveform Parameters

Precondition: The Subscriber must be authorized to use the Radio Set. The Operator must have configured the waveform(s) using Set Waveform Parameters which may include QoS parameters. Triggering event:

Main Flow: 1 The Radio Set receive a transmission from a channel. 2 The Radio Set converts the transmission to packet(s). 3 The Radio Set resolves the packet addressing, 4 The Radio Set convert packet addressing from network_1 to network_2, 5 The Radio Set finds the next hop address to route the packet(s) to the appropriate Subscriber(s). 6. The Radio Set finds an appropriate channel to transmit packet(s), 7 The Radio Set convert packet(s) to transmission, 8 The Radio Set transmit to the next hop through the appropriate channel.

Resulting event: Postcondition: When the data has been forwarded, the Radio Set returns to the state it was in before the scenario.

Alternatives: 5b The Radio Set report an error if no appropriate channel exists.

Nonfunctional requirements:

Comments: Routing may be ruled by QoS protocol.

## Control

Figure 13: Control

### Radio Control

Figure 14: Radio Control

#### Startup

Figure 15: Startup

##### Power-up and Initialize

Use Case Identifier: 11 - Power-up and Initialize

Description: An Actor such as the Operator, Maintainer, etc., applies power to the Radio Set which then performs activities such as: 1. BIT/diagnostics 2. Bus enumeration, 3. Communication network configuration 4. I/O configuration 5. Device initialization The Radio Set optionally loads default waveform(s) (see note 1).

External Actors: Operator, Maintainer

Related Use Cases: Initiate System BIT

Precondition: The power is off to the Radio Set (see note 2). The Operator has authorization to use the Radio Set. Triggering event:

Main Flow: 1 The Operator applies prime power to the Radio Set. 2a The Radio Set successfully completes power-up BIT per the Initiate System BIT use case, and the Radio Set reports status (e.g. OE is operational). 3a The Operator acknowledges the startup of the OE and begins authorized use of the Radio Set. 4a The Radio Set presents the options for the Operator to Configure Radio, Run Diagnostics, Receive/Transmit, etc. [end of case]

Resulting event: Postcondition: The Radio Set is ready to be configured and has optionally established default communications path(s).

Alternatives: 2b The Radio Set fails power-up BIT with a critical failure. 1 [end of case] 2c The Radio Set fails power-up BIT with a non-critical failure. 1 The Radio Set reports failure [continue at 3]. 3b The Operator finds that the operational status is a degraded mode which would not provide sufficient capability for current needs. 1 The Operator selects the shutdown option. [end of case]

Nonfunctional requirements:

Comments: Note 1: Default waveform boot-up is optionally a function of the Radio Set, or it may be partitioned out to an application element such as the GUI. Note 2: The previous shutdown of the Radio Set was performed in an orderly way as per the "Shutdown" Use Case. In other cases of more abrupt power-down, separate use case functionality would apply.

Use Case Identifier: 12 - Power Up

Description: Describes the start-up of a radio

External Actors: Supporter

Related Use Cases: Instantiate Communication Channel, Enable Application, Enable Communication Channel, Instantiate Application, Parse Deployment Plan, Power On Self Test

Precondition: Radio is off Triggering event: A supporter applies power to the radio.

Main Flow: 1. Supporter applies power to the radio 2. Power On Self Test (POST) is performed 3. SWRADIO infrastructure (framework) is deployed. 4. Communication Channels are instantiated. 5. Communication Channels are enabled. 6. If waveform deployment plan exists, it is parsed. 7. Applications are instantiated as per the deployment plan. 8. Applications are enables as per the deployment plan.

Resulting event: None Postcondition: Radio is running, waveform are deploy as per the power-up deployment plan

Alternatives: Radio could return to the last known running state. This would require the deployment plan to be updated each time a new waveform/service/application was deployed.

Nonfunctional requirements:

Comments:

Figure 16: The startup of the OE is successful

##### System Reset

Use Case Identifier: 13 - System Reset

Description: This use case begins when the Radio Set automatically begins to progress through initialization actions after a temporary power loss or after a Operator-requested system reset. The Radio Set performs initial start-up tasks and configures itself to an operational state as it was established before the reset. This configuration action includes (optionally) loading the waveforms that were loaded at the time of the reset. The Radio Set then resumes normal operation.

External Actors: Operator

Related Use Cases:

Precondition: The Radio Set reset is activated or power is resumed after a disruption for a number of seconds. The following are available in the Radio Set non-volatile memory: 1. The list of waveforms that were running or ready to run prior to the reset or power interrupt; 2. The state of the Radio Set "ReceiveOnly" attribute at the time of the reset or power interrupt; 3. An indication that the previous shutdown was not a result of a Operator-applied commanded shut-down (per Shutdown use case); Triggering event:

Main Flow: 1 This use case begins when the Radio Set automatically begins to progress through initialization actions after a temporary power loss or Operator-requested system reset. 2a The Radio Set successfully performs all initialization and reconfiguration activities. 3a The Radio Set attains operational status and notifies the Operator. [end of case]

Resulting event: Postcondition: The Radio Set is operational in a configuration equivalent to that before the reset.

Alternatives: The Radio Set notifies the Operator that a subsystem has not successfully re-initialized and requires specific action(s) to proceed. 2b 1 Operator corrects the problem and requests continued initialization. [continue at 2a] 2c The Radio Set notifies the Operator that a subsystem has not responded to re-initialization and that there are no recovery options. 1 Operator acknowledges the failure and initiates fault isolation activity (per Radio Set Diagnostics use case) or prompt shutdown of primary power if necessary. [end of case]

Nonfunctional requirements:

Comments:

Figure 17: The reconfiguration after reset is successful

#### Shutdown

Figure 18: Shutdown

##### Loss of Power

Use Case Identifier: 14 - Loss of Power

Description: The Radio Set loses power.

External Actors: Power Supplier

Related Use Cases:

Precondition: The Radio Set is powered on. Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Commanded Shutdown

Use Case Identifier: 15 - Commanded Shutdown

Description: Upon Operator-initiated Shutdown, the Radio Set automatically performs activities to cleanly prepare Radio Set for complete power-off event such as: 1. Place hardware devices into proper state for shutdown (per Security Rules for Shutdown). 2. Prompt Operator to correct invalid states for shutdown 3. Disconnect Communication network(s) 4. Update configuration data in preparation for next Startup (optionally) 5. Provide indication to the Operator when all waveforms and network connections have been terminated and OE software is in a shutdown configuration 6. Provides option to restart.

External Actors: Operator

Related Use Cases:

Precondition: The Radio Set is operational per successful Startup. The Operator has authorization to perform the Radio Set shutdown. Triggering event:

Main Flow: 1 The use case begins when the Operator issues a shutdown command to the Radio Set. 2a The Radio Set successfully performs all shutdown activities (disconnect communication network(s), etc.) and notifies the Operator. 3a The Operator acknowledges the shutdown completion and removes primary power from the Radio Set. [end of case]

Resulting event: Postcondition: The Radio Set is in a shutdown configuration and can be powered off or restarted.

Alternatives: 2b The Radio Set notifies the Operator that a hardware subsystem is not in proper shutdown state and requires a specific action to proceed. 1 Operator corrects problem and requests continued shutdown [continue at 2] 2c The Radio Set notifies the Operator that a priority application is in operation and that the Operator's authorization level does not permit the shutdown. [end of case] 3b The Operator initiates a restart. 1 The Radio Set restarts per the Startup use case. [end of case] 3c The Radio Set shuts itself off (suspends power) [end of case]

Nonfunctional requirements:

Comments: During commanded shutdown, the following security rules apply: Store Tables and applications, Erase RED memories and applications, Overwrite RED algorithms, Zeroize RED key, Zeroize all TRANSEC keys; The architecture must also accommodate shutdown cases of abrupt (intentional or unintentional) power removal. For example, a case maybe made to incorporate internal power supplies to provide hold-up power while software shuts down. When power is abruptly removed, there may be configuration variations that occur upon next power-up, but no damage will occur to the system . Use case scenarios for this would be handled separately.

Use Case Identifier: 16 – Shutdown

Description: Describes the shutdown of a radio

External Actors: Supporter

Related Use Cases:

Precondition: Radio is on Triggering event: A supporter applies power to the radio.

Main Flow: 1. Supporter requests radio shutdown 2. Each running Application is un-instantiated. 3. Each Communication Channel is un-instantiated. 4. The SWRADIO infrastructure is town down. 5. The supporter is informed that the radio can be powered off.

Resulting event: None Postcondition: Radio is running, waveform are deploy as per the power-up deployment plan

Alternatives: Loss of power.

Nonfunctional requirements:

Comments:

Figure 19: Shutdown command

#### Radio Silence

Use Case Identifier: 17 - Change Radio Transmission Mode

Description: Describes the use case steps for changing the radio’s transmission mode.

External Actors: Comm Supporter, Radio Manager

Related Use Cases: Inhibit Transmission Communication Channel

Precondition: Radio is Operational. Triggering event: A comm. supporter requested the radio’s transmissions to be changed.

Main Flow: 1. An actor requests transmission mode change for the radio. 2. The Radio System delegates change transmission mode commands to each communication channel so as to perform waveform specific actions associated with transmission mode request (basically makes use of the Change Comm Channel Transmission Mode use case)

Resulting event: None Postcondition: Radio is in an Operational Silent State or Operational Transmission State.

Alternatives: 2a) The Radio system performs no operations since the radio is in the correct transmission mode of operation.

Nonfunctional requirements:

Comments:

#### Radio Identification

TBD

#### Radio time

TBD

#### Control Communication Channel

Figure 20: Control Communication Channel

Figure 21: Communication Channel’s Usage State

Figure 22: Communication Channel’s Administrative State

##### Change Comm Channel Transmission Mode

Use Case Identifier: 18 - Change Comm Channel Transmission Mode

Description: This use case addresses the ability of an actor to change a communication channel transmission or emissions control (EMCON) mode.

External Actors: Comm Supporter, Radio Manager

Related Use Cases:

Precondition: Communication Channel can be in any state of operation. Triggering event:

Main Flow: 1. An actor selects transmission mode for a communication channel 2. Communication Channel performs application specific actions related to transmission mode request for the channel. 3. Communication Channel sends transmission mode request to its RF Channels. RF Channel configures its communication equipment (power amplifier, etc.) based upon the transmission mode request. 4. Communication Channel reports completion of its request to the actor.

Resulting event: Postcondition: Communication Channel and its RF Channels and comm. Equipments are in inhibit or enable transmission mode of operation.

Alternatives: 2a) Comm Channel is already in the correct transmission mode, no action is performed. 2b) Comm Channel is in an improper state (Half-Duplex Receive Only) that permits transmission mode settings. No action is performed. Error returned? 2c) The Comm Channel’s Usage State is IDLE, therefore no waveform specific action is taken. 3a) Failure to configure comm. equipment

Nonfunctional requirements:

Comments: Transmission mode is either enable transmission or disable transmission. When enabled transmission of data out to the antenna is permitted.

##### Change Comm Channel Administrative Mode

Use Case Identifier: 19 - Change Comm Channel Administrative Mode

Description: This use case addresses the ability of an actor to change a communication channel administrative state. The administrative state indicates the permission to use or prohibition against using the communication channel.

External Actors: Comm Supporter

Related Use Cases:

Precondition: Communication Channel is either in an administrative state of UNLOCKED, Transitioning to LOCKED, or LOCKED. Triggering event:

Main Flow: 1. An actor requests a change in the administrative mode of operation for a communication channel. 2. The Communication Channel transitions to either UNLOCKED, Transitioning to SHUTTING_DOWN or LOCKED depending of the state it is in as shown in the Figure Communication Channel’s Administrative State. 3. The Communication Channel sends a state change event indicating the administrative state the Communication Channel is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Communication Channel. - The sourceId field is the identifier attribute of the Communication Channel. - The stateChangeCategory field is ADMINISTRATIVE_STATE_EVENT. - The stateChangeFrom and stateChangeTo fields shall reflect the administrative state before and after the state change, respectively.

Resulting event: Postcondition: Communication Channel is in either UNLOCKED, SHUTTING_DOWN, or LOCKED.

Alternatives: 3a) No event is sent when there is no change in administrative state.

Nonfunctional requirements:

Comments: When the Communication Channel’s administrative state is LOCKED, it cannot be instantiated for waveform usage. The Administrative States are based upon CCITT (the International Telegraph and Telephone Consultative Committee), which is a permanent organ of ITT (International Telephone Union) Recommendation X.731 that was approved on Jan 17th 1992. This is also published as ISO/IEC International Standard 10164-2.

##### Configure Comm Channel

Use Case Identifier: 20 - Configure Comm Channel

Description: This use case addresses the ability of an actor to configure an instantiated communication channel.

External Actors: Comm User

Related Use Cases: Set Waveform Parameters

Precondition: Communication Channel’s Usage State is BUSY. Triggering event:

Main Flow: 1. An actor requests configuration changes to a communication channel. 2. Communication Channel performs waveform application specific actions related to requested configuration changes.

Resulting event: Postcondition: Communication Channel’s configuration has been modified.

Alternatives: 2.a) No configuration is performed, the configuration parameters are invalid for the waveform application; an exception is raised indicating this. 2.b) Partial configuration is performed, Some configuration parameters are invalid for the waveform application, and an exception is raised indicating this.

Nonfunctional requirements:

Comments:

##### Change Comm Channel Security Mode

Use Case Identifier: 21 - Change Comm Channel Security Mode

Description: This use case addresses the ability of an actor to change a communication channel security mode.

External Actors: Comm User

Related Use Cases:

Precondition: Communication Channel’s Usage State is BUSY. Triggering event:

Main Flow: 1. An actor selects security mode for communication channel. 2. Communication Channel performs waveform application specific actions related to security mode request for the channel. 3. Communication Channel requests security mode change for the associated Security Channel. 4. Communication Channel reports completion of its request to the actor.

Resulting event: Postcondition: Communication Channel’s security mode changes.

Alternatives: 2.a) Comm Channel is already in the correct security mode, no action is performed. 2.b) Security mode changes for waveform application is not allowed, no action is performed

Nonfunctional requirements:

Comments:

##### List Comm Channels

Use Case Identifier: 22 - List Comm Channels

Description: This use case addresses the ability of an actor to retrieve communication channels.

External Actors: Comm Supporter

Related Use Cases:

Precondition: Radio is Operational Triggering event:

Main Flow: 1. An actor requests communication channels information from the Radio. 2. The radio returns a list of communication channels, along with their characteristics.

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Instantiate Comm Channel

Use Case Identifier: 23 - Instantiate Comm Channel

Description: This use case addresses the ability of an actor to instantiate a communication channel with a specific waveform.

External Actors: Comm Support, Radio Manager, Comm User

Related Use Cases: Instantiate Application

Precondition: Communication Channel’s administrative state of UNLOCKED, Operational State is ENABLE, and Usage State is IDLE. Triggering event:

Main Flow: 1. An actor requests a communication channel to be instantiated with a specific waveform. 2. The communication channel requests waveform’s ApplicationFactory to instantiate the waveform using the Communication Channel’s devices and any initial configuration parameters. 3. The waveform’s ApplicationFactory uses input devices to allocate devices, load waveform’s components onto devices, and execute waveform’s components onto devices (basically makes use of the InstantiateApplication use case) 4. The Communication Channel’s Usage State is set to Busy. The Communication Channel sends a state change event indicating the usage state the Communication Channel is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Communication Channel. - The sourceId field is the identifier attribute of the Communication Channel. - The stateChangeCategory field is USAGE_STATE_EVENT. - The stateChangeFrom and stateChangeTo fields shall reflect the usage state before and after the state change, respectively. - The mode of operations: Waveform mode receive only, transmit only, full duplex, Data Encryption Security Mode - Secure and non-secure, Classification level – unclassified, Top Secret, etc.

Resulting event: Postcondition: Communication Channel’s Usage State is Busy.

Alternatives: 2.a) The request is denied due to the communication channel not in the correct pre-condition states 3.a) The waveform’s ApplicationFactory is unable to deploy due to various reasons.

Nonfunctional requirements: Need to be able to execute in an asynchronous manner.

Comments: The Administrative, Operational, and Usage States are based upon CCITT (the International Telegraph and Telephone Consultative Committee), which is a permanent organ of ITT (International Telephone Union) Recommendation X.731 that was approved on Jan 17th 1992. This is also published as ISO/IEC International Standard 10164-2.

##### Uninstantiate Comm Channel

Use Case Identifier: 24 - Uninstantiate Comm Channel

Description: This use case addresses the ability of an actor to uninstantiate a communication channel.

External Actors: Comm Support, Radio Manager, Comm User

Related Use Cases: Uninstantiate Application

Precondition: Communication Channel’s administrative state of UNLOCKED or SHUTTING_DOWN, Operational State is ENABLE, and Usage State is BUSY. Triggering event:

Main Flow: 1. An actor requests a communication channel to be uninstantiated. 2. The communication channel requests waveform’s Application component to teardown. 3. The waveform’s Application component unallocates devices’ capacities, unloads waveform’s components from devices, and terminates waveform’s components executions from devices (basically makes use of the UninstantiateApplication use case) 4. The Communication Channel’s Usage State is set to IDLE. The Communication Channel sends a state change event indicating the usage state the Communication Channel is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Communication Channel. - The sourceId field is the identifier attribute of the Communication Channel. - The stateChangeCategory field is USAGE_STATE_EVENT. - The stateChangeFrom and stateChangeTo fields shall reflect the usage state before and after the state change, respectively.

Resulting event: Postcondition: Communication Channel’s Usage State is IDLE. Optional the Communication Channel’s Administrative can be in the LOCKED state.

Alternatives: 2a) The request is denied due to the communication channel not in the correct pre-condition states 6) The Communication Channel’s Administrative State is set to LOCKED. The Communication Channel sends a state change event indicating the administrative state the Communication Channel is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Communication Channel. - The sourceId field is the identifier attribute of the Communication Channel. - The stateChangeCategory field is ADMINISTRATIVE_STATE_EVENT. The stateChangeFrom and stateChangeTo fields shall reflect the administrative state before and after the state change, respectively.

Nonfunctional requirements: Need to be able to execute in an asynchronous manner.

Comments: The Administrative, Operational, and Usage States are based upon CCITT (the International Telegraph and Telephone Consultative Committee), which is a permanent organ of ITT (International Telephone Union) Recommendation X.731 that was approved on Jan 17th 1992. This is also published as ISO/IEC International Standard 10164-2.

### Waveform Control

#### Change Frequency

TBD

#### Change QoS

TBD

#### Power Control

TBD

#### Antenna Management

(directional pointing, antenna choice, etc)

TBD

#### Bandwidth management

TBD

### Spectrum Control

#### Cosite

TBD

## Administration

Figure 23: Administration

### Run Diagnostics

Figure 24: Run Diagnostics

#### Enable Background BIT

Use Case Identifier: 25 - Enable Background BIT

Description: The Operator directs the Radio Set to run BIT in the background.

External Actors: Operator

Related Use Cases:

Precondition: Radio is operational and Operator has logged on and been accepted as valid Operator. Triggering event:

Main Flow: 1 Operator requests background BIT. 2a Radio validates that Operator has that privilege and enables background BIT 3a Background BIT runs.

Resulting event: Postcondition: Background BIT is running

Alternatives: 2b Operator does not have the privileges for this function 1 Radio notifies the Operator that he does not have this privilege and exits. 3b Background BIT detects and reports a failure. 1 Radio logs the failure, reports the failure to the Operator and determines the allowed Modes of Operation in accordance with its Security Policy.

Nonfunctional requirements:

Comments: The Operator privileges are defined by the authentication profile for Operator.

#### Initiate System BIT

Use Case Identifier: 26 - Initiate System BIT

Description: The Maintainer directs the Radio Set to run a system level BIT that is possibly destructive.

External Actors: Maintainer

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements: System BIT is not allowed during a Fill or Software Download operation.

Comments:

#### Initiate Channel BIT

Use Case Identifier: 27 - Initiate Channel BIT

Description: The Operator directs the Radio Set to run BIT for a specific radio channel.

External Actors: Operator

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements: Channel BIT is not allowed during a Fill or Software Download operation.

Comments:

#### View Fault Log

Use Case Identifier: 28 - View Fault Log

Description: The Maintainer, Administrator, and/or Security Officer view the fault log produced by BIT.

External Actors: Maintainer, Administrator, Security Officer

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

### Manage Time

Figure 25: Manage Time

#### Provide System Synchronization

Use Case Identifier: 29 - Provide System Synchronization

Description:

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

#### Provide Time to Waveforms

Use Case Identifier: 30 - Provide Time to Waveforms

Description: Time is distributed to the waveforms in the Radio Set.

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

#### Synchronize to External Time Source

Use Case Identifier: 31 - Synchronize to External Time Source

Description: The Radio Set is directed to synchronize with an external Time Source.

External Actors: Time Source

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

### Performance Management

Figure 26: Performance Management

#### Create PM Status Report

Use Case Identifier: 32 - Create PM Status Report

Description: The Administrator creates a Performance Monitoring Status report that will continuously monitor selected properties in the radio for a desired duration.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of monitorable properties. 2. The Administrator creates a PM status report and specifies the list of properties to monitor, the duration of the report, the window size, and the sampling rate. 3. The Radio System saves the PM status report configuration.

Resulting event: None Postcondition: PM status report is created and is ready to be activated

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Start PM Status Reporting

Use Case Identifier: 33 - Start PM Status Reporting

Description: The Administrator enables a previously created Performance Monitoring status report.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available PM status reports. 2. The Administrator enables the PM status reports to be activated. 3. The Radio System updates the mode of the PM status report. 4. The Radio System begins to monitor the desired properties and stores the report data. 5. The Administrator or any authorized user of the data can access the report output.

Resulting event: None Postcondition: PM status report output is created and is ready to be viewed or used by another component in the system.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Stop PM Status Reporting

Use Case Identifier: 34 - Stop PM Status Reporting

Description: The Administrator disables a previously running Performance Monitoring status report.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available PM status reports. 2. The Administrator disables the PM status report that was previously running. 3. The Radio System updates the mode of the PM status report. 4. The Radio System stops monitoring the desired properties. 5. The Administrator or any authorized user of the data can access the report output.

Resulting event: None Postcondition: PM status report output is created but no longer updating and is ready to be viewed or used by another component in the system.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Delete PM Status Report

Use Case Identifier: 35 - Delete PM Status Report

Description: The Administrator deletes a previously created Performance Monitoring status report.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available PM status reports. 2. The Administrator deletes one of the PM status reports. 3. The Radio System removes the PM status report from the radio storage. 4. The Radio System removes all related status report output data.

Resulting event: None Postcondition: PM status report is deleted and all associated output data.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Create PM Threshold Report

Use Case Identifier: 36 - Create PM Threshold Report

Description: The Administrator creates a threshold report that will continuously monitor selected properties in the radio for a desired duration, test each property for a desired threshold, and triggers a report output when a property exceeds its upper or lower threshold.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of monitorable properties. 2. The Administrator sets the threshold range for each property to be monitored 3. The Administrator creates a threshold report and specifies the list of properties to monitor, the thresholds, the duration of the report, the window size, and the sampling rate. 4. The Radio System saves the threshold report configuration.

Resulting event: None Postcondition: Threshold report is created and is ready to be activated

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Start PM Threshold Reporting

Use Case Identifier: 37 - Start PM Threshold Reporting

Description: The Administrator enables a previously created threshold report.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available threshold reports. 2. The Administrator enables the threshold reports to be activated. 3. The Radio System updates the mode of the threshold report. 4. The Radio System begins to monitor the desired properties and stores the report data when a threshold is reached. 5. The Administrator or any authorized user of the data can access the report output.

Resulting event: None Postcondition: Threshold report output is created and is ready to be viewed or used by another component in the system.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Stop PM Threshold Reporting

Use Case Identifier: 38 – Stop PM Threshold Reporting

Description: The Administrator disables a previously running threshold report.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available threshold reports. 2. The Administrator disables the threshold report that was previously running. 3. The Radio System updates the mode of the threshold report. 4. The Radio System stops monitoring the desired properties. 5. The Administrator or any authorized user of the data can access the report output.

Resulting event: None Postcondition: Threshold report output is created but no longer updating and is ready to be viewed or used by another component in the system.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Delete PM Threshold Report

Use Case Identifier: 39 - Delete PM Threshold Report

Description: The Administrator deletes a previously created threshold report.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available threshold reports. 2. The Administrator deletes one of the threshold reports. 3. The Radio System removes the threshold report from the radio storage. 4. The Radio System removes all related report output data.

Resulting event: None Postcondition: Threshold report is deleted and all associated output data.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Create PM On Change Report

Use Case Identifier: 40 - Create PM On Change Report

Description: The Administrator creates an on-change report that will continuously monitor selected properties in the radio for a desired duration and triggers a report output when a property changes in value.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of monitorable properties. 2. The Administrator creates an on-change report and specifies the list of properties to monitor, the duration of the report, the window size, and the sampling rate. 3. The Radio System saves the on-change report configuration.

Resulting event: None Postcondition: On-change report is created and is ready to be activated

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Start PM On Change Reporting

Use Case Identifier: 41 - Start PM On Change Reporting

Description: The Administrator enables a previously created on-change report.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available on-change reports. 2. The Administrator enables the on-change reports to be activated. 3. The Radio System updates the mode of the on-change report. 4. The Radio System begins to monitor the desired properties and stores the report data upon a change in the property. 5. The Administrator or any authorized user of the data can access the report output.

Resulting event: None Postcondition: On-change report output is created and is ready to be viewed or used by another component in the system.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Stop PM On Change Reporting

Use Case Identifier: 42 - Stop PM On Change Reporting

Description: The Administrator disables a previously running on-change report.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available on-change reports. 2. The Administrator disables the on-change report that was previously running. 3. The Radio System updates the mode of the on-change report. 4. The Radio System stops monitoring the desired properties. 5. The Administrator or any authorized user of the data can access the report output.

Resulting event: None Postcondition: On-change report output is created but no longer updating and is ready to be viewed or used by another component in the system.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Delete PM On Change Report

Use Case Identifier: 43 - Delete PM On Change Report

Description: The Administrator deletes a previously created on-change report.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available on-change reports. 2. The Administrator deletes one of the on-change reports. 3. The Radio System removes the on-change report from the radio storage. 4. The Radio System removes all on-change report output data.

Resulting event: None Postcondition: On-change report is deleted and all associated output data.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Create Intermediate Calculation

Use Case Identifier: 44 - Create Intermediate Calculation

Description: The Administrator creates an Intermediate Calculation operation that will continuously perform a basic arithmetic (+, -, *, /) calculation on one or two selected properties in the radio for a desired duration.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of monitorable properties. 2. The Administrator creates an intermediate calculation operation and specifies the list of properties to operate on, the type of calculation to perform, the location to store the resulting calculation, the duration of the calculation, and the sampling rate. 3. The Radio System saves the intermediate calculation operation configuration.

Resulting event: None Postcondition: Intermediate Calculation is created and is ready to be activated

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Start Intermediate Calculation

Use Case Identifier: 45 - Start Intermediate Calculation

Description: The Administrator enables a previously created intermediate calculation.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available intermediate calculations. 2. The Administrator enables the intermediate calculations to be activated. 3. The Radio System updates the mode of the intermediate calculation. 4. The Radio System begins to operate on the desired properties and stores the resulting calculation. 5. The Administrator or any authorized user of the data can access the calculation output.

Resulting event: None Postcondition: Calculation output is created and is ready to be viewed or used by another component in the system.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Stop Intermediate Calculation

Use Case Identifier: 46 - Stop Intermediate Calculation

Description: The Administrator disables a previously running intermediate calculation.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available intermediate calculations. 2. The Administrator disables the intermediate calculation that was previously running. 3. The Radio System updates the mode of the intermediate calculation. 4. The Radio System stops monitoring the desired properties. 5. The Administrator or any authorized user of the data can access the calculation output.

Resulting event: None Postcondition: Calculation output is created but no longer updating and is ready to be viewed or used by another component in the system.

Alternatives: None

Nonfunctional requirements: None

Comments: None

#### Delete Intermediate Calculation

Use Case Identifier: 47 - Delete Intermediate Calculation

Description: The Administrator deletes a previously created intermediate calculation

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator is logged into the system. Triggering event:

Main Flow: 1. The Administrator gets a list of available intermediate calculations 2. The Administrator deletes one of the intermediate calculations 3. The Radio System provides a list of any PM reports or other intermediate calculations that were dependent on the deleted intermediate calculation. 4. The Radio System removes the intermediate calculations from the radio storage. 5. The Radio System zeroes the resulting calculation location.

Resulting event: None Postcondition: Intermediate calculation is deleted.

Alternatives: None

Nonfunctional requirements: None

Comments: None

### Monitor Radio System

Figure 27: Monitor Radio System

#### Provide General Statistics

Use Case Identifier: 48 - Provide General Statistics

Description: The Operator accesses general Radio Set statistics.

External Actors: Operator

Related Use Cases:

Precondition: The Operator must be authorized to use the Radio Set and be privileged to access general statistics. Triggering event:

Main Flow: 1 The Operator is presented with a list of functions/statistics by the Radio Set for which the Operator has privilege to access (view or execute). 2 The Operator selects accessing general statistics. 3a The Radio Set supplies general statistics to the Operator. 4 The Operator has completed viewing general statistics and selects termination of the function. 7 The Radio Set notifies the Operator that the function has been terminated. [Continue at 1]

Resulting event: Postcondition: The Operator has accessed the desired general statistics and may continue or perform other operations.

Alternatives: 3b The Radio Set is unable to retrieve or route the general statistics or the function is not otherwise available 1 The Radio Set notifies the Operator that the requested function is not available.[Continue at 1] The Operator selects to cancel the operation 1 The Operator selects the cancel operation. 2 The Radio Set request verification of the cancel request. 3a The Operator verifies request. 3b The Operator aborts request. 4a The Radio Set terminates displaying general statistics [continue at 1]. 4b The Radio Set continues at the point prior to the Operator selecting the cancel operation.

Nonfunctional requirements: Privilege Rules : Operator must be authorized to view general statistics.

Comments:

Figure 28: Access general statistics

#### Provide OE Statistics

Use Case Identifier: 49 - Provide OE Statistics

Description: The Operator obtains Radio Set Operating Environment (OE) statistics.

External Actors: Operator

Related Use Cases:

Precondition: The Operator must be authorized to use the Radio Set and be privileged to access OE statistics. Triggering event:

Main Flow: 1 The Operator requests and is presented with a list of functions/statistics by the Radio Set for which the Operator has privilege to access (view or execute). 2a The Operator selects option to access OE statistics. 3a The Radio Set supplies OE statistics to the Operator. 4 The Operator has completed accessing OE statistics and selects termination of the function. 7 The Radio Set indicates to the Operator that the function has been terminated. [continue at 1]

Resulting event: Postcondition: The Operator has obtained the desired OE statistics and may continue or perform other operations.

Alternatives: 2b The Operator exits from OE Statistics monitoring 3b The Radio Set is unable to retrieve or route the OE statistics, or the function is not otherwise available 1 The Radio Set notifies the Operator that the requested function is not available[Continue at 1] The Operator selects to cancel the operation 1 The Operator selects the cancel operation. 2 The Radio Set requests verification of the cancel request. 3a The Operator verifies request. 3b The Operator aborts request. 4a The Radio Set terminates displaying OE statistics [continue at Main 1]. 4b The Radio Set continues at the point prior to the Operator selecting the cancel operation.

Nonfunctional requirements: Privilege Rules : Operator must be authorized to access OE statistics.

Comments:

Figure 29: Obtain OE statistics

#### Handle Special Monitoring Activity

Use Case Identifier: 50 - Handle Special Monitoring Activity

Description:

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

#### Provide Waveform Status

Use Case Identifier: 51 - Provide Waveform Status

Description: Provide status on an instantiated waveform.

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

#### Alarms

Figure 30: Alarms

##### Change Alarm Operational Mode

Use Case Identifier: 52 - Change Alarm Operational Mode

Description: Describes the use case steps for changing Alarm’s Operational (enable or disable) mode.

External Actors: Supporter

Related Use Cases: List Alarms

Precondition: Triggering event:

Main Flow: 1. Operator retrieves the alarm types from radio set. 2. Operator selects an alarm type to be enabled or disabled. 3. Radio Set changes alarm operational mode.

Resulting event: Postcondition: The Alarm is either in disable or enable operational mode.

Alternatives:

Nonfunctional requirements:

Comments: There can be different types of alarms that an radio set could understand.

##### List Alarms

Use Case Identifier: 53 - List Alarms

Description: List the Alarms in a radio set.

External Actors: Supporter

Related Use Cases:

Precondition: Triggering event: An actor requests a list of Alarms in the radio set.

Main Flow: 1. An actor requests a list of Alarms in the radio set. 2. The radio set returns a list Alarms configurations:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Modify Alarm

Use Case Identifier: 54 - Modify Alarm

Description: Describes the steps of modifying the alarm event for an alarm type within a radio set

External Actors: Operator

Related Use Cases: List Alarms

Precondition: Triggering event:

Main Flow: 1. Operator retrieves the default (active) alarm configuration from the radio. 2. Operator selects an event alarm to be modified. 3. Operator selects an alarm type by selecting from the available radio’s alarms repository. 4. Operator saves the changes

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements: None

Comments: Events are domain specific. There can be different types of alarms that an alarm service could understand.

##### Trigger Alarm

Use Case Identifier: 55 - Trigger Alarm

Description: Describes the steps of triggering an alarm within a radio set.

External Actors: Comm User

Related Use Cases:

Precondition: Triggering event: A radio set’s event (e.g., security violation, incoming call, time notice, etc.) has occurred.

Main Flow: 1. The radio set ( or a component within) detects an event condition. 2. The component that detected the event sends an event notification to the Alarm Service. 3. The Alarm Service triggers the event alarm message to the appropriate component (e.g., audio, button, generates an event) within the radio set.

Resulting event: Operator or radio system receives notice of the alarm. Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

#### Log Historical Data

Use Case Identifier: 56 - Log Historical Data

Description: The alarm condition, warnings and information messages of the registered producers is captured and logged. The logger only logs information while logging state is authenticated by the Administrator, Maintainer or Operator. The log level for the producer and consumer is also registered. The log message is based upon log data, current time, log level and producer identification and stores data into log file.

External Actors: Administrator, Maintainer or Operator

Related Use Cases:

Precondition: The Administrator, Maintainer or Operator has been authenticated by the Radio Set as having the attributes and the permissions per security rule. The Administrator, Maintainer or Operator has authenticated the logging state. The Administrator, Maintainer or Operator has registered each consumer and producer for logging level. Triggering event:

Main Flow:

Resulting event: Postcondition: The Radio Set returns to the operational condition.

Alternatives:

Nonfunctional requirements:

Comments:

#### View Historical Data

Use Case Identifier: 57 - View Historical Data

Description: View events and statistics captured by Log Historical Data.

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

#### Register to Receive Radio Events

Use Case Identifier: 58 - Register to Receive Radio Events

Description: An actor with an interest in certain events within the radio set/system requests to be notified when these events occur in/ are generated by the radio set/system.

External Actors: One of: Subscriber, Administrator, Operator, Maintainer, Security Officer

Related Use Cases:

Precondition: None Triggering event: An actor registers to receive radio events

Main Flow: 1. An actor registers to receive radio events. (The request indicates which event/s or event group/s the actor is interested in, and how the actor should be contacted when these occur.) 2. The radio set validates that the particular actor is permitted to register for all the specified event/s or event group/s. 3. The radio set validates that the particular actor is not already registered for any of the specified event/s or event group/s. 4. The radio set acknowledges the event registration request.

Resulting event: The radio set acknowledges the event registration request. Postcondition: The actor’s contact information is recorded against the specified event/s.

Alternatives: 2 a) Validation of the actor for one or more of the specified event/s or event groups/s fails. 3 a) 4.a) The radio set rejects the event registration request and no information is recorded against the specified events/s

Nonfunctional requirements:

Comments: Each type of actor is eligible to receive a pre-determined subset of radio events.

#### Notify Radio Event

Use Case Identifier: 59 - Notify Radio Event

Description: Notify all registered parties of the occurrence of a radio event from the radio set or a component within the radio set.

External Actors: One or more Subscribers, Administrators, Operators, Maintainers, or Security Officers (Comm User)

Related Use Cases:

Precondition: Triggering event: Any radio event.

Main Flow: 1. A radio event occurs. 2. The radio set contacts all actors who have registered an interest in the event, or in an event group to which the event belongs, using the contact information recorded for each actor.

Resulting event: Postcondition:

Alternatives: 2 a) A registered actor cannot be contacted using the recoded contact information. The radio set logs an error and continues to contact other registered actors.

Nonfunctional requirements:

Comments: This can be due to hardware or radio set state changes, radio set domain changes as items are removed or added, or threshold changes.

#### Unregister to Receive Radio Events

Use Case Identifier: 60 - Unregister to Receive Radio Events

Description: An actor who previously registered an interest in certain events within the radio set/system requests to no longer be notified when these events occur.

External Actors: One of: Subscriber, Administrator, Operator, Maintainer, Security Officer (Comm User)

Related Use Cases:

Precondition: The actor’s contact information is recorded against specified event/s. Triggering event: An actor unregisters to receive radio events

Main Flow: 1. An actor unregisters to receive radio events. (The request indicates which event/s or event group/s the actor wishes to unregister.) 2. The radio set validates that the particular actor is currently registered for all of the specified event/s or event group/s. 3. The radio set acknowledges the event unregister request.

Resulting event: The radio set acknowledges the event unregister request. Postcondition: No contact information for the actor is recorded against the specified event/s.

Alternatives: 2 a) The actor is not already registered for all specified events. 3.a) The radio set rejects the event unregister request and no change is made to the information recorded against the specified events/s

Nonfunctional requirements:

Comments:

### Manage Configuration

Figure 31: Manage Configuration

#### Manage Physical Configuration

Figure 32: Manage Physical Configuration

##### Install New Hardware Driver(s)

Use Case Identifier: 61 - Install New Hardware Driver(s)

Description: The Maintainer prepares the Radio Set for installing a new hardware resource. Upon Maintainer request, the Radio Set will guide the Maintainer through the process of installing the necessary drivers. At the end of the dialog, the Radio Set will bring the new hardware resource up and will start communicating with it by setting up resources that are provided as defined by the Domain profile.

External Actors: Maintainer

Related Use Cases: Initiate Local Software Download, Determine Co-Site Potential, List Device Drivers

Precondition: The Maintainer has been authenticated by the Radio Set as having the attributes and permissions given to a Maintainer. The Maintainer has installed new hardware device(s) into the Radio Set per Hardware Installation Procedures, and powered the Radio Set per the Startup use case. The Radio Set will have brought up the main interface option page for the Maintainer. Hardware install may require tamper event prior to install. If so, all cryptographic keys will need to be reloaded. Triggering event:

Main Flow: 1 Maintainer selects Maintain Physical Configuration 2 Maintainer selects the option Install New Hardware Device Driver 3a Radio Set prompts Maintainer to supply software driver(s) for newly installed hardware device(s) and Maintainer supplies the software; (Note: Radio Set attempts automatic detect of driver software but finds none to be applicable.) 4 Radio Set uses the Initiate Local Software Download use case to install the software driver 5a Radio Set verifies driver configuration and compatibility with the new hardware device 6 Radio Set uses Determine Co-Site Potential use case to establish the probability that this change will cause cosite interference 7a Radio Set informs Maintainer that the device is ready for use 8 Radio Set displays the main interface option page for the Maintainer

Resulting event: Postcondition: The Maintainer has performed the configuration management functions and the Radio Set will have established communication with the newly installed hardware as well as make its functionality available to the Operator. The Radio Set returns to the main interface option page for the Maintainer.

Alternatives: 3b Radio Set detects an existing installed driver for device(s) 1 Radio Set uses List Device Drivers use case to display existing device drivers and prompts Maintainer to select a replacement for the existing driver 2 [continue at 5] 5b The Radio Set requires a reboot to use the new driver 1 The Radio Set informs the Operator that a reboot is required. 2 The Maintainer selects continue 3 The Radio Set stores driver installation state information 4 The Radio Set reboots [continue at 6] 5c The Radio Set is unable to establish communications with the new hardware. 1 Radio Set informs Maintainer that the installed hardware resource does not respond 2 Maintainer selects retry [continue at 3] or [end of case] 5d The Radio Set determines that the driver configuration or device compatibility is not correct 1 Radio Set informs Maintainer that the installed driver is not correct [continue at 3a] 7b The Co-site potential of the new hardware is deemed unacceptable by the Maintainer based on information returned by the Determine Co-Site Potential use case. 1 Maintainer selects retry [continue at 3] or [end of case] Maintainer may cancel Manage Physical Configuration at any time [success] Maintainer may exit an operation that has stalled at any time [error]

Nonfunctional requirements:

Comments:

Figure 33: Installation of a new hardware device

##### List Hardware

Use Case Identifier: 62 - List Hardware

Description: List the status of all hardware in a radio set.

External Actors: One of: Maintainer, Operator, Administrator

Related Use Cases: Get Hardware Configuration

Precondition: Triggering event: An actor requests a list of hardware devices in the radio set.

Main Flow: 1. The radio set obtains the status of each hardware device in the system. 2. The radio set returns a list of hardware devices in the system and their charateristics. Types of characteristics are: - Hierarchy of nodes and device relationships - State information (administrative, operational, usage) - Configuration attributes

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Change Comm Hardware Administrative State

Use Case Identifier: 63 - Change Comm Hardware Administrative State

Description: This use case addresses the ability of an actor to change a hardware administrative state. The administrative state indicates the permission to use or prohibition against using the hardware.

External Actors: Comm Supporter

Related Use Cases:

Precondition: Hardware is either in an administrative state of UNLOCKED, Transitioning to LOCKED, or LOCKED. Triggering event:

Main Flow: 1. An actor requests a change in the administrative mode of operation for a hardware. 2. The hardware transitions to either UNLOCKED, Transitioning to SHUTTING_DOWN or LOCKED depending of the state it is in as shown in the following Figure. Figure. State Transition Diagram for Hardware’s Administrative State 3. The Hardware sends a state change event indicating the administrative state the hardware is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Hardware. - The sourceId field is the identifier attribute of the Hardware. - The stateChangeCategory field is ADMINISTRATIVE_STATE_EVENT. - The stateChangeFrom and stateChangeTo fields shall reflect the administrative state before and after the state change, respectively.

Resulting event: Postcondition: Communication Channel is in either UNLOCKED, SHUTTING_DOWN, or LOCKED.

Alternatives: 3a) No event is sent when there is no change in administrative state.

Nonfunctional requirements:

Comments: When the Communication Channel’s administrative state is LOCKED, it cannot be instantiated for waveform usage. The Administrative States are based upon CCITT (the International Telegraph and Telephone Consultative Committee), which is a permanent organ of ITT (International Telephone Union) Recommendation X.731 that was approved on Jan 17th 1992. This is also published as ISO/IEC International Standard 10164-2.

##### Add Hardware

Use Case Identifier: 64 - Add Hardware

Description: Hardware is added to an existing radio set.

External Actors: Maintainer

Related Use Cases: Perform Built In Test, Enable Hardware, Notify Radio Event

Precondition: Triggering event: One of: - The radio system detects that new hardware has been added. - The new hardware proactively notifies the radio set that it has been added. - The maintainer notifies the radio set that new hardware has been added.

Main Flow: 1. The radio system identifies the new hardware. 2. The radio system verifies that no incompatibilities exist between the new and pre-existing hardware. 3. The radio system instructs the new hardware to perform its power-on Built In Test. 4. The radio system verifies that all software required by the new hardware is available. 5. The radio system enables the hardware as defined in Enable Hardware Use Case.

Resulting event: An appropriate radio event is generated. Postcondition: The new hardware is available for use by applications.

Alternatives: 1 a) The new hardware cannot be identified - An appropriate failure radio event is generated. 2 a) Incompatibilities are detected between the new and pre-existing hardware. - The radio system disables the conflicting pre-existing hardware as defined in Disable Hardware Use Case - An appropriate failure radio event is generated. 3 a) The new hardware fails its power-on Built In Test. - An appropriate failure radio event is generated. 4 a) Not all software required by the new hardware is available. - An appropriate failure radio event is generated.

Nonfunctional requirements:

Comments: - Hardware may be added to a functioning radio set (Hotswap). - It is assumed that correctly functioning hardware provides interfaces via which it may be identified and instructed to conduct its Built In Test. - It is assumed that the new hardware initializes itself in the disabled state.

##### Set Hardware Configuration

Use Case Identifier: 65 - Set Hardware Configuration

Description: This use case addresses the ability of an actor to configure hardware.

External Actors: Supporter

Related Use Cases:

Precondition: Triggering event:

Main Flow: 1. An actor requests configuration changes to a hardware. 2. Hardware performs requested configuration changes.

Resulting event: Postcondition: Hardware configuration has been modified.

Alternatives: 2a) No configuration is performed, the configuration parameters are invalid for the hardware. 2b) Partial configuration is performed, Some configuration parameters are invalid for the Hardware Component.

Nonfunctional requirements:

Comments:

##### Get Hardware Configuration

Use Case Identifier: 66 - Get Hardware Configuration

Description: This use case addresses the ability of an actor to retrieve configure hardware

External Actors: Supporter

Related Use Cases:

Precondition: Triggering event:

Main Flow: 1. An actor requests configuration information from a hardware. 2. Hardware returns requested configuration information

Resulting event: Postcondition:

Alternatives: 2.a) No configuration is returned, the query parameters are invalid for the hardware. 2.b) Partial configuration is returned, some query parameters are invalid for the Hardware Component.

Nonfunctional requirements:

Comments:

##### Remove Hardware

Use Case Identifier: 67 - Remove Hardware

Description: Hardware is removed from a radio set.

External Actors: Maintainer

Related Use Cases: Remove Application, Change Hardware Administrative State, Notify Radio Event

Precondition: Triggering event: The maintainer notifies the radio set that hardware devices are to be removed.

Main Flow: 1. The radio system determines if removal if these devices will impact any other hardware. 2. The radio system removes (or redeploys) any applications that are making use of the hardware device to be removed, or any impacted hardware devices, as described in the Remove Application Use Case. 3. The radio system disables the hardware devices to be removed as well as any impacted hardware devices as described in Disable Hardware Use Case.

Resulting event: An appropriate radio event is generated. (The maintainer uses this event to determine that it is safe to power down/physically remove the hardware if required). Postcondition: The radio set continues to operate as permitted by the remaining hardware.

Alternatives:

Nonfunctional requirements:

Comments: 1. It is assumed that the hardware will be removed irrespective of whether applications, which are using, it are successfully removed or redeployed. 2. It is assumed that the hardware will be removed irrespective of whether it can be successfully disabled. 3. It may not (Hotswap case) be necessary to power down the radio set in order to physically remove the hardware.

##### Perform Hardware BIT

Use Case Identifier: 68 - Perform Hardware BIT

Description: An actor performs hardware BIT

External Actors: Maintainer

Related Use Cases: Change Hardware Administrative State

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Enable Hardware

Use Case Identifier: 69 - Enable Hardware

Description: Enable a hardware device in a radio set.

External Actors: One of: Maintainer, Operator, Administrator

Related Use Cases: None

Precondition: The hardware device is disabled. Triggering event: An actor requests that a hardware device be enabled.

Main Flow: 1. The operational state of the device is set to ENABLED.

Resulting event: An appropriate radio event is generated. Postcondition: The operational state of the device is set to ENABLED.

Alternatives:

Nonfunctional requirements:

Comments:

##### Disable Hardware

Use Case Identifier: 70 - Disable Hardware

Description: Disable a hardware device in a radio set.

External Actors: One of: Maintainer, Operator, Administrator

Related Use Cases: None

Precondition: The hardware device is enabled. Triggering event: An actor requests that a hardware device be disabled.

Main Flow: 1. The operational state of the device is set to ENABLED.

Resulting event: An appropriate radio event is generated. Postcondition: The new hardware is available for use by applications.

Alternatives:

Nonfunctional requirements:

Comments:

##### Determine Co-Site Potential

Use Case Identifier: 71 - Determine Co-Site Potential

Description: The Radio Set determines the probability for Co-Site interference when a change in physical configuration occurs due to actions of external actor. The Radio Set will notify the Maintainer (and other actors if affected) of this potential interference and provide indication of expected outcome under the condition.

External Actors: Maintainer

Related Use Cases: Install New Hardware Driver(s)

Precondition: The Maintainer has been authenticated by the system as having the attributes and permissions given to a Maintainer. The Radio Set has begun to process a physical configuration change request per the Install New Hardware use case. Triggering event:

Main Flow: 1 Radio Set analyzes the requested configuration change and determines that it would cause potential for co-site interference 2 Radio Set notifies the Maintainer of the condition 3a Maintainer selects to continue with the configuration change

Resulting event: Postcondition: The Radio Set notifies the Maintainer of a potential co-site interference condition.

Alternatives: 3b Maintainer selects to abort the configuration change 1 Radio Set aborts the configuration change [end of case] Maintainer may direct the Radio Set to undo the current action [success] Maintainer may exit an operation that has stalled at any time [error]

Nonfunctional requirements:

Comments:

Figure 34: Confirmation of configuration change

#### Manage Software Configuration

Figure 35: Manage Software Configuration

##### Initiate Local Software Download

Use Case Identifier: 72 - Initiate Local Software Download

Description: The Radio Set receives a download of new OE or Application software from a local interface and installs or modifies applications on the system accordingly.

External Actors: Administrator

Related Use Cases: System Reset

Precondition: The Administrator has been authenticated by the system as having the attributes and permissions given to an Administrator. The Radio Set brings up the main option interface page for the Administrator. Download device is connected to local interface. Triggering event:

Main Flow: 1 Administrator selects Manage Software Configuration 2 Administrator selects option to Install Software 3 Radio Set displays a list of interfaces for the download 4a Administrator selects a specific download interface from list 5a Radio Set successfully establishes communications with the interface 6a Radio Set receives the complete download from the device 7a Radio Set checks software integrity and authenticates the downloaded software per security rule. 8a Radio Set determines that the downloaded software is Application software and performs relevant installation or modification to existing application(s) 9 Radio Set notifies Administrator that application software installation is complete 10 Radio Set displays the main option interface page for the Administrator

Resulting event: Postcondition: The Radio Set returns to the main option interface page for the Administrator.

Alternatives: 4b Administrator selects the option "File previously loaded" from list 1 Radio Set displays a list of internally stored files ready for installation and Administrator selects the file(s) to be installed [continue at 7a] 5b Radio Set notifies Administrator that communication is not established with the interface and prompts for action(s) 1 Administrator selects the option Retry 2 [continue at 10] 3 Administrator selects the option Cancel 4 [continue at 10] 6b Radio Set notifies Administrator that the software download failed to be loaded onto the system and prompts for action. 1 Administrator selects the option Quit 2 [continue at 10] 3 Administrator selects the option Repair 4 Radio Set displays type of error and uses Manage Software Configuration Scenarios to repair system 5 [continue at 10] 7b Radio Set displays that the security authentication failed 1 The Radio Set logs audit data for failed software download 2 Administrator selects the option Continue 3 [continue at 10] 8b Radio Set determines that the downloaded software is Operating Environment Software 1 Radio Set notifies Administrator that Operating Environment Software is to be installed or modified 2 Administrator acknowledges and installation begins 3 Radio Set completes installation or modification of Operating Environment software 4 Radio Set uses System Reset use case to reboot system 5 Radio Set displays load Operating Environment complete 6 [continue at 10] 10b Radio Set fails to install software application 1 Radio Set logs failed install software attempt 2 [continue at 10] Administrator may cancel Manage Software Configuration at any time [success] Administrator may exit an operation that has stalled at any time [error]

Nonfunctional requirements: Download Security Rules : Software downloads are governed by a domain specific security policy. Radio Set logs (to audit database) the source non-repudiation data as required by the security policy. Radio Set provides sink non-repudiation data to Administrator as required by the security policy. Radio Set encrypts software files for local storage as required by the security policy. Domain Registration Rules : All applications are accompanied by an application profile. Each application profile contains information that describes the application’s hardware dependencies.

Comments:

Figure 36: Installation of an application software

Figure 37: Installation of OE

##### Initiate Solicited OTAP Software Download

Use Case Identifier: 73 - Initiate Solicited OTAP Software Download

Description: The Radio Set receives data over the air from External Radio Set and determines that the data contains a solicited software file(s) intended to update/modify Radio Set functionality. The Radio Set authenticates the file(s) using means of cryptographic key (i.e., the authentication data accompanies and is cryptographically bound to the transferred file). The Radio Set stores the received file(s) for software installation. The Radio Set returns any non-repudiation of receipt data to the Operator if required by the domain specific security policy.

External Actors: Administrator

Related Use Cases: Establish Waveform Communication Channel, Initiate Local Software Download

Precondition: The Administrator has been authenticated by the system as having the attributes and permissions given to an Administrator. The Radio Set brings up the main option interface page for the Administrator. Triggering event:

Main Flow: 1 Administrator uses Radio Set to negotiate with the External Radio Set to set up for Over The Air Programming(OTAP) file transfer 2a Administrator uses the Establish Waveform Communication Channel use case to receive subsequent OTAP file(s) over secure (encrypted) communication link. 3a Radio Set uses security functions to decrypt, check integrity and authenticate source of the downloaded software per Security Rules. 4a Radio Set confirms OTAP file is same as requested file(s) and saves to local storage 5 Radio Set provides non-repudiation of receipt information to External Radio Set as required by the domain specific security rule 6 Radio Set uses Initiate Local Software Download use case to install OTAP received file(s) 7 Radio Set displays the main option interface page for the Administrator

Resulting event: Postcondition: The Radio Set has installed files and responds to the initiating system with the status of the operation and the Radio Set continues with its normal idle operation.

Alternatives: 2b Radio Set does not receive an OTAP within a predetermined timeout interval 1 Administrator selects the option Retry 2 [continue at 1] 3 Administrator selects the option Cancel 4 [continue at 7] 3b Radio Set displays that the security authentication failed 1 Administrator selects the option Continue 2 Radio Set logs auditable event as required by the domain specific security policy 3 [continue at 7] 4b Radio Set determines OTAP application filename is not the requested filename 1 Administrator selects the option Retry 2 Radio Set logs auditable event as required by the domain specific security policy 3 [continue at 1] 4 Administrator selects the option Cancel 5 [continue at 7] External Radio Set may cancel Initiate OTAP Software Download at any time [success]

Nonfunctional requirements: Download Security Rules : Software downloads are governed by a domain specific security policy. Radio Set logs (to audit database) the source non-repudiation data as required by the security policy. Radio Set provides sink non-repudiation data to Administrator as required by the security policy. Radio Set encrypts software files for storage as required by the security policy.

Comments:

Figure 38: Install software using initiate OTAP

##### Initiate Unsolicited OTAP SW Download

Use Case Identifier: 74 - Initiate Unsolicited OTAP SW Download

Description:

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Transmit OTAP

Use Case Identifier: 75 - Transmit OTAP

Description: This use case is applied when a request is received from a remote location via the External Radio Set to transmit a program via OTAP to that System (solicited case). This use case is also used when the Maintainer initiates a transmit to the remote location via External Radio Set to manage its software configuration (unsolicited case).

External Actors: Maintainer

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Application Management

###### Uninstall Application

Use Case Identifier: 76 - Uninstall Application

Description: The Radio Set upon authenticated command will remove a selected application from the system mass storage in addition to all of the references to the application in any profiles that are kept on the Radio Set.

External Actors: Administrator

Related Use Cases: List Installed Applications

Precondition: The Administrator must have logged onto the Radio Set and have been authenticated by the system as having the attributes and permissions given to an Administrator. The Radio Set displays the main option interface page for the Administrator. Triggering event:

Main Flow: 1 Administrator selects maintain software configuration 2 Administrator selects option to uninstall an existing software application 3 Radio Set uses List Installed Applications to display a list of installed software applications to the Administrator 4 Administrator selects the application to uninstall 5a Radio Set requests the Administrator for confirmation to perform uninstall action 6 Radio Set removes the application and all references to that software application from the domain and logs an auditable event as required by the domain specific security policy 7a Radio Set displays status of the uninstall action to the Administrator 8 Radio Set displays the main option interface page for the Administrator

Resulting event: Postcondition: The Radio Set has removed the selected application and references to that application and returns the Administrator to the main option interface page for the Administrator.

Alternatives: 5b Radio Set request confirmation to perform action 1 Administrator selects the option Cancel 2 [continue at 8] 7b Radio Set displays that the uninstall option failed 1 Radio Set requests from the Administrator whether to retry or quit 2 Administrator selects the option Retry 3 [continue at 6] 4 Administrator selects the option Quit 5 [continue at 8] Administrator may cancel Uninstall Software at any time [success] Administrator may exit an operation that has stalled at any time [error]

Nonfunctional requirements:

Comments:

###### Remove Application

Use Case Identifier: 77 - Remove Application

Description: This use case describes the steps for uninstalling an application such as waveform from the Radio Set.

External Actors: Supporter

Related Use Cases: List Installed Applications, Notify Radio Event

Precondition: The Radio is operational. Triggering event: The Supporter selects an application to be uninstalled from the Radio

Main Flow: 1. The actor retrieves a list of installed applications from the Radio Set. 2. The actor selects an application to be uninstalled from the Radio Set. 3. The Radio Set uninstalls the application from the radio, so no more application instances can be created from the application. 4. The Radio Set removes all the application files from its file system. 5. An Event is sent indicating the Application that was removed.

Resulting event: The Radio sends an event indicating which application has been uninstalled from this radio. Postcondition: The Application factory deployer component and its associated files are removed from radio set.

Alternatives:

Nonfunctional requirements:

Comments:

Figure 39: Uninstall an existing software application

###### List Installed Applications

Use Case Identifier: 78 - List Installed Applications

Description: The Radio Set will provide the ability to display a list of applications along with the major attributes that are associated with each application that exists on the system. Managing the application must be done by an Administrator that has the proper authority for such actions.

External Actors: Administrator

Related Use Cases: None

Precondition: The Administrator must have logged onto the Radio Set and have been authenticated by the system as having the attributes and permissions given to an Administrator. The Radio Set will have brought up the main interface option page for the Administrator. Triggering event:

Main Flow: 1 Administrator selects maintain software configuration 2 Administrator selects option List 3 Radio Set uses the File Manager to provide a list of the software applications on the system 4a Radio Set displays a list of Applications that exist on the system. Display includes: Date of install, version, and size 5 Radio Set displays main option interface page for the Administrator

Resulting event: Postcondition: The Radio Set has displayed a list of installed applications as required and returns the system to the main interface option page for the Administrator.

Alternatives: 4b Radio Set presents options to the Administrator 1 Administrator selects the option Quit [continue at 5] 2 [continue at 5] 3 Administrator selects an Application followed by the option Detail 4 Radio Set uses the File Manager to provide the detailed list of software applications 5 Radio Set displays the Application name and software that make up the Application with the attributes: of Version, Size, and any Associations for each of the software application. 6 Administrator selects the option Continue 7 [continue at 5] Administrator may cancel manage software list applications at any time Administrator may exit an operation that has stalled at any time

Nonfunctional requirements:

Comments:

Use Case Identifier: 79 - List Applications

Description: This use case addresses the ability of an actor to retrieve installed applications from the radio.

External Actors: Comm Supporter

Related Use Cases:

Precondition: Radio is Operational Triggering event:

Main Flow: 1. An actor requests installed applications information from the Radio. 2. The radio returns a list of installed applications, along with their characteristics.

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

Figure 40: List installed applications

###### List Device Drivers

Use Case Identifier: 80 - List Device Drivers

Description:

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

#### Configure Radio

Figure 41: Configure Radio

##### Get Radio Configuration

Use Case Identifier: 81 - Get Radio Configuration

Description: This use case addresses the ability of an actor to retrieve radio configuration information and radio’s attributes.

External Actors: Comm Supporter

Related Use Cases: List Deployment Plans, List Instantiated Applications, List Installed Applications, List Communication Channels, List Hardware

Precondition: Radio is Operational Triggering event:

Main Flow: 1. An actor requests Radio information. 2. The radio returns configuration information based upon the request. Types of requests can be: - Radio Identification - Radio Time - Location - Instantiated Waveforms - Communication Channels - Operational State - Usage State - Administrative State - Installed Waveforms - Log - File System - Devices - Deployment Plans

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Set Radio Configuration

Use Case Identifier: 82 - Set Radio Configuration

Description: This use case addresses the ability of an actor to set radio configuration information of the Radio Set.

External Actors: Supporter

Related Use Cases: None

Precondition: Radio set is Operational Triggering event:

Main Flow: 1. An actor requests Radio Set information. 2. The radio set returns configuration information based upon the request. Types of requests can be: - Radio Identification - Radio Time

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Communications Channel Control

Figure 42: Communications Channel Control

###### Establish Waveform Communication Channel

Use Case Identifier: 83 - Establish Waveform Communication Channel

Description: A Operator-specified waveform is loaded and assigned to a radio channel.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator must have been granted access to the system and authorized to establish a communication channel. The waveforms in the list of installed waveforms have been installed per the Manage Software Configuration use case. Triggering event:

Main Flow: 1 The Operator requests that the Radio Set establish a waveform communication channel. 2a The Radio Set presents a list of the installed waveforms in its Domain according to Installed Waveform Rules. 3 The Operator selects a waveform from the list and assigns it to a channel. 4a The Radio Set determines that the software resources required to support the waveform are installed in the Domain according to Installed Software rules. 5a The Radio Set determines that the devices required to support the waveform are installed in the Domain according to Installed Device Rules. 6a The Radio Set determines that the installed devices are currently not allocated to another waveform. 7a The Radio Set loads the waveform software onto the allocated devices. 8a The Radio Set instantiates the set of resources that comprise the waveform 9 The Radio Set establishes a communications path utilizing the resources. 10 The Radio Set checks the integrity of the activated channel and it passes all required tests. 11 The Radio Set indicates to the Operator that the activated channel is operational and ready for use.[end of case]

Resulting event: Postcondition: The waveform is instantiated to a radio channel and is ready for configuration by the Operator.

Alternatives: 4b The software resource(s) are not installed in the Radio Set’s domain 1 The Radio Set notifies the Operator that the software resource(s) required by the waveform are not installed [continue at 2]. 5b The Radio Set determines that the required devices are not installed. 1 The Radio Set notifies the Operator that the device(s) required by the waveform are not installed [continue at 2]. 6b The Radio Set determines that the required devices are currently allocated to another waveform of lower priority. 1 The Radio Set requests confirmation from the Operator that it is acceptable to remove the lower priority communication channel. 2 The Operator confirms request to preempt. 3 The Radio Set performs the de-allocation using Configure Radio. 4 The Radio Set notifies the Operator that the de-allocation is complete. [continue at 7a] 6c The Radio Set determines that the required devices are currently allocated to another waveform of equal or higher priority. 1 The Radio Set notifies Operator that the requested communication channel can not be realized due to resource/device conflicts. [continue at 2.] 7b The Radio Set encounters an error when loading the waveform SW. 1 The Radio Set notifies the Operator that an error occurred when loading the waveform SW. [continue at 2] 8b The waveform instantiation fails (a resource could not be created). 1 The Radio Set notifies the Operator that the load failed [continue at 2]. 10a The Radio Set checks the integrity of the activated channel and it fails critical test(s). 1 The Radio Set notifies the Operator of the channel failure and prompts the Operator for Retry/Abort. 2 The Operator aborts. 3 The Radio Set cancels events performed thus far in this flow. [end of case] 6b, 2b The Operator aborts the request to preempt. [continue at 2] The Operator can cancel at any time.

Nonfunctional requirements: Installed Waveform Rules : Installed waveforms are represented in the Application Profile. The characteristics of a waveform are represented in an Application Profile for the waveform. Installed Software Rules The software resources required by a waveform are represented in the Resource Profile The characteristics of a software resource are represented in a Profile for the software resource. Installed Device Rules The devices required by the waveform software are defined by the Application Profile for the software application. The devices installed in the Radio Set are represented in the Device Profile. Device properties are represented in a Device Profile.

Comments:

Figure 43: Nominal activation of a channel

###### Release Waveform Communication Channel

Use Case Identifier: 84 - Release Waveform Communication Channel

Description: The Operator chooses to remove an established communication channel.

External Actors: Operator

Related Use Cases: Stop Waveform

Precondition: The Operator must have been granted access to the system (logged in). The waveform must have already been stopped by Stop Waveform. Triggering event:

Main Flow: 1 The Operator selects Release Waveform Communication Channel 2 The Radio Set presents the Operator with the list of active channels according to Privilege Rules. 3 The Operator selects the waveform channel to release. 4 The Radio Set disconnects the software resources assigned to the waveform 5a The Radio Set disengages the software resources assigned to the waveform 6 The Radio Set releases the physical resources assigned to the waveform 7 The Radio Set unloads the waveform software 8 The Radio Set informs the Operator that the operation is complete.

Resulting event: Postcondition: The channel is removed and the associated resources are freed.

Alternatives: 5b One or more software resources are being used by another waveform channel. 1 The Radio Set does not release those resources [continue at 6].

Nonfunctional requirements: Privilege Rules : The Operator is only presented the waveform channels that have been stopped by the Stop Waveform command, and for which it has access rights.

Comments:

Figure 44: Release a waveform communication channel

##### Configure Waveform

Figure 45: Configure Waveform

###### Set Waveform Parameters

Use Case Identifier: 85 - Set Waveform Parameters

Description: The Operator Actor sets the parameters for an existing waveform on a channel.

External Actors: Operator

Related Use Cases: Establish a Waveform Communication Channel

Precondition: The waveform must already have been loaded and instantiated with the Establish a Waveform Communication Channel use case. Triggering event:

Main Flow: 1 The Operator selects a Radio Set channel to configure according to Channel Access Rules. 2 The Radio Set presents the configuration parameters (frequency, power level, bit rate etc.) of the waveform assigned to the channel to the Operator. 3 The Operator sets one or more of the parameters. 4a The Radio Set accepts the parameters and configures the resources accordingly. 5 The Radio Set indicates to the Operator that the waveform has been configured

Resulting event: Postcondition: The waveform is fully configured and is ready for by the Operator Actor in the Transmit/Receive use case.

Alternatives: 4b The Radio Set rejects the parameter(s) 1 The Radio Set notifies the Operator the parameter(s) have been rejected

Nonfunctional requirements: Channel Access Rules : The Operator is only presented the channels for which the Operator has access rights. Waveform Parameter Rules The waveform parameters presented to the Operator are based on the parameters defined by the application profile of the installed waveform software. Parameters can be presented as pre-defined configuration sets (presets). The limits and granularity of each parameter are defined by the application profile for the installed waveform software and cannot be changed by the Operator. The Radio Set notifies the Operator if any waveform parameters are not supported by the Operator interface. The Operator has the option of disabling this notification. The Radio Set disables any parameters in the Operator interface that are not supported by the waveform.

Comments:

Figure 46: Configure a waveform

###### Start Waveform

Use Case Identifier: 86 - Start Waveform

Description: The Operator chooses to start a loaded waveform.

External Actors: Operator

Related Use Cases: Set Waveform Parameters

Precondition: The Operator must have been granted access to the system (authenticated). The waveform must have already been configured by Set Waveform Parameters. Triggering event:

Main Flow: 1 The Operator selects the waveform channel to start. 2 The Radio Set signals the waveform resources to allow processing of communications traffic.

Resulting event: Postcondition: The waveform is started and ready to process communications traffic.

Alternatives:

Nonfunctional requirements: The Operator is only presented the waveform channels for which the Operator has access rights.

Comments:

Figure 47: Start of a waveform channel

###### Stop Waveform

Use Case Identifier: 87 - Stop Waveform

Description: The Operator chooses to prevent a waveform from processing communications traffic.

External Actors:

Related Use Cases: Start Waveform

Precondition: The Operator must have been granted access to the system (authenticated). The waveform must have already been started by Start Waveform. Triggering event:

Main Flow: 1 The Operator selects the waveform channel to stop. 2 The Radio Set signals the waveform resources to stop processing of communications traffic.

Resulting event: Postcondition: The waveform is stopped and not processing communications traffic.

Alternatives:

Nonfunctional requirements: The Operator is only presented the waveform channels for which it has access rights.

Comments:

Figure 48: Stop of a waveform channel

##### Configure Retransmission

Figure 49: Configure Retransmission

###### Configure Repeater

Use Case Identifier: 88 - Configure Repeater

Description: The Operator instantiates a repeater between two External Radio Sets.

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

###### Configure Bridge

Use Case Identifier: 89 - Configure Bridge

Description: The Operator instantiates and configures a bridge between two waveforms.

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

###### Configure Router

Use Case Identifier: 90 - Configure Router

Description: The Operator configures a router.

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Configure Gateway

Use Case Identifier: 91 - Configure Gateway

Description: The Operator instantiates and configures a gateway between two waveforms.

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

##### Determine Cosite Potential

Use Case Identifier: 92 - Determine Cosite Potential

Description: The Radio Set will attempt to detect any potential of Cosite interference that may exist whenever any information is entered into the system that modifies or adds any selectable component that may aggravate the cosite problems within the system. The Radio Set will warn the Operator of this potential.

External Actors: Operator

Related Use Cases:

Precondition: The Operator performs an action that causes clock speeds, frequencies, or other R/F related parameters to be changed or added to the system. Triggering event:

Main Flow:

Resulting event: Postcondition: The Radio Set will warn the Operator of the cosite potential and allow the Operator to continue with the desired operation after providing the Operator with the option of undoing the potential problem.

Alternatives: Operator may direct the Radio Set to undo the current action Operator may exit an operation that has stalled at any time [error]

Nonfunctional requirements:

Comments:

Figure 50: Continue with desired operation

##### Deployment Plan

Figure 51: Deployment Plan

###### List Deployment Plans

Use Case Identifier: 93 - List Deployment Plans

Description: This use case addresses the ability of an actor to retrieve Deployment Plans from a radio set.

External Actors: Supporter

Related Use Cases:

Precondition: Radio Set is Operational Triggering event:

Main Flow: 1. An actor requests deployment plans information from the Radio Set. 2. The radio set returns a list of deployment plans, along with their characteristics.

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System

###### Enable Deployment Plan

Use Case Identifier: 94 - Enable Deployment Plan

Description: This use case addresses the ability of an actor to enable a Deployment Plan within a radio set.

External Actors: Supporter

Related Use Cases: List Deployment Plans

Precondition: Radio Set is Operational Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System

###### Disable Deployment Plan

Use Case Identifier: 95 - Disable Deployment Plan

Description: This use case addresses the ability of an actor to disable a Deployment Plan within a radio set.

External Actors: Supporter

Related Use Cases: List Deployment Plans

Precondition: Radio Set is Operational Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System.

###### Add Deployment Plan

Use Case Identifier: 96 - Add Deployment Plan

Description: This use case addresses the ability of an actor to add a Deployment Plan from a radio set.

External Actors: Supporter

Related Use Cases: Notify Radio Event

Precondition: Radio Set is Operational Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System

###### Remove Deployment Plan

Use Case Identifier: 97 - Remove Deployment Plan

Description: This use case addresses the ability of an actor to remove a Deployment Plan within a radio set.

External Actors: Supporter

Related Use Cases: Notify Radio Event

Precondition: Radio Set is Operational Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System

## Manage Security

Figure 52: Manage Security

### Manage Operator Profile

Figure 53: Manage Operator Profile

#### Define Operator

Use Case Identifier: 98 - Define Operator

Description: The Security Officer defines a Operator based on an authentication profile and a privilege set. Example privileges include ability to download a waveform, download a new CF, instantiate a waveform, instantiate a CF, execute a key fill, or setup a communication path.

External Actors: Security Officer

Related Use Cases: None

Precondition: The Security Officer must have logged onto the Radio Set and been authenticated by the Radio Set as a valid Security Officer. The Radio Set presents a list of authorized functions for the Security Officer type. Triggering event: The Security Officer selects Define Operator.

Main Flow: The Radio Set presents a list of authorized functions for the Security Officer type. 2 The Security Officer selects Define Operator. 3 The Radio Set requests Operator name and authentication profile. 4a The Security Officer enters the desired Operator name [freeform] and authentication profile [list]. 5 The Radio Set presents a list of authorized privileges for the authentication profile. 6 The Security Officer selects the desired privilege(s) for the Operator. The Radio Set builds a Operator identification based on the Operator name, authentication profile and privilege set, then notifies Security Officer via the HMI that a new Operator has been successfully defined.

Resulting event: Postcondition: The Security Officer has completed the definition of a Operator.

Alternatives: 4b The Security Officer provides an invalid Operator name. 1 The Radio Set rejects the invalid Operator name. [continue at 3] The Security Officer cancels the function requested. This may be done at any point up to entering the Operator privileges.

Nonfunctional requirements:

Comments: The Operator privileges are defined by the authentication profile for Operator.

Figure 54: A new operator is successfully defined

#### Modify Operator

Use Case Identifier: 99 - Modify Operator

Description: The Security Officer chooses to modify a Operator. The modification can include Operator name, authentication profile, or privilege set.

External Actors: Security Officer

Related Use Cases: None

Precondition: The Security Officer must have logged onto the Radio Set and been authenticated by the Radio Set as a valid Security Officer and a valid Operator is already defined. The Radio Set presents a list of authorized functions for the Security Officer type. Triggering event: The Security Officer selects Modify Operator.

Main Flow: 1 2 The Security Officer selects Modify Operator. 3 The Radio Set requests Operator name. 4a The Security Officer enters the desired Operator name. 5 The Radio Set presents the defined Operator name and authentication profile in an editable form. 6a The Security Officer modifies the Operator name and authentication profile as desired. 7 The Radio Set presents a list of authorized privileges for the authentication profile. 8 The Security Officer selects the desired privilege(s) for the Operator. The Radio Set builds a Operator identification based on the Operator name, authentication profile, and privilege set then notifies Security Officer via the HMI that a new Operator has been successfully defined. [continue at 1]

Resulting event: Postcondition: The Security Officer has completed the modification of a Operator.

Alternatives: 4b The Security Officer provides an invalid Operator name. 1 The Radio Set rejects the invalid Operator name. [continue at 3] 6b The Security Officer provides an invalid Operator name. 1 The Radio Set rejects the invalid Operator name. [continue at 5] The Security Officer cancels the function requested. This may be done at any point up to entering the Operator privileges.

Nonfunctional requirements:

Comments: The Operator privileges are defined by the authentication profile for Operator.

Figure 55: Modification of a user identification

#### Delete Operator

Use Case Identifier: 100 - Delete Operator

Description: The Security Officer chooses to delete a Operator.

External Actors: Security Officer

Related Use Cases: None

Precondition: The Security Officer must have logged onto the Radio Set and been authenticated by the Radio Set as a valid Security Officer and a valid Operator is already defined. The Radio Set presents a list of authorized functions for the Security Officer type. Triggering event: The Security Officer selects Delete Operator.

Main Flow: 1 2 The Security Officer selects Delete Operator. 3 The Radio Set requests Operator name. 4a The Security Officer enters the desired Operator name. 5 The Radio Set requests confirmation that the Security Officer desires to delete the Operator. 6a The Security Officer confirms request to delete the designated Operator. 7 The Radio Set removes the designated Operator identification from the Radio Set Operator structure. 8 The Radio Set notifies Security Officer of the deletion of the designated Operator. [continue at 1]

Resulting event: Postcondition: The Security Officer has completed the deletion of a Operator.

Alternatives: 4b The Security Officer provides an invalid Operator name. 1 The Radio Set rejects the invalid Operator name. [continue at 3] The Security Officer cancels the function requested. This may be done at any point up to entering the Operator privileges. 6b needs to be written

Nonfunctional requirements:

Comments: The Operator privileges are defined by the authentication profile for Operator.

Figure 56: Deletion of a user identification

#### Lock Operator

Use Case Identifier: 101 - Lock Operator

Description: The Security Officer chooses to lock out a Operator preventing access without deleting.

External Actors: Security Officer

Related Use Cases: None

Precondition: The Security Officer must have logged onto the Radio Set and been authenticated by the Radio Set as a valid Security Officer and a valid Operator is already defined. The Radio Set presents a list of authorized functions for the Security Officer type. Triggering event: The Security Officer selects Lock Operator.

Main Flow: 1 2 The Security Officer selects Lock Operator. 3 The Radio Set requests Operator name. 4a The Security Officer enters the desired Operator name. 5a The Radio Set requests confirmation that the Security Officer desires to lock out the Operator. 6a The Security Officer confirms request to lock out the designated Operator. 7 The Radio Set removes the designated Operator identification from the Radio Set access structure. 8 The Radio Set notifies Security Officer of the lock out of the designated Operator.[continue at 1]

Resulting event: Postcondition: The Security Officer has completed the lock out of a Operator.

Alternatives: 4b The Security Officer provides an invalid Operator name. 1 The Radio Set rejects the invalid Operator name. [continue at 3] 5b Operator is already locked out The Radio Set notifies the Security Officer that the designated Operator is already locked. [continue at 3] 6b needs to be written The Security Officer cancels the function requested. This may be done at any point up to entering the Operator privileges.

Nonfunctional requirements:

Comments: Can this be triggered via multiple password failure attempts.

Figure 57: Lockout an operator

#### Unlock Operator

Use Case Identifier: 102 - Unlock Operator

Description: The Security Officer chooses to restore access to a Operator.

External Actors: Security Officer

Related Use Cases: None

Precondition: The Security Officer must have logged onto the Radio Set and been authenticated by the Radio Set as a valid Security Officer and a valid Operator is already defined. The Radio Set presents a list of authorized functions for the Security Officer type. Triggering event: The Security Officer selects Unlock Operator.

Main Flow: 2 The Security Officer selects Unlock Operator. 3 The Radio Set requests Operator name. 4a The Security Officer enters the desired Operator name. 5 The Radio Set requests confirmation that the Security Officer desires to restore access to the Operator. 6a The Security Officer confirms request to restore access to the designated Operator. 7 The Radio Set adds the designated Operator identification to the Radio Set access structure. 8 The Radio Set notifies Security Officer of the returned access to the designated Operator. [continue at 1]

Resulting event: Postcondition: The Security Officer has completed the Operator access restoration.

Alternatives: 4b The Security Officer provides an invalid Operator name. 1 The Radio Set rejects the invalid Operator name. [continue at 3] 5b Operator is not locked out 1 The Radio Set notifies the Security Officer that the designated Operator is not locked. [continue at 3] The Security Officer cancels the function requested. This may be done at any point up to entering the Operator privileges. 6b needs to be written

Nonfunctional requirements:

Comments:

Figure 58: Unlock an operator

### Manage Security Threats

Figure 59: Manage Security Threats

#### Detect Physical Tamper

Use Case Identifier: 103 - Detect Physical Tamper

Description: An Intruder may attempt to physically tamper with the Radio Set. The Radio Set shall implement physical level tampering detection such that a detection of unauthorized cover or access panel removal will cause a security exception.

External Actors: Intruder

Related Use Cases: Radio Zeroize

Precondition: The Radio Set must be operational. Triggering event:

Main Flow: 1 Intruder attempts to gain access to the inside of the Radio Set without entering a special Maintenance mode. 2 Radio Set detects tamper and executes a security exception.

Resulting event: Postcondition: The Radio Set has executed a security exception.

Alternatives:

Nonfunctional requirements:

Comments:

Figure 60: Detection of unauthorized access removal

### Manage Keys

Figure 61: Manage Key Fill

#### Load Keys

Use Case Identifier: 104 - Load Keys

Description: The Security Officer shall have the capability to load new keys. The Radio Set shall require authentication of key information before key load. The Radio Set shall attempt to authenticate the key fill only once. If the authentication is unsuccessful a security exception shall be generated. Note: Key load includes a GPS fill.

External Actors: Security Officer

Related Use Cases:

Precondition: The Security Officer must been authenticated as having access to the Load Keys function. Triggering event:

Main Flow: 1 The Radio Set presents a list of authorized functions to the Security Officer. 2 The Security Officer selects initiate Load Keys option. 3 The Radio Set instructs the Security Officer to attach the Fill Device to the Radio Set fill port and initiates key load. 4a The Security Officer attaches the Fill Device to the Radio Set fill port and initiates key load. 5a The Radio Set authenticates the key information. 6a The Radio Set loads the key information, logs the load, and sends audit information to fill port. 7 The Radio Set notifies the Security Officer of a successful key load.[end of case]

Resulting event: Postcondition: The keys have been successfully loaded or a security exception has been generated.

Alternatives: 4b The Security Officer does not correctly attach the Fill Device to the Radio Set. 1 The Radio Set will not initiate key load and notifies Security Officer of problem. [continue at 4a] 5b 1 The Radio Set fails to authenticate key fill and generates a security exception. [end] 6b The Radio Set fails to successfully load the authenticated key fill. 1 The Radio Set provides the Security Officer with the options to attempt load again or cancel. 1a The Security Officer selects load again. [continue at 6a] 1b The Security Officer cancels the key load. [end of case] The Security Officer may cancel up to selecting fill or when given the option to attempt load again.

Nonfunctional requirements:

Comments: Load using a fill device is only one type of key load

Figure 62: Successful key load

#### Initiate Software Zeroize of the Radio Set

Use Case Identifier: 105 - Zeroize Radio

Description: Zeroize radio

External Actors: Comm Supporter

Related Use Cases: Zeroize INFOSEC

Precondition: Radio is in Operational mode. Triggering event: An actor requested the radio to be zeroized

Main Flow: 1. An actor requests the radio to be zeroized. 2. The Radio system zeroizes Information Security Devices 3. The Radio System zeroizes Classified-processing devices (normally referred to as red-side processors).

Resulting event: None Postcondition: Radio is in Zeroize State.

Alternatives:

Nonfunctional requirements:

Comments:

Use Case Identifier: 106 - Initiate Software Zeroize of the Radio Set

Description: The Security Officer commands the Radio Set to erase all its fill information.

External Actors: Security Officer

Related Use Cases: Radio Zeroize

Precondition: The Security Officer must be authenticated as having access to initiate software zeroize of the Radio Set. Triggering event:

Main Flow: 1 The Radio Set HMI provides the option to zeroize the Radio Set. 2 The Security Officer selects the Zeroize Radio Set option. 3a The Radio Set zeroizes the Radio Set and logs the zeroize. 4 The Radio Set notifies the Security Officer of a successful zeroize.

Resulting event: Postcondition: The Security Officer has zeroized the Radio Set or the Radio Set has been incapacitated.

Alternatives: 3b The Radio Set fails to successfully zeroize the Radio Set. 1 The Radio Set notifies the Security Officer of the unsuccessful zeroize and provides the Security Officer with the options to attempt zeroize again or cancel.. 2a The Security Officer selects zeroize again. [continue at 3a] 2b The Security Officer incapacitates the radio. [end of case] The Security Officer may cancel up to selecting fill or when given the option to attempt zeroize again

Nonfunctional requirements: A software zeroize of the Radio Set zeroizes all key information within the Radio Set. Note: An unsuccessful zeroize attempt may result in the box being declared SECRET and handled accordingly.

Comments:

Figure 63: Successful zeroize

#### Initiate Software Zeroize of a Specific Waveform

Use Case Identifier: 107 - Initiate Software Zeroize of a Specific Waveform

Description: The Administrator initiates the erasure of waveform specific information fills.

External Actors: Administrator

Related Use Cases:

Precondition: The Administrator must been authenticated as having access to initiate software zeroize of a specific waveform(s). Triggering event:

Main Flow: 1 The Radio Set provides the option to zeroize the specific waveform(s). 2 The Administrator selects the specific waveform to zeroize. 3a The Radio Set zeroizes the specific waveform and logs the zeroize. 4 The Radio Set notifies the Administrator of a successful zeroize.

Resulting event: Postcondition: The Administrator has zeroized the specific waveform.

Alternatives: 3b The Radio Set fails to successfully zeroize the specific waveform. The Radio Set notifies the Administrator of the unsuccessful zeroize. 1 The Radio Set provides the Administrator with the options to attempt zeroize again or cancel. 1a The Administrator selects zeroize again. [continue at 3a] 1b The Administrator selects cancel. [end] The Administrator may cancel up to selecting fill or when given the option to attempt zeroize again

Nonfunctional requirements: A software zeroize of a specific waveform erases all key information relative to that waveform(s).

Comments:

Figure 64: Successful zeroize

#### Initiate Information Fill

Use Case Identifier: 108 - Initiate Information Fill

Description: The Security Officer initiates an information fill.

External Actors: Security Officer

Related Use Cases:

Precondition: The Security Officer must been authenticated with access to initiate information fill. Triggering event:

Main Flow:

Resulting event: Postcondition: The Radio Set has completed an information fill.

Alternatives:

Nonfunctional requirements:

Comments:

#### Over the Air Rekey

Use Case Identifier: 109 - Initiate OTAR

Description: The Security Officer shall have the capability to initiate Over the Air Rekey of Radio Set keys. The transferred keys are stored in the Cryptographic Manager.

External Actors: Security Officer

Related Use Cases:

Precondition: An encrypted control channel must have already been established. Triggering event:

Main Flow:

Resulting event: Postcondition: The OTAR has been initiated.

Alternatives:

Nonfunctional requirements:

Comments:

Use Case Identifier: 110 - Receive Over the Air Rekey

Description: The Radio Set receives and authenticates Over the Air Rekey (OTAR) of specific waveform(s) or Radio Set keys. The transferred keys are stored inside the Cryptographic tamper-proof secure area.

External Actors: Security Officer

Related Use Cases:

Precondition: An encrypted control channel must have already been established. Triggering event: Rekeying request.

Main Flow:

Resulting event: Security audit event Postcondition: The specific waveform(s) or Radio Set keys have authenticated and been stored in the Cryptographic Manager.

Alternatives:

Nonfunctional requirements:

Comments:

Figure 65: Successful OTAR

#### Over the Air Zeroize

Use Case Identifier: 111 - Initiate OTAZ

Description: The Security Officer initiates an Over the Air Zeroize of keys.

External Actors: Security Officer

Related Use Cases:

Precondition: An encrypted control channel must have already been established. Triggering event:

Main Flow:

Resulting event: Postcondition: The OTAZ has been initiated.

Alternatives:

Nonfunctional requirements: A software zeroize affects all key information relative to a waveform or the entire Radio Set.

Comments:

Use Case Identifier: 112 - Receive Over the Air Zeroize

Description: The Radio Set receives an over the air Zeroize (OTAZ) command from an External Radio Set to zeroize a specific waveform(s) of the Radio Set. Due to the nature of the OTAZ command, the command consists of two parts to increase the reliability of accurate execution.

External Actors: Security Officer or Operator

Related Use Cases:

Precondition: An encrypted control channel must have already been established. Triggering event:

Main Flow: 1 The Radio Set receives Part I of an OTAZ command. 2a The Radio Set authenticates the Security Officer or Operator from the External Radio Set. 3a The Radio Set authenticates the command. 4 The Radio Set acknowledges the authentication of Part I. (optional depending on command) 5a The Radio Set receives Part II of an OTAZ command. 6a The Radio Set authenticates the sender of the command. 7a The Radio Set authenticates the command. 8 The Radio Set acknowledges the authentication of Part II. (optional depending on command) 9 The Radio Set executes the OTAZ. 10 The Radio Set informs the Operator of the successful execution of the OTAZ.

Resulting event: Postcondition: The specific waveform(s) of the Radio Set have been zeroized.

Alternatives: 2b The Radio Set can not authenticate the sender of the command. 1 The Radio Set blocks any further action on this particular reception and logs the attempt with the Part I command. 2 Part I is NAKed as required. 3 Information received with Part II is stored with the attempt log for this OTAZ. 4 Part II is NAKed as required. 3b The Radio Set can not authenticate the command. 1 The Radio Set blocks any further action on this particular reception and logs the attempt with the Part I command 2 Information received with Part II is stored with the attempt log. 3 Part II is NAKed as required. 6b The Radio Set can not authenticate the sender of the command. 1 The Radio Set blocks any further action on this particular reception and logs the attempt. 2 Part II is NAKed as required. 7b The Radio Set can not authenticate the command. 1 The Radio Set blocks any further action on this particular reception and logs the attempt. 2 Information received with Part II is stored with the attempt log. 3 Part II is NAKed as required.

Nonfunctional requirements: The Transmit/Receive use case is used to receive the OTAZ Command. The Radio Set intercepts and interprets the OTAZ command instead of passing it on to the Subscriber. All fill locations in the Radio Set are zeroized upon completion of a Radio Set directed OTAZ All fill locations associated with a waveform are zeroized upon completion of a waveform directed OTAZ

Comments:

Figure 66: Successful OTAZ

#### Over the Air Transfer of Keys

Use Case Identifier: 113 - Initiate OTAT

Description: The Radio Set initiates Over-The-Air-Transfer (OTAT) of Radio Set keys.

External Actors:

Related Use Cases: Transmit Data Stream (non-packet)

Precondition: An encrypted control channel must have already been established. Triggering event:

Main Flow:

Resulting event: Postcondition: The OTAT has been initiated.

Alternatives:

Nonfunctional requirements: An OTAT is transparent to the Operator with the transferred keys being stored in the Key Manager.

Comments:

Use Case Identifier: 114 - Receive Over the Air Transfer of Keys

Description: The Radio Set receives and authenticates Over the Air Transfer (OTAT) of Radio Set keys from an External Radio Set

External Actors: Security Officer

Related Use Cases: Receive Stream Data (non-packet)

Precondition: An encrypted control channel must have already been established. Triggering event:

Main Flow:

Resulting event: Postcondition: The Radio Set keys have authenticated and been stored in the Key Manager.

Alternatives:

Nonfunctional requirements: The OTAT uses the Transmit/Receive use case to receive the OTAT. The OTAT is intercepted by the Radio and is not passed on to the Subscriber. An OTAT is transparent to the Operator with the transferred keys being stored in the Key Manager.

Comments:

Figure 67: Successful OTAT of keys

#### Initiate Algorithm Load

Use Case Identifier: 115 - Initiate Algorithm Load

Description: The Radio Set receives a Cryptographic Algorithm file download. The Security Officer executes the load via a fill device attached at the key fill port.

External Actors: Security Officer

Related Use Cases:

Precondition: Algorithm is encrypted and pre-loaded in a tamper-protected fill device. The Security Officer is authorized to perform the load. Radio Set is powered up and operating. Radio Set key fill port is available to accept algorithm download from fill device per the Fill Rule(s) below. Triggering event:

Main Flow: 1 The Security Officer attaches the Fill Device to the Radio Set Key Fill Port. 2 The Radio Set recognizes the presence of the Fill Device and configures itself to accept an algorithm load. 3 The Radio Set indicates to the Security Officer (via the Fill Device) that it is ready to receive the download. 4 The Security Officer engages the download sequence. 5a The Radio Set indicates to the Security Officer (via the Fill Device) that the algorithm download was successful. 6a The Security Officer detaches the Fill Device from the Key Fill Port. 7 The Radio Set recognizes the removal of the Fill device and returns to prior state.

Resulting event: Postcondition: The algorithm download was successfully completed and the Security Officer was notified to that effect.

Alternatives: 6b Load multiple crypto devices in same session (repeat sequence under control of the Fill Device). 5b Radio Set indicates to the Security Officer that download was unsuccessful 1 Security Officer conducts diagnostics to determine problem.

Nonfunctional requirements: Fill Rules : Prior to final storage of the algorithm, the Radio Set must determine that: 1) the load is authorized 2) the load passes an integrity test (is not corrupted) Privilege Rules : Person performing the load must be a COMSEC custodian

Comments: Other devices beside fill devices could be used.

Figure 68: Initiate algorithm load

### Security Policy

TBD

### Access Control

TBD

### Authentication/Integrity/Confidentiality

TBD

### Security Audit

TBD

## Fault Management

Figure 69: Fault Management

### Gather POST Results

Use Case Identifier: 116 - Gather POST Results

Description: The Radio System gathers the POST results and provides them to the Operator on power-up.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event:

Main Flow: 1. The Radio System gets a list of the current HW devices in the system. 2. The Radio System verifies that each HW device has registered into the system and then obtains the POST results from each HW device. 3. The Radio System provides the POST results to the Operator.

Resulting event: None Postcondition: POST results are collected and provided to the Operator.

Alternatives: None

Nonfunctional requirements: None

Comments: None

### Get Radio Health

Use Case Identifier: 117 - Get Radio Health

Description: An Operator requests the current health of the radio.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system Triggering event:

Main Flow: 1. The Operator requests the current health of the radio. 2. The Radio System provides a summary status of the current health of the radio.

Resulting event: None Postcondition: Current health of the radio is provided to the Operator.

Alternatives: None

Nonfunctional requirements: None

Comments: None

### Start Continuous BIT

Use Case Identifier: 118 - Start Continuous BIT

Description: The Radio System begins to perform continuous (background) BIT.

External Actors: None

Related Use Cases: None

Precondition: POST completed successfully Triggering event: POST is complete

Main Flow: 1. The Radio System commands each HW device to run continuous BIT

Resulting event: None Postcondition: Each HW device has been commanded to run continuous BIT

Alternatives: None

Nonfunctional requirements: None

Comments: None

### Monitor Continuous BIT

Use Case Identifier: 119 - Monitor Continuous BIT

Description: The Radio System monitors for Continuous BIT failures.

External Actors: None

Related Use Cases: None

Precondition: None Triggering event: Continuous BIT successfully started

Main Flow: 1. The Radio System subscribes to and is listening to an event channel for BIT alarms. 2. At a periodic rate, the Radio System commands each HW device to start continuous BIT. This is similar to “pinging” the HW device component to ensure that it is still alive. 3. If a HW device has a continuous BIT failure, it submits a BIT alarm to the event channel. 4. The Radio System detects a failure if a BIT alarm is detected or a HW device fails its “ping” test.

Resulting event: None Postcondition: The Radio System has detected a HW failure due to a BIT alarm or a failure to communicate with a HW device.

Alternatives: None

Nonfunctional requirements: None

Comments: None

### Report Continuous BIT Fault

Use Case Identifier: 120 - Report Continuous BIT Fault

Description: The Operator is notified of a BIT fault.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event:

Main Flow: 1. The Radio System has detected a Continuous BIT fault. 2. The Operator is presented with a BIT fault indication.

Resulting event: None Postcondition: The Operator is notified of a BIT fault indication.

Alternatives: None

Nonfunctional requirements: None

Comments: None

### Stop Continuous BIT

Use Case Identifier: 121 - Stop Continuous BIT

Description: The Operator has decided to stop continuous BIT through-out the radio.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event:

Main Flow: 1. The Operator selects to stop continuous BIT for the entire radio. 2. The Radio System accepts the request and stops continuous BIT on each HW device in the radio.

Resulting event: None Postcondition: Continuous BIT has been stopped throughout the radio.

Alternatives: None

Nonfunctional requirements: None

Comments: None

### Stop Specific Continuous BIT

Use Case Identifier: 122 - Stop Specific Continuous BIT

Description: The Operator needs to run a more comprehensive BIT for a specific HW device so the continuous BIT for that device is disabled

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event:

Main Flow: 1. The Operator selects the HW device to stop the continuous BIT on. 2. The Radio System commands the HW device to stop running continuous BIT. 3. The Radio System reports to the Operator that the HW device is no longer running continuous BIT

Resulting event: None Postcondition: The Radio System has stopped running continuous BIT on the selected HW device.

Alternatives: None

Nonfunctional requirements: None

Comments: None

### Restart Specific Continuous BIT

Use Case Identifier: 123 - Restart Specific Continuous BIT

Description: The Operator needs to restart continuous BIT for a device that had been previously not running continuous BIT.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event:

Main Flow: 1. The Operator selects the HW device to restart the continuous BIT on. 2. The Radio System commands the HW device to start running continuous BIT. 3. The Radio System reports to the Operator that the HW device is running continuous BIT

Resulting event: None Postcondition: The Radio System has restarted running continuous BIT on the selected HW device.

Alternatives: None

Nonfunctional requirements: None

Comments: None

### Run Specific Commanded BIT

Use Case Identifier: 124 - Run Specific Commanded BIT

Description: The Operator selects a specific commanded BIT to run on a HW device.

External Actors: Operator

Related Use Cases: Stop Specific Continuous BIT

Precondition: The Operator is logged into the system. Triggering event:

Main Flow: 1. The Operator selects from a list of possible commanded BIT tests for a specific HW device. 2. The Radio System commands the HW device to run the specified commanded BIT. 3. The Radio System provides the test results to the Operator.

Resulting event: None Postcondition: Commanded BIT test results are provided to the Operator.

Alternatives: None

Nonfunctional requirements: None

Comments: None

## Support

Figure 70: Support

# Waveform Use Cases

## FM3TR Waveform

TBD

## WCDMA Waveform

TBD

## HF 4285 Military Waveform

HF 4285 illustrate a case of waveform processing data.

### HF 4285 : Reception

Use Case Identifier: 125 - HF 4285 Reception

Description: Reception

External Actors: Air Interface, Operator, Subscriber

Related Use Cases: None

Precondition: Radio Set is initialized. Waveform is initialized and active. Triggering event: None

Main Flow: 1. Operator open a session with Radio Set. 2. Operator create a channel, 3. Operator configures the channel, 4. Subscriber receive data from channel.

Resulting event: None Postcondition: Channel created and active. Data received and available for use.

Alternatives: None

Nonfunctional requirements: None

Comments: None

Figure 71: HF 4285 Receive data

### HF 4285 : Transmission

Use Case Identifier: 126 - HF 4285 Transmission

Description: Transmission

External Actors: Air Interface, Operator, Subscriber

Related Use Cases: None

Precondition: Radio Set is initialized. Waveform is initialized and active. Triggering event: None

Main Flow: 1. Operator open a session with Radio Set. 2. Operator create a channel, 3. Operator configures the channel, 4. Subscriber send data to channel.

Resulting event: None Postcondition: Channel created and active. Data sent.

Alternatives: None

Nonfunctional requirements: None

Comments: None

Figure 72: HF 4285 Transmit data

# Open questions

| N° | Ref. | Question | Answer |
| --- | --- | --- | --- |
| Gen | Actors | All Use Cases should use the Actors set defined in the initial submission. If Actors are missing or seem unclear : raise it ! |  |
| Gen | Radio Set / Radio System | Check to be conformant with definitions given in initial submission. Scope of Use Case (Radio Set or Radio System) is important. |  |
| 1 | Zeroize Radio | replace “radio is” by “Radio Set is” |  |
| 2 | Performance Management Manage Physical Configuration Fault Management RADIO MGMT | replace “radio system” by “Radio Set” |  |
| 3 | RADIO MGMT | “Comm Supporter”, “ Comm User”, “ Radio Manager” are undefined actors. Should be added ? |  |
| 4 | RADIO MGMT | State diagrams should not be incorporated in use cases templates but expressed outside as an introduction that help to better understand pre and post conditions. |  |
| 5 | RADIO MGMT | Design consideration should not be taken into account within Use Cases. (name of attributes, etc.) |  |
| 6 | Performance Management | Do these Use Cases are good level (too much generic) ? |  |
| 7 | Log Historical Data | Actors are “Administrator, Maintainer or Operator”. Which actor shall be represented on use case diagram and sequence diagram ? same question for use cases: - Register to Receive Radio Events, - Startup |  |
| 8 | List Deployment Plans | Actor “Supporter” is used. |  |
| 9 | General | Do not use Microsoft styles in use cases templates |  |
| 10 | Remove Application | Merge with “Uninstall Application” |  |
| 11 | List Applications | Merge with “List Installed Applications” |  |
| 12 | zeroize radio | Merge with “Initiate Software Zeroize of the Radio Set” |  |
| 13 | Power up | Merge with “Power-up and Initialize” |  |
| 14 | Shutdown | Merge with “Commanded Shutdown” |  |
| 15 | Power Up | Missing use cases: Instantiate Communication Channel Enable Application Enable Communication Channel Instantiate Application Parse Deployment Plan Power On Self Test |  |
| 16 | Zeroize Radio | Missing use case: Zeroize INFOSEC |  |
| 17 | Over the Air Rekey | 2 scenarios for 1 use case |  |
| 18 | Over the Air Zeroize | 2 scenarios for 1 use case |  |
| 19 | Over the Air Transfer of Keys | 2 scenarios for 1 use case |  |
| 20 | Control Communication Channel | States diagrams are not in the Rose model |  |
| 21 | Diagram Actors | “External Radio Set” is not on the diagram |  |
| 22 | Radio Set Use Cases | “Baseband System” is not on the diagram “Actors” |  |

Empty template for use case:

Use Case Identifier: 127 -

Description:

External Actors:

Related Use Cases:

Precondition: Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: