| # Use Case Identifier: Power Up | Traceability |
| --- | --- |
| Description: Describes the start-up of a radio |  |
| External Actors: Supporter |  |
| Related Use Cases: Instantiate Communication Channel, Enable Application, Enable Communication Channel, Instantiate Application, Parse Deployment Plan, Power On Self Test |  |
| Precondition: Radio is off Triggering event: A supporter applies power to the radio. |  |
| - Supporter applies power to the radio - Power On Self Test (POST) is performed - SWRADIO infrastructure (framework) is deployed. - If waveform deployment plan exists, it is parsed. - Logical Communication Channels are instantiated. - Logical Communication Channels are enabled. - Applications are instantiated as per the deployment plan. - Applications are enabled as per the deployment plan. |  |
| Resulting event: None Postcondition: Radio is running, waveform are deploy as per the power-up deployment plan |  |
| Alternatives: Radio could return to the last known running state. This would require the deployment plan to be updated each time a new waveform/service/application was deployed. |  |
| Nonfunctional requirements: |  |
| Comments: |  |