1	Radio Management Use Cases	3

1.1	Deployment Plan Use Cases	3

1.1.1	List Deployment Plans	3

1.1.2	Enable Deployment Plan	5

1.1.3	Disable Deployment Plan	6

1.1.4	Add Deployment Plan	7

1.1.5	Remove Deployment Plan	7

1.2	Radio Use Cases	9

1.2.1	PowerUp	9

1.2.2	Shutdown	14

1.2.3	Get Radio Configuration	15

1.2.4	Set Radio Configuration	16

1.2.5	Change Radio Transmission Mode	17

1.2.6	Change Radio Administrative State	18

1.2.7	Zeroize Radio Set	20

1.3	Comm Channel Use Cases	21

1.3.1	Change Comm Channel Transmission Mode	21

1.3.2	Change Comm Channel Administrative State	22

1.3.3	Configure Comm Channel	24

1.3.4	Change Comm Channel Security Mode	25

1.3.5	List Comm Channels	26

1.3.6	Instantiate Comm Channel	27

1.3.7	Uninstantiate Comm Channel	29

1.3.8	Perform Comm Channel BIT	31

1.4	Application Use Cases	32

1.4.1	List Applications	32

1.4.2	Remove Application	33

1.4.3	Add Application	33

1.4.4	Change Application Operational Mode	33

1.4.5	Instantiate Application	33

1.4.6	Uninstantiate Application	33

1.5	Hardware Use Cases	34

1.5.1	List Hardware	34

1.5.2	Change Comm Hardware Administrative State	35

1.5.3	Add Hardware	37

1.5.4	Set Hardware Configuration	39

1.5.5	Get Hardware Configuration	40

1.5.6	Remove Hardware	41

1.5.7	Perform Hardware BIT	42

1.6	Alarms Use Cases	42

1.6.1	Change Alarm Operational Mode	42

1.6.2	List Alarms	44

1.6.3	Modify Alarm	45

1.6.4	Trigger Alarm	46

1.6.5	Remove Alarm Configuration	46

1.6.6	Create Alarm Configuration	46

1.6.7	Enable Alarm Configuration	46

1.6.8	Events	47

1.6.9	Notify Radio Event	47

1.6.10	Register to Receive Radio Events	48

1.6.11	Unregister to Receive Radio Events	49

# Radio Management Use Cases

The use cases in this document map to these requirements in the RFP.

Mandatory Related RFP Requirements

6.5.2	Platform Independent Model

- Proposals shall define interfaces between waveform components and communication channels, radio devices and radio services.

- Proposals shall define SDR device interfaces and radio services  in the PIM. The device interfaces shall include interfaces for, as a minimum, audio and modem devices.

g)	Proposals shall define communication channel creation and management interfaces.

Optional RFP Requirements

b)	The PIM may provide interfaces for management of radio communications.

c)	The PIM may provide interfaces for radio services.

Note: There is still some use cases not completed yet for Application and Alarms.

## Deployment Plan Use Cases

### List Deployment Plans

Use Case Identifier: List Deployment Plans

Description: This use case addresses the ability of an actor to retrieve Deployment Plans from a radio set.

External Actors: Supporter

Related Use Cases:

Precondition: Radio Set is Operational Triggering event:

Main Flow: - An actor requests deployment plans information from the Radio Set. - The radio set returns a list of deployment plans, along with their characteristics. GD – Would be helpful to list some of the characteristics such as: 1) linkage of waveforms to resources such as channel assignment, operating frequency, or other specifics from the SCA property set/Domain Profile (or from the deployment working group). This would provide a better basis for expanding to the classes and methods required to define the Radio Services.

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System

### Enable Deployment Plan

Use Case Identifier: Enable Deployment Plan

Description: This use case addresses the ability of an actor to enable a Deployment Plan within a radio set.

External Actors: Supporter

Related Use Cases: List Deployment Plans

Precondition: Radio Set is Operational Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System GD – Assume the intent is at a high level to specify the waveform suite to be brought up in the collective set of channels for the radio (or radios). This might involve an Operator picking a deployment plan that would affect load of waveforms in multiple radios. Also need to consider potential for set of cold start defaults for a radio independent of an operator action (box could boot up with set of waveforms independent of an HMI). Another factor is return to some last known (or saved) state (e.g., waveforms, frequency settings, RF Power level, etc.).

### Disable Deployment Plan

Use Case Identifier: Disable Deployment Plan

Description: This use case addresses the ability of an actor to disable a Deployment Plan within a radio set.

External Actors: Supporter

Related Use Cases: List Deployment Plans

Precondition: Radio Set is Operational Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System.

### Add Deployment Plan

Use Case Identifier: Add Deployment Plan

Description: This use case addresses the ability of an actor to add a Deployment Plan from a radio set.

External Actors: Supporter

Related Use Cases: Notify Event

Precondition: Radio Set is Operational Triggering event:

Main Flow: 1.

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System

### Remove Deployment Plan

Use Case Identifier: Remove Deployment Plan

Description: This use case addresses the ability of an actor to remove a Deployment Plan within a radio set.

External Actors: Supporter

Related Use Cases: Notify Event

Precondition: Radio Set is Operational Triggering event:

Main Flow:

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: This use case would also be applicable for Radio System

## Radio Use Cases

### PowerUp (Copyright 2003 -- General Dynamics Decision Systems)

Use Case Identifier: PowerUp

Description: This use case addresses PowerUp of the radio set.

External Actors: Operator, Node Boot Up, Internal Actors: Domain Manager, CORBA Naming Services, Application Factory, Resource (primary Radio Control Resources)

Related Use Cases: Enable Deployment Plan. Events, Get Radio Configuration

Precondition: The required software has been installed within the radio set. The power is off. The ITU is connected and external equipment is connected and powered up .The Operator logs in to any required external equipment, if required. Triggering event: The Operator switches Power-ON or presses the reset button

Main Flow: - Node Boot Up signals power has been applied or reset. - The Domain Manager begins executing and binds with CORBA Naming Services. perform deviceManagerStartup (ref fig. 3-10 in SCA 2.2) to initiate the Device Manager and launch, register, initialize and configure Devices - The Domain Manager initializes Resource - The Domain Manager starts Resource - Resource registers primary Radio Control Resources with Domain Manager (repeat steps 3-5 for each Resource) - The Domain Manager performs Built In Test by initiating runTest on each Testable Resource. - Domain Manager performs create an application sequence (ref fig. 3-14 in SCA 2.2) to create Application Factory and to execute, initialize and configure all startup Applications as defined by a startup profile. - Use Case ends.

Resulting event: The ITU is ready to install new applications or load previously installed applications. Postcondition: Commands can be sent to the ITU from the Operator.

Alternatives:

Nonfunctional requirements:

Comments:

Power Up – Copyright 2003 General Dynamics Decision Systems

Device Manager Startup from SCA 2.2

Create an Application from SCA 2.2

### Shutdown

### Get Radio Configuration

Use Case Identifier: Get Radio Configuration

Description: This use case addresses the ability of an actor to retrieve radio configuration information and radio’s attributes.

External Actors: Comm Supporter

Related Use Cases: List Deployment Plans, List Instantiated Applications, List Applications, List Communication Channels, List Hardware

Precondition: Radio is Operational Triggering event:

Main Flow: - An actor requests Radio information. - The radio returns configuration information based upon the request. Types of requests can be: - Radio Identification - Radio Time - Location - Instantiated Waveforms - Communication Channels - Operational State - Usage State - Adminstrative State - Installed Waveforms - Log - Devices - Deployment Plans

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: GD – Should cover scenarios such as an HMI/Operator connecting to an operational radio and then being sent a status information block for all waveforms instantiated. Also should address possibility of multiple HMI/Operators connected to a radio – 1) Operator A changes out a waveform, 2) Operator B should be notified (e.g., via an event?). Should be a general concept of an HMI registering to receive events for a radio (e.g., receive notifications of waveform state changes, failures, etc.). Seems there should also be authentication of an operator prior to allowing access to control the radio or retrieve status information.

### Set Radio Configuration

Use Case Identifier: Set Radio Configuration

Description: This use case addresses the ability of an actor to set radio configuration information of the Radio Set.

External Actors: Supporter

Related Use Cases:

Precondition: Radio set is Operational Triggering event:

Main Flow: - An actor requests Radio Set information. - The radio sets configuration information based upon the request. Types of requests can be: - Radio Identification - Radio Time – - Location - Generate event indicacting that radio information has changed.

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

### Change Radio Transmission Mode

Use Case Identifier: Change Radio Transmission Mode

Description: Describes the use case steps for changing the radio set’s transmission mode.

External Actors: Supporter

Related Use Cases: Inhibit Transmission Communication Channel

Precondition: Radio is Operational. Triggering event: A comm. supporter requested the radio set’s transmissions to be changed.

Main Flow: - An actor requests transmission mode change for the radio. - The Radio Set delegates change transmission mode commands to each communication channel so as to perform waveform specific actions associated with transmission mode request (basically makes use of the Change Comm Channel Transmission Mode use case)

Resulting event: None Postcondition: Radio Set is in an Operational Silent State or Operational Transmission State.

Alternatives: 2a) The Radio Set performs no operations since the radio is in the correct transmission mode of operation.

Nonfunctional requirements:

Comments:

### Change Radio Administrative State

Use Case Identifier: Change Radio Administrative State

Description: This use case addresses the ability of an actor to change a radio administrative state. The administrative state indicates the permission to use or prohibition against using the radio’s communication channels.

External Actors: Comm Supporter Internal Actors: Communication Channel

Related Use Cases: Change Communication Channel Administrative State, Notify Event

Precondition: Radio is either in an administrative state of UNLOCKED, Transitioning to LOCKED, or LOCKED. Triggering event:

Main Flow: - An actor requests a change in the administrative mode of operation for a Radio. - The Radio transitions to either UNLOCKED, Transitioning to SHUTTING_DOWN or LOCKED depending of the state it is in as shown in the following Figure. Figure. State Transition Diagram for Radio’s Administrative State 3. The Radio sends the corresponding administrative state requests to each of its Communication Channels. 4. Upon successful state change of the communication channels, the Radio sends a state change event indicating the administrative state the Radio is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Radio. - The sourceId field is the identifier attribute of the Radio. - The stateChangeCategory field is ADMINISTRATIVE_STATE_EVENT. - The stateChangeFrom and stateChangeTo fields shall reflect the administrative state before and after the state change, respectively.

Resulting event: Postcondition: Radio Set is in either UNLOCKED, SHUTTING_DOWN, or LOCKED.

Alternatives: 3a) No request is sent when there is no change in administrative state. 4a) No event is sent when there is no change in administrative state.

Nonfunctional requirements:

Comments: When the Radio’s administrative state transitions to LOCKED when all its communication channels are LOCKED. When the Radio’s administrative state transitions to SHUTTING_DOWN when all its communication channels are not LOCKED. The Administrative States are based upon CCITT (the International Telegraph and Telephone Consultative Committee), which is a permanent organ of ITT (International Telephone Union) Recommendation X.731 that was approved on Jan 17th 1992. This is also published as ISO/IEC International Standard 10164-2.

### Zeroize Radio Set

Use Case Identifier: Zeroize Radio Set

Description: Zeroize radio

External Actors: Supporter

Related Use Cases: Zeroize INFOSEC

Precondition: Radio Set is in Operational mode. Triggering event: An actor requested the radio to be zeroized

Main Flow: - An actor requests the radio to be zeroeized. - The Radio set zeroizes Information Security Devices - The Radio Set zeroizes Classified-processing devices (normally referred to as red-side processors).

Resulting event: None Postcondition: Radio is in Zeroize State.

Alternatives:

Nonfunctional requirements:

Comments:

## Comm Channel Use Cases

### Change Comm Channel Transmission Mode

Use Case Identifier: Change Comm Channel Transmission Mode

Description: This use case addresses the ability of an actor to change a communication channel transmission or emissions control (EMCON) mode.

External Actors: Comm Supporter, Radio Manager

Related Use Cases:

Precondition: Communication Channel can be in any state of operation. Triggering event:

Main Flow: - An actor selects transmission mode for a communication channel - Communication Channel performs application specific actions related to transmission mode request for the channel. - Communication Channel sends transmission mode request to its RF Channels. - RF Channel configures its communication equipment (power amplifier, etc.) based upon the transmission mode request. - Communication Channel reports completion of its request to the actor.

Resulting event: Postcondition: Communication Channel and its RF Channels and comm. Equipments are in inhibit or enable transmission mode of operation.

Alternatives: 2a) Comm Channel is already in the correct transmission mode, no action is performed. 2b) Comm Channel is in an improper state (Half-Duplex Receive Only) that permits transmission mode settings. No action is performed. Error returned? 2c) The Comm Channel’s Usage State is IDLE, therefore no waveform specific action is taken. 3a) Failure to configure comm. equipment

Nonfunctional requirements:

Comments: Transmission mode is either enable transmission or disable transmission. When enabled transmission of data out to the antenna is permitted.

### Change Comm Channel Administrative State

Use Case Identifier: Change Comm Channel Administrative State

Description: This use case addresses the ability of an actor to change a communication channel adminstrative state. The administrative state indicates the permission to use or prohibition against using the communication channel.

External Actors: Comm Supporter

Related Use Cases:

Precondition: Communication Channel is either in an administrative state of UNLOCKED, Transitioning to LOCKED, or LOCKED. Triggering event:

Main Flow: - An actor requests a change in the administrative mode of operation for a communication channel. - The Communication Channel transitions to either UNLOCKED, Transitioning to SHUTTING_DOWN or LOCKED depending of the state it is in as shown in the following Figure. Figure. State Transition Diagram for Communication Channel’s Administrative State 3. The Communication Channel sends a state change event indicating the administrative state the Communication Channel is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Communication Channel. - The sourceId field is the identifier attribute of the Communication Channel. - The stateChangeCategory field is ADMINISTRATIVE_STATE_EVENT. - The stateChangeFrom and stateChangeTo fields shall reflect the administrative state before and after the state change, respectively.

Resulting event: Postcondition: Communication Channel is in either UNLOCKED, SHUTTING_DOWN, or LOCKED.

Alternatives: 3a) No event is sent when there is no change in administrative state.

Nonfunctional requirements:

Comments: When the Communication Channel’s administrative state is LOCKED, it cannot be instantiated for waveform usage. The Administrative States are based upon CCITT (the International Telegraph and Telephone Consultative Committee), which is a permanent organ of ITT (International Telephone Union) Recommendation X.731 that was approved on Jan 17th 1992. This is also published as ISO/IEC International Standard 10164-2.

### Configure Comm Channel

Use Case Identifier: Configure Comm Channel

Description: This use case addresses the ability of an actor to configure an instantiated communication channel.

External Actors: Comm User

Related Use Cases: Configure Waveform Parameters

Precondition: Communication Channel’s Usage State is BUSY. Triggering event:

Main Flow: - An actor requests configuration changes to a communication channel. - Communication Channel performs waveform application specific actions related to requested configuration changes.

Resulting event: Postcondition: Communication Channel’s configuration has been modified.

Alternatives: 2a) No configuration is performed, the configuration parameters are invalid for the waveform application; an exception is raised indicating this. 2b) Partial configuration is performed, Some configuration parameters are invalid for the waveform application, and an exception is raised indicating this.

Nonfunctional requirements:

Comments:

### Change Comm Channel Security Mode

Use Case Identifier: Change Comm Channel Security Mode

Description: This use case addresses the ability of an actor to change a communication channel security mode.

External Actors: Comm User

Related Use Cases:

Precondition: Communication Channel’s Usage State is BUSY. Triggering event:

Main Flow: - An actor selects security mode for communication channel. - Communication Channel performs waveform application specific actions related to security mode request for the channel. - Communication Channel requests security mode change for the associated Security Channel. - Communication Channel reports completion of its request to the actor.

Resulting event: Postcondition: Communication Channel’s security mode changes.

Alternatives: 2a) Comm Channel is already in the correct security mode, no action is performed. 2b) Security mode changes for waveform application is not allowed, no action is performed

Nonfunctional requirements:

Comments:

### List Comm Channels

Use Case Identifier: List Comm Channels

Description: This use case addresses the ability of an actor to retrieve communication channels from the Radio Set.

External Actors: Supporter

Related Use Cases:

Precondition: Radio Set is Operational Triggering event:

Main Flow: - An actor requests communication channels information from the Radio. - The radio returns a list of communication channels, along with their characteristics.

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments: GD – Need to resolve how this Use Case relates to the “Get Radio Configuration” use case.

### Instantiate Comm Channel

Use Case Identifier: Instantiate Comm Channel

Description: This use case addresses the ability of an actor to instantiate a communication channel with a specific waveform.

External Actors: Comm Support, Radio Manager, Comm User

Related Use Cases: Instantiate Application

Precondition: Communication Channel’s administrative state of UNLOCKED, Operational State is ENABLE, and Usage State is IDLE. Triggering event:

Main Flow: - An actor requests a communication channel to be instantiated with a specific waveform. - The communication channel requests waveform’s ApplicationFactory to instantiate the waveform using the Communication Channel’s devices and any initial configuration parameters. - The waveform’s ApplicationFactory uses input devices to allocate devices, load waveform’s components onto devices, and execute waveform’s components onto devices (basically makes use of the InstantiateApplication use case) GD – allocation of devices, loading of waveform components and initiating dataflow seem to be additional states of the waveform that should also be available to an operator. - The Communication Channel’s Usage State is set to Busy. - The Communication Channel sends a state change event indicating the usage state the Communication Channel is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Communication Channel. - The sourceId field is the identifier attribute of the Communication Channel. - The stateChangeCategory field is USAGE_STATE_EVENT. - The stateChangeFrom and stateChangeTo fields shall reflect the usage state before and after the state change, respectively. - The mode of operations: - Waveform mode - receive only, transmit only, full duplex, - Data Encryption Security Mode - Secure and non-secure, - Classification level – unclassified, Top Secret, etc.

Resulting event: Postcondition: Communication Channel’s Usage State is Busy.

Alternatives: 2a) The request is denied due to the communication channel not in the correct pre-condition states 3a) The waveform’s ApplicationFactory is unable to deploy due to various reasons.

Nonfunctional requirements: Need to be able to execute in an asynchronous manner.

Comments: The Administrative, Operational, and Usage States are based upon CCITT (the International Telegraph and Telephone Consultative Committee), which is a permanent organ of ITT (International Telephone Union) Recommendation X.731 that was approved on Jan 17th 1992. This is also published as ISO/IEC International Standard 10164-2.

### Uninstantiate Comm Channel

Use Case Identifier: Uninstantiate Comm Channel

Description: This use case addresses the ability of an actor to uninstantiate a communication channel.

External Actors: Supporter, Comm. User

Related Use Cases: Uninstantiate Application

Precondition: Communication Channel’s administrative state of UNLOCKED or SHUTTING_DOWN, Operational State is ENABLE, and Usage State is BUSY. Triggering event:

Main Flow: - An actor requests a communication channel to be uninstantiated. - The communication channel requests waveform’s Application component to teardown. - The waveform’s Application component unallocates devices’ capacities, unloads waveform’s components from devices, and terminates waveform’s components executions from devices (basically makes use of the UninstantiateApplication use case) - The Communication Channel’s Usage State is set to IDLE. - The Communication Channel sends a state change event indicating the usage state the Communication Channel is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Communication Channel. - The sourceId field is the identifier attribute of the Communication Channel. - The stateChangeCategory field is USAGE_STATE_EVENT. - The stateChangeFrom and stateChangeTo fields shall reflect the usage state before and after the state change, respectively.

Resulting event: Postcondition: Communication Channel’s Usage State is IDLE. Optional the Communication Channel’s Administrative can be in the LOCKED state.

Alternatives: 2a) The request is denied due to the communication channel not in the correct pre-condition states 6) The Communication Channel’s Administrative State is set to LOCKED. The Communication Channel sends a state change event indicating the administrative state the Communication Channel is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Communication Channel. - The sourceId field is the identifier attribute of the Communication Channel. - The stateChangeCategory field is ADMINISTRATIVE_STATE_EVENT. The stateChangeFrom and stateChangeTo fields shall reflect the administrative state before and after the state change, respectively.

Nonfunctional requirements: Need to be able to execute in an asynchronous manner.

Comments: The Administrative, Operational, and Usage States are based upon CCITT (the International Telegraph and Telephone Consultative Committee), which is a permanent organ of ITT (International Telephone Union) Recommendation X.731 that was approved on Jan 17th 1992. This is also published as ISO/IEC International Standard 10164-2.

### Perform Comm Channel BIT

## Application Use Cases

### List Applications

Use Case Identifier: List Applications

Description: This use case addresses the ability of an actor to retrieve installed applications from the radio.

External Actors: Comm Supporter

Related Use Cases:

Precondition: Radio is Operational Triggering event:

- An actor requests installed applications information from the Radio. - The radio returns a list of installed applications, along with their characteristics.

Resulting event: Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

### Remove Application

Use Case Identifier: Remove Application

Description: This use case describes the steps for uninstalling an application such as waveform from the Radio Set.

External Actors: Supporter

Related Use Cases: List Applications, Notify Event

Precondition: The Radio is operational. Triggering event: The Supporter selects an application to be uninstalled from the Radio

- The actor retrieves a list of installed applications from the Radio Set. - The actor selects an application to be uninstalled from the Radio Set. - The Radio Set uninstalls the application from the radio, so no more application instances can be created from the application. - The Radio Set removes all the application files from its file system. - An Event is sent indicating the Application that was removed.

Resulting event: The Radio sends an event indicating which application has been uninstalled from this radio. Postcondition: The Application factory deployer component and its associated files are removed from radio set.

Alternatives:

Nonfunctional requirements:

Comments:

### Add Application

### Change Application Operational Mode

### Instantiate Application

### Uninstantiate Application

## Hardware Use Cases

### List Hardware

Use Case Identifier: List Hardware

Description: List the status of all hardware in a radio set.

External Actors: Maintainer, Operator, Administrator

Related Use Cases: Get Hardware Configuration

Precondition: Triggering event: An actor requests a list of hardware devices in the radio set.

Main Flow: - The radio set obtains the status of each hardware device in the system. - The radio set returns a list of hardware devices in the system and their characteristics. Types of characteristics are: - Hierarchy of nodes and device relationships - State information (administrative, operational, usage) - Configuration attributes

Resulting event:. Postcondition:.

Alternatives:

Nonfunctional requirements:

Comments:

### Change Comm Hardware Administrative State

Use Case Identifier: Change Comm Hardware Administrative State

Description: This use case addresses the ability of an actor to change a hardware administrative state. The administrative state indicates the permission to use or prohibition against using the hardware.

External Actors: Comm Supporter

Related Use Cases:

Precondition: Hardware is either in an administrative state of UNLOCKED, Transitioning to LOCKED, or LOCKED. Triggering event:

Main Flow: - An actor requests a change in the administrative mode of operation for a hardware. - The hardware transitions to either UNLOCKED, Transitioning to SHUTTING_DOWN or LOCKED depending of the state it is in as shown in the following Figure. Figure. State Transition Diagram for Hardware’s Administrative State 3. The Hardware sends a state change event indicating the administrative state the hardware is currently in. The event data will be populated as follows: - The producerId field is the identifier attribute of the Hardware. - The sourceId field is the identifier attribute of the Hardware. - The stateChangeCategory field is ADMINISTRATIVE_STATE_EVENT. - The stateChangeFrom and stateChangeTo fields shall reflect the administrative state before and after the state change, respectively.

Resulting event: Postcondition: Communication Channel is in either UNLOCKED, SHUTTING_DOWN, or LOCKED.

Alternatives: 3a) No event is sent when there is no change in administrative state.

Nonfunctional requirements:

Comments: When the Communication Channel’s administrative state is LOCKED, it cannot be instantiated for waveform usage. The Administrative States are based upon CCITT (the International Telegraph and Telephone Consultative Committee), which is a permanent organ of ITT (International Telephone Union) Recommendation X.731 that was approved on Jan 17th 1992. This is also published as ISO/IEC International Standard 10164-2.

### Add Hardware

Use Case Identifier: Add Hardware

Description: Hardware is added to an existing radio set.

External Actors: Maintainer, Notify Event

Related Use Cases: - Perform Built In Test - Enable Hardware

Precondition: Triggering event: One of: - The radio system detects that new hardware has been added. - The new hardware proactively notifies the radio set that it has been added. - The maintainer notifies the radio set that new hardware has been added.

Main Flow: - The radio system identifies the new hardware. - The radio system verifies that no incompatibilities exist between the new and pre-existing hardware. - The radio system instructs the new hardware to perform its power-on Built In Test. - The radio system verifies that all software required by the new hardware is available. - The radio system enables the hardware as defined in Enable Hardware Use Case.

Resulting event: An appropriate radio event is generated. Postcondition: The new hardware is available for use by applications.

Alternatives: 1 a) The new hardware cannot be identified - An appropriate failure radio event is generated. 2 a) Incompatibilities are detected between the new and pre-existing hardware. - An appropriate failure radio event is generated. 3 a) The new hardware fails its power-on Built In Test. - An appropriate failure radio event is generated. 4 a) Not all software required by the new hardware is available. - An appropriate failure radio event is generated.

Nonfunctional requirements:

Comments: - Hardware may be added to a functioning radio set (Hotswap). - It is assumed that correctly functioning hardware provides interfaces via which it may be identified and instructed to conduct its Built In Test. - It is assumed that the new hardware initializes itself in the disabled state.

### Set Hardware Configuration

Use Case Identifier: Configure Hardware

Description: This use case addresses the ability of an actor to configure hardware.

External Actors: Supporter

Related Use Cases:

Precondition: Triggering event:

Main Flow: - An actor requests configuration changes to a hardware device. - Hardware performs requested configuration changes.

Resulting event: Postcondition: Hardware configuration has been modified.

Alternatives: 2a) No configuration is performed, the configuration parameters are invalid for the hardware. 2b) Partial configuration is performed, Some configuration parameters are invalid for the Hardware Component.

Nonfunctional requirements:

Comments:

### Get Hardware Configuration

Use Case Identifier: Get Hardware Configuration

Description: This use case addresses the ability of an actor to retrieve configure hardware.

External Actors: Supporter

Related Use Cases:

Precondition: Triggering event:

Main Flow: - An actor requests configuration information from a hardware device. - Hardware returns requested configuration information

Resulting event: Postcondition:

Alternatives: 2a) No configuration is returned, the query parameters are invalid for the hardware. 2b) Partial configuration is retrurned, some query parameters are invalid for the Hardware Component.

Nonfunctional requirements:

Comments:

### Remove Hardware

Use Case Identifier: Remove Hardware

Description: Hardware is removed from a radio set.

External Actors: Maintainer, Notify event

Related Use Cases: - Remove Application - Change Hardware Administrative State

Precondition: Triggering event: The maintainer notifies the radio set that hardware devices are to be removed.

Main Flow: - The radio system determines if removal if these devices will impact any other hardware. - The radio system removes (or redeploys) any applications that are making use of the hardware device to be removed, or any impacted hardware devices, as described in the Remove Application Use Case. - The radio system disables the hardware devices to be removed as well as any impacted hardware devices as described in Disable Hardware Use Case.

Resulting event: An appropriate radio event is generated. (The maintainer uses this event to determine that it is safe to power down/physically remove the hardware if required). Postcondition: The radio set continues to operate as permitted by the remaining hardware.

Alternatives:

Nonfunctional requirements:

Comments: - It is assumed that the hardware will be removed irrespective of whether applications, which are using it, are successfully removed or redeployed. - It is assumed that the hardware will be removed irrespective of whether it can be successfully disabled. - It may not (Hotswap case) be necessary to power down the radio set in order to physically remove the hardware.

### Perform Hardware BIT

Use Case Identifier: Perform Hardware BIT

Description: An actor performs hardware BIT

External Actors: Maintainer

Related Use Cases: Change Hardware Administrative State

Precondition: Triggering event:

Main Flow:

Resulting event:

Alternatives:

Nonfunctional requirements:

Comments:

## Alarms Use Cases

### Change Alarm Operational Mode

Use Case Identifier: Change Alarm Operational Mode

Description: Describes the use case steps for changing Alarm’s Operational (enable or disable) mode.

External Actors: Supporter

Related Use Cases: List Alarms

Precondition: Triggering event:

Main Flow: - Operator retrieves the alarm types from radio set. - Operator selects an alarm type to be enabled or disabled. - Radio Set changes alarm operational mode.

Resulting event: Postcondition: The Alarm is either in disable or enable operational mode.

Alternatives:

Nonfunctional requirements:

Comments: There can be different types of alarms that an radio set could understand.

### List Alarms

Use Case Identifier: List Alarms

Description: List the Alarms in a radio set.

External Actors: Supporter

Related Use Cases:

Precondition: Triggering event: An actor requests a list of Alarms in the radio set.

Main Flow: - An actor requests a list of Alarms in the radio set. - The radio set returns a list Alarms configurations:

Resulting event:. Postcondition:.

Alternatives:

Nonfunctional requirements:

Comments:

### Modify Alarm

Use Case Identifier: Modify Alarm

Description: Describes the steps of modifying the alarm event for an alarm type within a radio set

External Actors: Operator

Related Use Cases: List Alarms

Precondition: Triggering event:

Main Flow: - Operator retrieves the default (active) alarm configuration from the radio. - Operator selects an event alarm to be modified. - Operator selects an alarm type by selecting from the available radio’s alarms repository. - Operator saves the changes

Resulting event: Postcondition:

Alternatives: -

Nonfunctional requirements: - None

Comments: - Events are domain specific. There can be different types of alarms that an alarm service could understand.

### Trigger Alarm

Use Case Identifier: Trigger Alarm

Description: Describes the steps of triggering an alarm within a radio set.

External Actors: Comm User

Related Use Cases:

Precondition: Triggering event: A radio set’s event (e.g., security violation, incoming call, time notice, etc.) has occurred.

Main Flow: - The radio set ( or a component within) detects an event condition. - The component that detected the event sends an event notification to the Alarm Service. - The Alarm Service triggers the event alarm message to the appropriate component (e.g., audio, button, generates an event) within the radio set.

Resulting event: Operator or radio system receives notice of the alarm. Postcondition:

Alternatives:

Nonfunctional requirements:

Comments:

### Remove Alarm Configuration

### Create Alarm Configuration

### Enable Alarm Configuration

### Events

### Notify Radio Event

Use Case Identifier: Notify Radio Event

Description: Notify all registered parties of the occurrence of a radio event from the radio set or a component within the radio set.

External Actors: Comm User

Related Use Cases:

Precondition: Triggering event: Any radio event.

Main Flow: - A radio event occurs. - The radio set contacts all actors who have registered an interest in the event, or in an event group to which the event belongs, using the contact information recorded for each actor.

Resulting event: Postcondition:

Alternatives: 2 a) A registered actor cannot be contacted using the recoded contact information. - The radio set logs an error and continues to contact other registered actors.

Nonfunctional requirements:

Comments: This can be due to hardware or radio set state changes, radio set domain changes as items are removed or added, or threshold changes

### Register to Receive Radio Events

Use Case Identifier: Register to Receive Radio Events

Description: An actor with an interest in certain events within the radio set/system requests to be notified when these events occur in/ are generated by the radio set/system.

External Actors: Comm User, Radio Set, Radio System

Related Use Cases:

Precondition: None Triggering event: An actor registers to receive radio set events

Main Flow: - An actor registers to receive radio events. (The request indicates which event/s or event group/s the actor is interested in, and how the actor should be contacted when these occur.) - The radio set validates that the particular actor is permitted to register for all the specified event/s or event group/s. - The radio set validates that the particular actor is not already registered for any of the specified event/s or event group/s. - The radio set acknowledges the event registration request.

Resulting event: The radio set acknowledges the event registration request. Postcondition: The actor’s contact information is recorded against the specified event/s.

Alternatives: 2 a) Validation of the actor for one or more of the specifed event/s or event groups/s fails. 3 a) 4.a) The radio set rejects the event registration request and no information is recorded against the specified events/s

Nonfunctional requirements:

Comments: - Each type of actor is eligible to receive a pre-determined subset of radio events.

### Unregister to Receive Radio Events

Use Case Identifier: Unregister to Receive Radio Events

Description: An actor who previously registered an interest in certain events within the radio set/system requests to no longer be notified when these events occur.

External Actors: Comm User

Related Use Cases:

Precondition: The actor’s contact information is recorded against specified event/s. Triggering event: An actor unregisters to receive radio events

Main Flow: - An actor unregisters to receive radio events. (The request indicates which event/s or event group/s the actor wishes to unregister.) - The radio set validates that the particular actor is currently registered for all of the specified event/s or event group/s. - The radio set acknowledges the event unregister request.

Resulting event: The radio set acknowledges the event unregister request. Postcondition: No contact information for the actor is recorded against the specified event/s.

Alternatives: 2 a) The actor is not already registered for all specified events. 3.a) The radio set rejects the event unregister request and no change is made to the information recorded against the specified events/s

Nonfunctional requirements:

Comments: