# Use Case Identifier: GatherPOSTResults

Description: The Radio System gathers the POST results and provides them to the Operator on power-up.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event: The Operator requests POST Results or unsolicited at power-on.

- The Radio System gets a list of the current HW devices in the system. - The Radio System verifies that each HW device has registered into the system and then obtains the POST results from each HW device. - The Radio System provides the POST results to the Operator.

Resulting event: None Postcondition: POST results are collected and provided to the Operator.

Alternatives: ### None

Nonfunctional requirements: None

Comments: None

# Use Case Identifier: GetRadioHealth

Description: An Operator requests the current health of the radio.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system Triggering event:

- The Operator requests the current health of the radio. - The Radio System provides a summary status of the current health of the radio.

Resulting event: None Postcondition: Current health of the radio is provided to the Operator.

Alternatives: ### None

Nonfunctional requirements: None

Comments: Health states need to be defined at each level Radio, Application, Communication Channel, Devices

# Use Case Identifier: StartContinuousBIT

Description: The Radio System begins to perform continuous (background) BIT.

External Actors: None

Related Use Cases: None

Precondition: POST completed successfully Triggering event: POST is complete

- The Radio System commands each HW device to run continuous BIT

Resulting event: None Postcondition: Each HW device has been commanded to run continuous BIT

Alternatives: ### None

Nonfunctional requirements: None

Comments: - Do all the H/W devices need to run continuous BIT? Or can it be constrained? - Need mechanism for providing BIT status - Need mechanism for providing test conditions/parameters to the BIT operations. - Again need concept of Radio, Application, Comm Channel & Device level BIT.

# Use Case Identifier: MonitorContinuousBIT

Description: The Radio System monitors for Continuous BIT failures.

External Actors: None

Related Use Cases: None

Precondition: None Triggering event: Continuous BIT successfully started

- The Radio System subscribes to and is listening to an event channel for BIT alarms. - At a periodic rate, the Radio System commands each HW device to start continuous BIT. This is similar to “pinging” the HW device component to ensure that it is still alive. - If a HW device has a continuous BIT failure, it submits a BIT alarm to the event channel. - The Radio System detects a failure if a BIT alarm is detected or a HW device fails its “ping” test.

Resulting event: None Postcondition: The Radio System has detected a HW failure due to a BIT alarm or a failure to communicate with a HW device.

Alternatives: ### None

Nonfunctional requirements: None

Comments: None

# Use Case Identifier: ReportContinuousBITFault

Description: The Operator is notified of a BIT fault.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event:

- The Radio System has detected a Continuous BIT fault. - The Operator is presented with a BIT fault indication.

Resulting event: None Postcondition: The Operator is notified of a BIT fault indication.

Alternatives: ### None

Nonfunctional requirements: None

Comments: None

# Use Case Identifier: StopContinuousBIT

Description: The Operator has decided to stop continuous BIT through-out the radio.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event:

- The Operator selects to stop continuous BIT for the entire radio. - The Radio System accepts the request and stops continuous BIT on each HW device in the radio.

Resulting event: None Postcondition: Continuous BIT has been stopped throughout the radio.

Alternatives: ### None

Nonfunctional requirements: None

Comments: None

# Use Case Identifier: StopSpecificContinuousBIT

Description: The Operator needs to stop continuous BIT for a specified Device.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event:

- The Operator selects the device on which to stop continuous BIT. - The Radio System commands the device to stop running continuous BIT. - The Radio System reports to the Operator that the device is no longer running continuous BIT

Resulting event: None Postcondition: The Radio System has stopped running continuous BIT on the selected device.

Alternatives: ### None

Nonfunctional requirements: None

Comments: None

# Use Case Identifier: RestartSpecificContinuousBIT

Description: The Operator needs to restart continuous BIT for a device that had been previously not running continuous BIT.

External Actors: Operator

Related Use Cases: None

Precondition: The Operator is logged into the system. Triggering event:

- The Operator selects the HW device to restart the continuous BIT on. - The Radio System commands the HW device to start running continuous BIT. - The Radio System reports to the Operator that the HW device is running continuous BIT

Resulting event: None Postcondition: The Radio System has restarted running continuous BIT on the selected HW device.

Alternatives: ### None

Nonfunctional requirements: None

Comments: None

# Use Case Identifier: RunSpecificCommandedBIT

Description: The Operator selects a specific commanded BIT to run on a HW device.

External Actors: Operator

Related Use Cases: StopSpecificContinuousBIT

Precondition: The Operator is logged into the system. Triggering event:

- The Operator selects from a list of possible commanded BIT tests for a specific HW device. - The Radio System commands the HW device to run the specified commanded BIT. - The Radio System provides the test results to the Operator.

Resulting event: None Postcondition: Commanded BIT test results are provided to the Operator.

Alternatives: ### None

Nonfunctional requirements: None

Comments: None