On Protocol Stacks, OSI Layers and SDR

The purpose of this document is to clarify the interface definitions of the various layers in the OSI layers as pertaining to a SDR system within the context of the document “Comments on the PIM and PSM for SWRADIO Components” of February 20th.

As proposed in [1], any communication equipment can be view as an apparatus to transform a received symbol stream into another symbol stream. This transformation is accomplished via the use of protocol stacks. The symbol stream comes into the equipment goes into a protocol stack at the physical level, rises in the stack to the application level where it exists as information. It then may enter another protocol stack as information and travels down this new stack to exit via the physical layer as a potentially very different symbol stream. Communication equipment may support multiple stacks.  A simplified bloc diagram illustrating this principle is given in Figure 1.

Figure 1 - Communication equipment viewed as protocol conversion equipment.

The proposed protocol stack layer is mirrored from the OSI standard [2] in which the following seven layers are present. The layers are:

- Application Layer;

- Presentation Layer;

- Session Layer;

- Transport Layer;

- Network Layer;

- Data Link Layer;

- Physical Layer;

Typical radio equipment, cellular phone like, have a limited protocol stack, usually including only the physical and data link layers on the RF side and only a physical layer on the IO/user side.

## Stack layers VS API

In general, when faced with an implementation problem, the designer should answer the question whether the desired functionality can be accommodated with “available” stack layers (standard or specific). If this is the case then the implementation issue is resolved. If the answer is no then the choice becomes to either create a new stack layer or layers, as part of a standardization effort or as part of a specific implementation, or to add API(s) to an existing stack layer or layers to accommodate the new functionality.

Given that APIs are common across all same level stack layers, the option to add an API must be weighed against the portability and interchangeability issues that will be raised by such a choice. Creating specific layers for every standard and specific protocols, with common APIs, appears to be a much more portable solution.

For the designer, the issue will then become that of finding an already implemented layer in is stack layer library or adding a new stack layer to the library. If all APIs are common then all designers will then be able to benefit from the new layer. Furthermore if the designer’s organization decides to promote this new layer as part of a standard then the entire community could benefit.

## General Note on API

In general there are two ways of attaining a particular functionality. The first is to use small specific modules with compatible interfaces and the second is to use larger parameterized blocks that can emulate multiple functionalities. The main advantage of the first is efficiency while the main advantage of the second is reduced component count. The first strategy is preferable when little communality exists between “similar functionality algorithms” while the second strategy is useful when clear mathematical relationships exist between all possible structures. Traditionally DSP packages like Simulink and SPW have followed this approach.

The SDR framework enables both approaches. The concept of dynamic instantiation can be used to instantiate a particular algorithm instead of another. If both algorithms are self contained or use identical interfaces then the use of one or the other is only a question of waveform definition. In this case APIs have to be consistent and general enough to support this change. Similarly, if “similar functionality algorithms” are general and their behavior is controllable by a small number of parameters then few general APIs are required. Both of these approaches generally lead to few very general APIs.

Problems only arise when trying to define general common APIs for dissimilar algorithms. The approach taken here was to favor the use of small efficient modules to implement dissimilar algorithms and thus rely on the instantiation process to enable the creation of multiple waveform equipment. Whenever large communality was found, then general APIs with mathematical concepts were used. The APIs where generally drafted along the lines of the ISO model according to the proposition of [1].

In the context of this document we try not make any assumption on the design and implementation choices faces by the designers of a particular system. This leads to the conclusion that some APIs may in fact have firmware and/or hardware implementations as opposed to a system call. This is obvious for very high speed point to point systems where power efficient implementation in the tens of megabaud dictate that there cannot be processors involve in the signal processing chain.

# Physical Layer

The role of this layer is to act as transducer between the physical medium and the Data Link Layer.

On the RF side of any wireless device, physical medium access is constrained by regulatory bodies. These bodies typically specify a mask, for a given frequency location and maximum power output. The ability to control these parameters then constitutes the minimum requirements of any physical layer.

On the IO/user side of communication equipment, a wide variety of standard interfaces are found. Typical examples include telephony standards, Ethernet, RS232… Most interfaces specify power levels and pulse shape. These are very similar to the power and frequency response characteristics presented earlier.

This text will support the view that the physical layer is responsible for the transmission and reception of a symbol stream. It is further assumed that both the transmitter and the receiver will receive and transmit symbols at the baud rate.

The physical layer is responsible for the following transformations/operations on this symbol stream:

- Channel adaptation filtering

- Frequency conversion

- Amplification

- Transducer response

- Timing

- QOS measurement

In general these transformations apply to the transmitter as well as to the receiver portion of the physical layer. All transformations may or may not be required in any particular physical layer.

It is also interesting to note that in a particular stack some of the adaptive controls for all of these functions may be decision directed or data aided; in these cases the physical service user will not set a static parameter but rather constantly call the physical service provider to dynamically change the parameters as desired by the data link layer.

These transformations/operations correspond with the IF-RF facilities, including the possibility of digital RF/IF processing, and the IO facilities in the SWRADIO initial submission.

## Channel adaptation filtering

Channel adaptation filtering may or may not be present in a particular physical layer. Channel adaptation filtering may or may not include any or all of the following functions:

- Time domain filtering

- Frequency domain filtering

- Space-frequency combining

A non-exhaustive list of components involved in channel adaptation filtering could be digital FIR and IIR filters, analog passive and active filters, SAW filters, time and frequency domain equalizers, FFT and IFFT, MIMO combiners, duplexers and antenna or antenna arrays.

In general, the specification of any channel adaptation filtering is performed via a frequency response. It is assumed that the physical layer would then partition this frequency response into partial responses for the entire chain considering the abilities and components involved in the chain in question. It is also understood that the physical layer would be able to report on the frequency response in the chain.

The API for this functionality is then the ability to set and read a frequency response.

## Frequency Conversion

Frequency conversion may or may not be present in a particular physical layer. Frequency conversion may or may not include any or all of the following functions:

- Mixing

- Frequency synthesis

A non-exhaustive list of components involved in frequency conversion could be NCO VCO, XO, DDS, doublers, harmonic selectors and mixers.

In general, the specification of any frequency conversion is simply specifying the frequency of interest. It is assumed that the physical layer would then partition this frequency setting into intermediate settings for the entire chain considering the abilities and components involved in the chain in question. It is also understood that the physical layer would be able to report on the frequency settings in the chain.

The API for this functionality is then the ability to set and read a frequency.

## Amplification

Amplification may or may not be present in a particular physical layer. Amplification may or may not include any or all of the following functions:

- Amplification

- Power control

A non-exhaustive list of components involved in amplification could be power amplifiers, low noise amplifiers, voltage controlled amplifiers, -law converters, squelch functions.

In general, the specification of any amplification is performed via a power setting. It is assumed that the physical layer would then partition this power setting into partial settings for the entire chain considering the abilities and components involved in the chain in question. It is also understood that the physical layer would be able to report on the settings in the chain.

The API for this functionality is then the ability to set and read a power setting.

## Transducer response

Transducer response may or may not be present in a particular physical layer. Transducer response may or may not include any or all of the following functions:

- Transducer pattern control

- Co-channel mitigation

A non-exhaustive list of components involved in transducer response could be antenna arrays, microphones, speakers, and multiple channel combiners.

In general, the specification of any transducer response is performed via a radiation pattern. It is assumed that the physical layer would then partition this radiation pattern into partial patterns for the entire chain considering the abilities and components involved in the chain in question. It is also understood that the physical layer would be able to report on the patterns in the chain.

The API for this functionality is then the ability to set and read a radiation pattern.

## Timing

Timing may or may not be present in a particular physical layer. Timing may or may not include any or all of the following functions:

- Clock generation and recovery

- Burst synchronization

A non-exhaustive list of components involved in timing could be XO, OCXO, VCO, DDS, PLL, buffers, special control signals.

In general, the specification of any timing is performed via a frequency setting. It is assumed that the physical layer would then partition this frequency setting into partial settings for the entire chain considering the abilities and components involved in the chain in question. It is also understood that the physical layer would be able to report on the settings in the chain.

The API for this functionality is then the ability to set and read a baud rate.

## QOS Measurement

QOS measurement may or may not be present in a particular physical layer. QOS measurement may or may not include any or all of the following functions:

- Error probability measurement

- Availability measurement

- Transmission rate

A non-exhaustive list of components involved in QOS measurements could be signal samplers (ie buffers, power splitters…), instrumentation type measurement (Power, peak to average, adjacent channel position, eye opening, BER monitors) and throughput estimators, (edge detectors, PLL…).

In general, the specification of QOS measurements is performed via performance criteria. It is assumed that the physical layer would then partition these performance criteria into specific items for the entire chain considering the abilities and components involved in the chain in question. It is also understood that the physical layer would be able to report on the performance criteria in the chain.

The API for this functionality is then the ability to set and read performance criteria..

## ISO Model Physical Layer Service Specifications

The ISO model specifies the following services that the Physical Layer should offer to the Data Link Layer.

Service

Physical-connections

Physical-service-data-units

Physical-connection-endpoints

Data-circuit identification

Sequencing

Fault condition notification

Quality of service parameters

Table 1 - OSI Specified Physical Layer Services.

Creating an API using the above discussion and the ISO specification leads us to the following possible class diagram (APIs).

Figure 2 – Physical layer class diagram

# Data Link Layer

The role of this layer is to entail the channel specific techniques that are required to provide a reliable link to the upper layers. Reliable means as error free as the upper layers need it to be. A link itself is a logical point-to-point communication path between two data link layer entities located on adjacent nodes using the same data link layer protocol.  From the link entities point of view, data exchanged on the link cannot transit in any other layer above the data link layer.

The criterion for the presence of a data link layer is the presence of digital signal processing.  A completely analog radio has a transparent data link layer.

After the physical layer, whether the layers are on the RF side or the IO/user side of the communication equipment, it should not matter. It should be transparent. The APIs therefore should be kept general and goal oriented.

Again, this text will support the view that the physical layer is responsible for the transmission and reception of a symbol stream. It is further assumed that the data link layer communicates with the physical layer via a symbol stream at the baud rate.

The data link layer is responsible for the conversion between data stream to symbol stream and symbol stream to reliable data link. This implies complex signal processing algorithms whose measure of efficiency is only determined by whether or not the layer meets its QOS target. From this discussion, it stems that the APIs cannot have signal processing angles.

The data link layer is responsible for the following transformations/operations on this data stream:

- QOS measurement

The functions of the data link layer correspond with most of the Waveform processing facilities and the most elaborate IO facilities in the SWRADIO initial submission. Note that the MAC layer is not included here as it is a sub layer of the link layer. It should not need exposed API other that the ones provided by the link layer to the network layer.

## QOS Measurement

QOS measurement may or may not be present in a particular data link layer. QOS measurement may or may not include any or all of the following functions:

- Error probability measurement

- Availability measurement

- Transmission rate

A non-exhaustive list of components involved in QOS measurements could be source and channel encoder/decoder pairs, framing, interleaving, modulation/demodulation, various detectors and estimators.

In general, the specification of QOS measurements is performed via performance criteria. It is assumed that the data link layer would then partition these performance criteria into specific items for the entire chain considering the abilities and components involved in the chain in question. It is also understood that the data link layer would be able to report on the performance criteria in the chain.

The API for this functionality is then the ability to set and read performance criteria.

## ISO Model Data Link Layer Service Specifications

The ISO model specifies the following services that the Data Link Layer should offer to the Network Layer.

| Service | Connection mode |
| --- | --- |
| data-link-addresses | Both |
| data-link-connection | Connection |
| data-link-service-data-units | Both |
| data-link-connection-endpoint-identifiers | Connection |
| error notification | Connection |
| quality of service parameters | Both |
| reset | Connection |

Table 2 - OSI Specified Data Link Layer Services.

Mapping these services into APIs lead to the following possible class diagram (APIs).

Figure 3 – Data link layer class diagram

# References

- J. Belzile, “Comments on the PIM and PSM for SWRADIO Components”, Submitted to the OMG DSIG SWRADIO working group, Feb 20th.

- ITU-T, “Information Technology – Open systems Interconnection – Basic Reference Model: The basic model”, ITU-T Recommendation X.200

# Appendix 1

To aid the reader, the text of the ITU-T X200 recommendation with regards to the purpose of each layer is reproduced in this appendix.

### Application Layer

The purpose of the application layer, as defined by the OSI is:

As the highest layer in the Reference Model of Open Systems Interconnection, the Application Layer provides the sole means for the application process to access the OSIE. Hence the Application Layer has no boundary with a higher layer.

The aspects of an application process which need to be taken into account for the purpose of OSI are represented by one or more application-entities.

An application-entity represents one and only one application process in the OSIE. Different application processes may be represented by application-entities of the same application-entity-type. An application process may be represented by a set of application-entitites: each application-entity in this set may be, but need not be, of different application-entity-type.

### Presentation Layer

The purpose of the presentation layer, as defined by the OSI is

The Presentation Layer provides for the representation of information that application-entities either communicate or refer to in their communication.

The Presentation Layer provides for common representation of the data transferred between application-entities. This relieves application-entities of any concern with the problem of “common” representation of information, i.e. it provides them with syntax independence.

The Presentation Layer ensures that the information content of the Application Layer data is preserved during transfer. Cooperating application-entities are responsible for determining the set of abstract syntaxes they employ in their communication. The Presentation Layer is informed of the abstract syntaxes that are to be employed. Knowing the set of abstract syntaxes to be used by the application-entities, the Presentation Layer is responsible for selecting mutually acceptable transfer syntaxes.

### Session Layer

The purpose of the session layer, as defined by the OSI is:

The purpose of the Session Layer is to provide the means necessary for cooperating presentation-entities to organize and to synchronize their dialogue and to manage their data exchange. To do this, the Session Layer provides services to establish a session-connection between two presentation-entities, to support orderly data exchange interactions, and to release the connection in an orderly manner.

The only function of the Session Layer for connectionless-mode communication is to provide a mapping of transport-addresses to session-addresses.

A session-connection is created when requested by a presentation-entity at a session-service-access-point. The session-connection exists until it is released by either the presentation-entities or the session-entities.

The initiating presentation-entity designates the destination presentation-entity by a session address. In general, there is a many-to-one correspondence between the session-addresses and transport-addresses. This does not imply multiplexing of session-connections onto transport-connections, but does imply that at session-connection establishment time, more than one presentation-entity is a potential target of a session-connection establishment request arriving on a given transport-connection. However, when needed, there may be a one-to-one correspondence between the session-address and the transport-address.

### Transport Layer

The purpose of the transport layer, as defined by the OSI is:

The transport-service provides transparent transfer of data between session-entities and relieves them from any concern with the detailed way in which reliable and cost effective transfer of data is achieved.

The Transport Layer optimizes the use of the available network-service to provide the performance required by each session-entity at minimum cost. This optimization is achieved within the constraints imposed by the overall demands of all concurrent session-entities and the overall quality and capacity of the network-service available to the Transport Layer.

All protocols defined in the Transport Layer have end-to-end significance, where the ends are defined as transport entities having transport associations. Therefore, the Transport Layer is OSI end open system oriented and transport-protocols operate only between OSI end open systems.

The Transport Layer is relieved of any concern with routing and relaying since the network-service provides data transfer from any transport-entity to any other, including the case of tandem subnetworks (see Network Layer).

The transport functions invoked in the Transport Layer to provide a requested service quality depend on the quality of the network-service. The quality of the network-service depends on the way the network-service is achieved (see Network Layer).

### Network Layer

The purpose of the network layer, as defined by the OSI is:

The Network Layer provides the functional and procedural means for connectionless-mode or connection-mode transmission among transport-entities and, therefore, provides to the transport-entities independence of routing and relay considerations.

The Network Layer provides the means to establish, maintain, and terminate network-connections between open systems containing communicating application-entities and the functional and procedural means to exchange network-service-data-units between transport-entities over network-connections.

It provides to the transport-entities independence from routing and relay consideration associated with the establishment and operation of a given network-connection. This includes the case where several subnetworks are used in tandem or in parallel. It makes invisible to transport-entities how underlying resources such as data-link-connections are used to provide network-connections.

Any relay functions and hop-by-hop service enhancement protocols used to support the network-service between the OSI end systems are operating below the Transport Layer, i.e. within the Network Layer or below.

### Data Link Layer

The purpose of the data link layer, as defined by the OSI is:

The Data Link Layer provides functional and procedural means for connectionless-mode among network-entities, and for connection-mode for the establishment, maintenance, and release data-link-connections among network-entities and for the transfer of data-link-service-data-units. A data-link-connection is built upon one or several physical-connections.

The Data Link Layer detects and possibly corrects errors which may occur in the Physical Layer.

In addition, the Data Link Layer enables the Network Layer to control the interconnection of data-circuits within the Physical Layer.

### Physical Layer

The purpose of the physical layer, as defined by the OSI is:

The Physical Layer provides the mechanical, electrical, functional and procedural means to activate, maintain, and de-activate physical-connections for bit transmission between data-link-entities. A physical-connection may involve intermediate open systems, each relaying bit transmission within the Physical Layer. Physical Layer entities are interconnected by means of a physical medium.

# Appendix 2

The dynamic view of the APIs are presented here to support the static class diagrams found in the document.

Figure 3 – DataLink connection establishment

Figure 4 – DataLink connection tear down

Figure 5 – DataLink connectionless data transfer

Figure 6 – Physical connection binding

Figure 7 – Physical connection unbinding

Figure 8 – Physical data circuits interconnection

Some protocols may or may not have processing in any given layers. The layer is then considered to be present but to be transparent. This is so that the concept of information stream.

A good example is error control coding where drastically different decoding algorithms (RS, Viterbi, Sequential, CSOC) are present.

A good example is equalizer adaptation. The only difference in many cases is the error function used to adapt the filter (LMS, CMA…).

It is also believed that some efficiency gains can be made by defining “lower level” common interfaces. This activity is known as HAL (Hardware abstraction layer) definition in the SDRF (Software Defined Radio Forum).

This normally includes the RF unit signal processing capabilities, the IO device functionality and much of the baseband real time signal processing.

In the current proposition, the physical layer is assumed to receive symbols from the data link layer as opposed to bits or strings of bits (serial or parallel interpretation) in the ISO interpretation. This difference can be attributed to the fact that the ISO standard was written with more of a “wired” than a “wireless” approach. In the wired world spectrum efficiency is rarely a concern and hence binary modulation schemes are extremely popular. In that context a bit may be equal to a symbol. In the wireless world, the distinction is capital.

For the purpose of this proposition, a symbol is defined as the unit of data that is to be directly ported on the physical medium. For example, a CDMA symbol is a chip, a radar symbol is a chirp.

Digital to analog and analog to digital conversions are not mentioned as there usage is only an implementation issue and not a functional property of the physical layer.

This type of transformation is absent from most IO interface as they are mostly base band signaling.

As a form of non-linear amplification.

In some cases, the physical layer must be informed not only of the symbols to transmit and the operations to apply to those symbols but also of the symbol rate.

Hardware fault management should be handled by the operating environment.

A data_link_connection is established and released dynamically. Therefore there is no public API function call associated with it.