On Waveforms Characteristics and OSI Layer APIs

The purpose of this document is to clarify the relationship between waveform characteristics as defined in [1], the model of [3] and the APIs defined in [2].

In [1], it is proposed that waveforms be classified along an N-dimensional mapping. The suggestion is that any communication equipment can be mapped into a set of main characteristics and that the proposed N dimensional model could be use to specify waveforms and to implement waveforms in a controlled/automatic manner. It is further suggested that these dimensions also map into OSI Layer.

It is the purpose of this document to show and motivate the mapping of the N dimensional waveform characteristic model into the OSI seven layer model. It is also the purpose of the document to motivate the APIs proposed in [2] by showing that they allow the implementation of these characteristics in their respective layers.

We recognized that the identification of each dimension in the waveform characteristic may not be completed at this time and that some refining may be needed. This mapping is thus preliminary; however we are confident that as the waveform characteristic model evolves that a layer of the OSI model can be identified for each characteristic.

Table 1. Waveform Characteristics and their Relation to the OSI Layer Model

| Waveform Characteristic | Examples of Possible Options | OSI Layer Maping |
| --- | --- | --- |
| Packet content | voice (analog & digital), data (digital), voice & data (digital) | Presentation |
| Connection type | networking, point-to-point, broadcast, multicast | Link |
| Switching type | circuit, packet | Network |
| LOS | Exist, beyond | Physical & Link |
| Access mode | PTT, trunked, DAMA,TDMA,CDMA, CSMA, FDMA | Link |
| Security | transmission, communication, none | ALL |
| Spectral mask | center frequency, instantaneous frequency, bandwidth, power | Physical |
| Channel coding | Convolutional, turbo, Reed Solomon, CRC,… none | Link & Transport |
| Routing | RF packets, IO packets | Network |
| Source coding | CVSD, MPEG, MP3, ACELP, none | Presentation |
| Channel estimation | adaptive, trained, blind, none | Link |
| Power control | open loop, closed loop, none | Link |
| Diversity | Transmit, Receive, Frequency, Space, Time, MIMO | Link |

Table 1 above assigns for every general waveform characteristic one ore more OSI model layer involved in the implementation of the algorithms of the mentioned functionality.

In general there are two ways of attaining a particular functionality. The first is to use small specific modules with compatible interfaces and the second is to use larger parameterized blocks that can emulate multiple functionalities. The main advantage of the first is efficiency while the main advantage of the second is reduced component count. The first strategy is preferable when little communality exists between “similar functionality algorithms” while the second strategy is useful when clear mathematical relationships exist between all possible structures. Traditionally DSP packages like Simulink and SPW have followed this approach.

The SDR framework enables both approaches. The concept of dynamic instantiation can be used to instantiate a particular algorithm instead of another. If both algorithms are self contained or use identical interfaces then the use of one or the other is only a question of waveform definition. In this case APIs have to be consistent and general enough to support this change. Similarly, if “similar functionality algorithms” are general and their behavior is controllable by a small number of parameters then few general APIs are required. Both of these approaches generally lead to few very general APIs.

Problems only arise when trying to define general common APIs for dissimilar algorithms. The approach taken here was to favor the use of small efficient modules to implement dissimilar algorithms and thus rely on the instantiation process to enable the creation of multiple waveform equipment. Whenever large communality was found, then general APIs with mathematical concepts were used. The APIs where generally drafted along the lines of the ISO model according to the proposition of [3].

As stated in [2], the APIs proposed rely heavily on the fact that instantiation can be used to resolve the problems associated with the control of dissimilar algorithms at the same OSI Layer. This will become very clear in the discussion of the waveform characteristics and their relation with the proposed APIs. Note also that only Data Link Layer and Physical Layer APIs where identified in [2]. The discussion of the APIs for the other Layers is therefore just a suggestion based on the experience with the previous two Layers.

# Packet content

Packet content refers to actual meaning of the information being transported. The OSI layer related to this is the Presentation Layer.

The Presentation Layer provides for common representation of the data transferred between application entities. This relieves application-entities of any concern with the problem of “common” representation of information, i.e. it provides them with syntax independence.

It is proposed that the APIs related to this functionality map to the OSI proposed services. It is also proposed that the APIs not specify the functionality of the Presentation Layer but rather provide a standard interface to the Application Layer. In this way the various instances of the Presentation Layer can be interchangeable and instantiated depending on the situation at hand.

# Connection type

Connection type is a Data Link Layer process. The connection type is specific to the link-level network protocol used by the Data Link Layer. The Network Layer is unaware of the underlying type of networks on which network packets gets transported.  As an example, consider these specific Data Link Layer protocols:

- Networking: Ethernet/TokenRing/ATM

- Point-to-point: PPP/SLIP/HDLC normal

- Broadcast/Multicast: Ethernet/TokenRing

Each of these protocols is able to carry IP (Network Layer) traffic.  The Network Layer unifies all these protocols to enable interconnection between them.  Therefore, the connection type is an attribute of the Data Link Layer.

In the OSI stack, the type of connection, as implied in the option choice, is defined at the Data Link Layer. Furthermore, the Data Link Layer defines whether the process is connection oriented or connectionless oriented.

It is proposed that the APIs related to this functionality map to the OSI proposed services. It is also proposed that the APIs not specify the functionality of the Data Link Layer but rather provide a standard interface to the Network Layer. In this way the various instances of the Data Link Layer can be interchangeable and instantiated depending on the situation at hand.

# Switching type

Switching is a routing operation. The OSI layer related to this function is the Network Layer.

The Network Layer provides the functional and procedural means for connectionless-mode or connection-mode transmission among transport-entities and, therefore, provides to the transport-entities independence of routing and relay considerations.

It is proposed that the APIs related to this functionality map to the OSI proposed services. It is also proposed that the APIs not specify the functionality of the Network Layer but rather provide a standard interface to the Transport Layer. In this way the various instances of the Network Layer can be interchangeable and instantiated depending on the situation at hand.

# LOS

The ability to operate beyond line of sight is an attribute of the protocol stack chosen. This ability depends on the physical medium (i.e. the RF frequency used) hence the Physical Layer, and on the signal processing defined in the Data Link Layer.

The Physical Layer provides the mechanical, electrical, functional and procedural means to activate, maintain, and de-activate physical-connections for bit transmission between data-link-entities. A physical-connection may involve intermediate open systems, each relaying bit transmission within the Physical Layer. Physical Layer entities are interconnected by means of a physical medium.

The Data Link Layer detects and possibly corrects errors which may occur in the Physical Layer.

The APIs provided by the Physical Layer enable the control of power, frequency, timing, spectral mask and antenna patterns. These parameters completely control the space-time-frequency combination of the physical medium.

The APIs provided by the Data Link Layer map to the OSI proposed services. This avoids the problem of trying to control the widely differing error control signal processing algorithms found at the Data Link Layer via a common API. It is preferable to instantiate the proper Data Link Layer than to try to great a general one and control it via an API.

# Access mode

Access mode is a function of the Data Link Layer.

The Data Link Layer provides functional and procedural means for connectionless-mode among

network-entities, and for connection-mode for the establishment, maintenance, and release data-link-connections among network-entities and for the transfer of data-link-service-data-units. A data-link-connection is built upon one or several physical-connections.

The APIs provided by the Data Link Layer map to the OSI proposed services. This avoids the problem of trying to control the widely differing access control algorithms found at the Data Link Layer via a common API. It is preferable to instantiate the proper Data Link Layer than to try to great a general one and control it via an API.

# Security

Security, or encryption, is a special case in the sense that various protocols have been created at almost every layer in the stack. For example consider SSL (transport), IPSec (network), PGP (Presentation) and CDMA-Spread Spectrum (link). All of these address specific issues and need not all be present. Furthermore, none of these are military, or NSA, grade. It is felt that military or NSA grade encryption is really an application rather than a part of a layer.

It is also felt that no APIs should be developed for the security protocols inside the layers because the instantiation process can handle the “control” of the security at the proper level. In the case of military and NSA grade encryption APIs, this should be left to the proper authorities but it is felt that “control” APIs will never be defined for security reasons. Data flow APIs are possible but seen as unlikely again.

# Spectral mask

Spectral mask is a function of the Physical Layer.

The Physical Layer provides the mechanical, electrical, functional and procedural means to activate, maintain, and de-activate physical-connections for bit transmission between data-link-entities. A physical-connection may involve intermediate open systems, each relaying bit transmission within the Physical Layer. Physical Layer entities are interconnected by means of a physical medium.

The APIs provided by the Physical Layer enable the control of power, frequency, timing, spectral mask and antenna patterns. These parameters completely control the space-time-frequency combination of the physical medium.

# Channel coding

Channel coding is part of error control. Error control can be found in at least two important locations: at the Data Link Layer and at the Transport Layer

The Data Link Layer detects and possibly corrects errors which may occur in the Physical Layer.

g)end-to-end error detection and any necessary monitoring of the quality of service;

f) end-to-end error recovery;

The Data Link Layer controls link error control while the Transport Layer is preoccupied with end-to-end error control.

The APIs provided by the Data Link Layer map to the OSI proposed services. This avoids the problem of trying to control the widely differing error control signal processing algorithms found at the Data Link Layer via a common API. It is preferable to instantiate the proper Data Link Layer than to try to great a general one and control it via an API.

It is proposed that the APIs related to the Transport Layer error control map to the OSI proposed services. It is also proposed that the APIs not specify the functionality of the Transport Layer but rather provide a standard interface to the Session Layer. In this way the various instances of the Transport Layer can be interchangeable and instantiated depending on the situation at hand.

# Routing

The OSI layer related to this function in the Network Layer.

The Network Layer provides the functional and procedural means for connectionless-mode or connection-mode transmission among transport-entities and, therefore, provides to the transport-entities independence of routing and relay considerations.

It is proposed that the APIs related to this functionality map to the OSI proposed services. It is also proposed that the APIs not specify the functionality of the Network Layer but rather provide a standard interface to the Transport Layer. In this way the various instances of the Network Layer can be interchangeable and instantiated depending on the situation at hand.

# Source coding

Source coding refers to compression of data with knowledge of the source. The OSI layer related to this is the Presentation Layer.

b) representation of the abstract syntax chosen by the application-entities in the transfer syntax negotiated or renegotiated, including format and special purpose transformations (for example, data compression);

It is proposed that the APIs related to this functionality map to the OSI proposed services. It is also proposed that the APIs not specify the functionality of the Presentation Layer but rather provide a standard interface to the Application Layer. In this way the various instances of the Presentation Layer can be interchangeable and instantiated depending on the situation at hand.

# Channel estimation

Channel estimation is related to error control as its function is to eliminate correlated noise introduced by inter-symbol interference, multi-path interference and other co-channel noises. It is therefore part of the Data Link Layer.

The Data Link Layer detects and possibly corrects errors which may occur in the Physical Layer.

The APIs provided by the Data Link Layer map to the OSI proposed services. This avoids the problem of trying to control the widely differing access control algorithms found at the Data Link Layer via a common API. It is preferable to instantiate the proper Data Link Layer than to try to create a general one and control it via an API.

# Power control

Power control is about meeting the link QOS target with minimal power so as not to disturb other links. This is a Data Link Layer preoccupation.

The Data Link Layer detects and possibly corrects errors which may occur in the Physical Layer.

The Physical Layer API provides the ability to read and set power settings, furthermore the QOS API of the Data Link Layer enables the setting of the objectives. The APIs provided are thus sufficient to enable power control within the Data Link Layer.

Like for the categories, the APIs do not control the power control algorithm of choice, this is left to the instantiation of the proper Data Link Layer.

# Diversity

Diversity is a means by which a Data Link Layer can instantiate multiple links and combine them to achieve a better quality of service.

The Data Link Layer detects and possibly corrects errors which may occur in the Physical Layer.

The Physical Layer API provides the ability to read and set frequency, timing, and spectrum settings, furthermore the QOS API of the Data Link Layer enables the setting of the objectives. The APIs provided are thus sufficient to enable diversity combining within the Data Link Layer.

Like for the categories, the APIs do not control the diversity combining algorithms of choice, this is left to the instantiation of the proper Data Link Layer.

# Conclusions

From this cross reference, it is obvious that natural relationship exist between the waveform characteristics and the OSI model. It is believed that the waveform characteristics proposed in [1] can be used to characterize almost any waveform. Furthermore, as there seems to exist a mapping from those characteristics to the OSI model we are force to the conclusion that the two models are in fact interrelated as originally proposed.

From the discussion of the categories, it is also evident that the role of the proposed APIs is not to control the particular algorithms involved in each layer as there are too many possibilities and permutation but rather as to make similar level layers interchangeable.

Assuming that the designer is armed with a complete set of similar level layers to pick and choose from, then assembling a waveform is only a manner of assembling the proper stack. This works if the communication between the layers is uniform which is what the APIs should do.

# References

- J.Smith, J. Kulp, M. Bicer, T. Demirbilek and J. Anton, “SDR – “Do You Care to Buy the Softest?,” Military Communications Conference, March 25th.

- J. Belzile, “On Protocol Stacks, OSI Layers and SDR”, Submitted to the OMG DSIG SWRADIO working group, March 18th.

- J. Belzile, “Comments on the PIM and PSM for SWRADIO Components”, Submitted to the OMG DSIG SWRADIO working group, Feb 20th.

- ITU-T, “Information Technology – Open systems Interconnection – Basic Reference Model: The basic model”, ITU-T Recommendation X.200

A good example is error control coding where drastically different decoding algorithms (RS, Viterbi, Sequential, CSOC) are present.

A good example is equalizer adaptation. The only difference in many cases is the error function used to adapt the filter (LMS, CMA…).

It is also believed that some efficiency gains can be made by defining “lower level” common interfaces. This activity is known as HAL (Hardware abstraction layer) definition in the SDRF (Software Defined Radio Forum).

Paragraph 7.2.2.2 of [4]

Paragraph 7.5.2.1 of [4]

Paragraph 7.7.2 of [4]

Paragraph 7.6.2.2 of [4]

Paragraph 7.6.2.1 of [4]

Paragraph 7.7.2 of [4]

Paragraph 7.6.2.2 of [4]

Paragraph 7.4.4.1.1 of [4]

Paragraph 7.5.2.1 of [4]

Paragraph 7.2.4 of [4]

Paragraph 7.6.2.2 of [4]

Paragraph 7.6.2.2 of [4]

Paragraph 7.6.2.2 of [4]