Software Communications Architecture Specification

MSRC-5000SCA

V2.1

13 July, 2001

Prepared for the

Joint Tactical Radio System (JTRS) Joint Program Office

Prepared by the

Modular Software-programmable Radio Consortium

under Contract No.  DAAB15-00-3-0001

Revision Summary

| 1.0 | Formal release for initial validation. |
| --- | --- |
| 1.1 | Incorporate approved Change Proposals, numbers 97, 99, 110, 160, 161, 162, 164, 171, 177, 178, 179, 180, 193, 195, 201, 204, 205, 208, 209, 211, 216. |
| 2.0 | Incorporate approved Change Proposals, numbers 39, 105, 119, 147, 175, 186, 191, 192, 210, 217, 218, 219, 220, 222, 223, 225, 226, 227, 229, 231, 232, 235, 237, 240, 243, 249, 255, 258, 266, 270, 275, 276, 277, 278, 282, 283, 285, 299, 307, 308, 310, 311, 332, 335, 336, 337, 341, 342, 343, 344, 345. |
| 2.1 | Incorporate approved Change Proposals, numbers 88, 102, 142, 306, 316, 353, 357, 358, 359, 360, 365, 366, 367, 369, 370, 371, 372, 373, 419, 468, 471, 472, 473, 475, 476, 477 |

Changes from the previous revision, other than editorial corrections, are marked with change bars in the margins.

Change Proposals are controlled by the JTRS Change Control Board.  CPs incorporated into the SCA are considered "closed" and can be seen on the JTRS web site at: www.jtrs.sarda.army.mil/docs/documents/sca_ccb.html.

Table of Contents

Foreword	vii

1	INTRODUCTION	1-1

1.1	Scope.	1-1

1.2	Compliance.	1-2

1.2.1	Joint Technical Architecture Compliance.	1-2

1.3	Document conventions, Terminology, and Definitions.	1-2

1.3.1	Conventions and Terminology.	1-2

1.3.1.1	Unified Modeling Language.	1-2

1.3.1.2	Interface Definition Language.	1-3

1.3.1.3	eXtensible Markup Language.	1-3

1.3.1.4	Color Coding.	1-3

1.3.1.5	Requirements Language.	1-3

1.3.1.6	CF Interface and Operation Identification.	1-3

1.3.2	Definitions.	1-3

1.4	Document Content.	1-4

1.5	Applicable Documents.	1-4

1.5.1	Government Documents.	1-4

1.5.2	Commercial Documents.	1-4

2	Overview	2-1

3	Software Architecture Definition.	3-1

3.1	Operating Environment	3-1

3.1.1	Operating System.	3-1

3.1.2	Middleware & Services.	3-2

3.1.2.1	CORBA.	3-2

3.1.2.2	CORBA Extensions.	3-2

3.1.2.2.1	Naming Service.	3-2

3.1.2.3	Log Service.	3-3

3.1.2.3.1	Use of Log Service.	3-3

3.1.2.3.2	LogServiceModule.	3-3

3.1.2.3.3	Log.	3-5

3.1.3	Core Framework.	3-13

3.1.3.1	Base Application Interfaces.	3-14

3.1.3.1.1	Port.	3-15

3.1.3.1.2	LifeCycle.	3-16

3.1.3.1.3	TestableObject.	3-18

3.1.3.1.4	PortSupplier.	3-19

3.1.3.1.5	PropertySet.	3-20

3.1.3.1.6	Resource.	3-22

3.1.3.1.7	ResourceFactory.	3-23

3.1.3.2	Framework Control Interfaces.	3-26

3.1.3.2.1	Application.	3-26

3.1.3.2.2	ApplicationFactory.	3-31

3.1.3.2.3	DomainManager.	3-37

3.1.3.2.4	Device.	3-49

3.1.3.2.5	LoadableDevice.	3-58

3.1.3.2.6	ExecutableDevice.	3-61

3.1.3.2.7	AggregateDevice.	3-64

3.1.3.2.8	DeviceManager.	3-66

3.1.3.3	Framework Services Interfaces.	3-74

3.1.3.3.1	File.	3-74

3.1.3.3.2	FileSystem.	3-78

3.1.3.3.3	FileManager.	3-83

3.1.3.3.4	Timer.	3-87

3.1.3.4	Domain Profile.	3-87

3.1.3.4.1	Software Package Descriptor.	3-88

3.1.3.4.2	Software Component Descriptor.	3-88

3.1.3.4.3	Software Assembly Descriptor.	3-88

3.1.3.4.4	Properties Descriptor.	3-88

3.1.3.4.5	Device Package Descriptor.	3-89

3.1.3.4.6	Device Configuration Descriptor.	3-89

3.1.3.4.7	Profile Descriptor	3-89

3.1.3.4.8	DomainManger Configuration Descriptor.	3-89

3.1.3.5	Core Framework Base Types.	3-89

3.1.3.5.1	Data Type.	3-89

3.1.3.5.2	DeviceSequence.	3-89

3.1.3.5.3	FileException.	3-89

3.1.3.5.4	InvalidFileName.	3-90

3.1.3.5.5	InvalidObjectReference.	3-90

3.1.3.5.6	InvalidProfile.	3-90

3.1.3.5.7	OctetSequence.	3-90

3.1.3.5.8	Properties.	3-90

3.1.3.5.9	StringSequence.	3-90

3.1.3.5.10	UnknownProperties.	3-90

3.1.3.5.11	DeviceAssignmentType.	3-90

3.1.3.5.12	DeviceAssignmentSequence.	3-90

3.2	Applications.	3-91

3.2.1	General Application Requirements.	3-91

3.2.1.1	OS Services.	3-91

3.2.1.2	CORBA Services.	3-91

3.2.1.3	CF Interfaces.	3-91

3.2.2	Application Interfaces.	3-91

3.2.2.1	Utility Applications.	3-93

3.2.2.1.1	Installer Utility.	3-93

3.2.2.2	Service APIs.	3-95

3.2.2.2.1	Service Definitions.	3-95

3.2.2.2.2	API Transfer Mechanisms.	3-95

3.3	Logical Device.	3-96

3.3.1	OS Services.	3-98

3.3.2	CORBA Services.	3-98

3.3.3	CF Interfaces.	3-98

3.3.4	Profile	3-98

3.4	General Software Rules.	3-98

3.4.1	Software Development Languages.	3-99

3.4.1.1	New Software.	3-99

3.4.1.2	Legacy Software.	3-99

4	Hardware Architecture Definition	4-1

4.1	Basic Approach.	4-1

4.2	Class Structure.	4-1

4.2.1	Top Level Class Structure.	4-2

4.2.2	HWModule(s) Class Structure.	4-3

4.2.3	Class Structure with Extensions.	4-3

4.2.3.1	RF Class Extension.	4-4

4.2.3.2	Modem Class Extension.	4-5

4.2.3.3	Processor Class Extension.	4-6

4.2.3.4	INFOSEC Class.	4-7

4.2.3.5	I/O Class Extension.	4-8

4.2.4	Attribute Compostion.	4-8

4.3	Domain Criteria.	4-9

4.4	Performance Related Issues.	4-9

4.5	General Hardware Rules.	4-9

4.5.1	Device Profile.	4-9

4.5.2	Hardware Critical Interfaces.	4-10

4.5.2.1	Interface Definition.	4-10

4.5.2.2	Interface Standards.	4-10

4.5.2.2.1	Interface Selection.	4-10

4.5.3	Form Factor.	4-10

4.5.4	Modularity.	4-10

5	Security Architecture Definition	5-1

5.1	Additional CF Security Requirements.	5-1

5.1.1	Application.	5-1

5.1.2	ApplicationFactory.	5-1

5.1.3	DomainManager.	5-1

6	Common Services and Deployment Considerations	6-1

6.1	Common System Services.	6-1

6.2	Operational and Deployment Considerations.	6-1

7	Architecture Compliance	7-1

7.1	Certification Authority.	7-1

7.2	Responsibility for Compliance Evaluation.	7-1

7.3	Evaluating Compliance.	7-1

7.4	Registration.	7-1

Appendix A.  Glossary

Appendix B.  SCA APPLICATION ENVironment PROFILE

Appendix C.  Core Framework IDL

Appendix D.  Domain Profile

List of Figures

Figure 1-1.  The Architecture Framework and its Relationship to Implementation	1-2

Figure 1-2.  Color Coding Used in Document Figures	1-3

Figure 2-1.  Software Structure	Error! Bookmark not defined.

Figure 2-2.  Example Message Flows with and without Adapters	Error! Bookmark not defined.

Figure 2-3.  Software Reference Model	Error! Bookmark not defined.

Figure 2-4.  Conceptual Model of Resources	Error! Bookmark not defined.

Figure 2-5.  Example of Modem Resources	Error! Bookmark not defined.

Figure 2-6.  Example of Networking Resources	Error! Bookmark not defined.

Figure 2-7.  Examples of I/O Resources	Error! Bookmark not defined.

Figure 2-8.  Examples of Security Devices and Resources	Error! Bookmark not defined.

Figure 2-9.  Example of Utility Resources	Error! Bookmark not defined.

Figure 2-10.  External Network Protocols and SCA Support	Error! Bookmark not defined.

Figure 2-11.  SCA-Supported Networking Mapped to OSI Network Model	Error! Bookmark not defined.

Figure 2-12.  Hardware Architecture Framework	Error! Bookmark not defined.

Figure 3-1.  Notional Relationship of OE and Application to the SCA AEP	3-2

Figure 3-2.  Log UML.	3-5

Figure 3-3.  Core Framework IDL Relationships	3-14

Figure 3-4.  Port Interface UML	3-15

Figure 3-5.  LifeCycle Interface UML	3-17

Figure 3-6.  TestableObject Interface UML	3-18

Figure 3-7.  PortSupplier Interface UML	3-19

Figure 3-8.  PropertySet Interface UML	3-20

Figure 3-9.  Resource Interface UML	3-22

Figure 3-10.  ResourceFactory Interface UML	3-24

Figure 3-11.  Application Interface UML	3-27

Figure 3-12.  Application Behavior	3-30

Figure 3-13.  ApplicationFactory UML	3-31

Figure 3-14.  ApplicationFactory Behavior	3-36

Figure 3-15.  DomainManager Interface UML	3-37

Figure 3-16.  DomainManager Sequence Diagram for registerDeviceManager Operation	3-41

Figure 3-17.  DomainManager Sequence Diagram for registerDevice Operation	3-43

Figure 3-18. DomainManager Sequence Diagram for registerService Operation	3-48

Figure 3-19.  Device Interface UML	3-50

Figure 3-20.  State Transition Diagram for adminState	3-52

Figure 3-21.  State Transition Diagram for allocateCapacity and deallocateCapacity	3-55

Figure 3-22.  Release Aggregated Device Scenario	3-56

Figure 3-23.  Release Composite Device Scenario	3-57

Figure 3-24.  Release Composite & Aggregated Device Scenario	3-57

Figure 3-25.  Release Composite Device in SHUTTING_DOWN State Scenario	3-58

Figure 3-26.  LoadableDevice Interface UML	3-59

Figure 3-27.  ExecutableDevice Interface UML	3-61

Figure 3-28.  AggregateDevice Interface UML	3-65

Figure 3-29.  DeviceManager UML	3-67

Figure 3-30.  DeviceManager Startup Scenario	3-71

Figure 3-31.  File Interface UML	3-75

Figure 3-32.  FileSystem Interface UML	3-78

Figure 3-33.  FileManager Interface UML	3-84

Figure 3-34.  Relationship of Domain Profile XML File Types	3-88

Figure 3-35.  PushPort Data Interfaces	3-92

Figure 3-36.  PullPort Data Interfaces	3-93

Figure 3-37.  Device Installation Sequence Diagram	3-94

Figure 3-38.  Software Installation Sequence Diagram	3-95

Figure 3-39.  Standard and Alternate Transfer Mechanism	3-96

Figure 3-40.  Device and DeviceManager Interfaces Relationships	3-97

Figure 4-1.  Top Level Hardware Class Structure	4-2

Figure 4-2.  Hardware Module Class Structure	4-3

Figure 4-3.  RF Class Extension	4-5

Figure 4-4.  Modem Class Extension	4-6

Figure 4-5.  Processor Class	4-7

Figure 4-6.  INFOSEC Class	4-7

Figure 4-7.  I/O Class Extension	4-8

Figure 4-8.  Typical Hardware Device Description using the SCA HW Class Structure	4-9

Foreword

Introduction.  The Software Communication Architecture (SCA) specification is published by the Joint Tactical Radio System (JTRS) Joint Program Office (JPO).  This program office was established to pursue the development of future communication systems, capturing the benefits of the technology advances of recent years which are expected to greatly enhance interoperability of communication systems and reduce development and deployment costs.  The goals set for the JTRS program are:

- Greatly increased operational flexibility and interoperability of globally deployed systems,

- Reduced supportability costs,

- Upgradeability in terms of easy technology insertion and capability upgrades, and

- Reduced system acquisition and operation cost.

In order to achieve these goals, the SCA has been structured to

- provide for portability of applications software between different SCA implementations,

- leverage commercial standards to reduce development cost,

- reduce development time of new waveforms through the ability to reuse design modules, and

- build on evolving commercial frameworks and architectures.

The SCA is deliberately designed to meet commercial application requirements as well as military applications.  It is the expectation of the Government that the basic SCA will become a commercially approved standard.  It is for this reason that a wide cross section of industry has been invited to participate in the development and the validation of the SCA.  The SCA is not a system specification, as it is intended to be implementation independent, but a set of rules that constrain the design of systems to achieve the objectives listed above.  The SCA specification version 1.0 established the baseline for architecture validation.  The validation effort demonstrated that multiple vendors could independently design systems, which, when built according to the SCA requirements, meet the program goals outlined above.  Lessons learned during the validation have been incorporated in SCA version 2.0.

The SCA documentation consists of the basic architecture specification, a supplement on military security, a supplement on definition of application program interfaces, and a rationale document.

Software Structure.  The software framework of the SCA defines the Operating Environment (OE) and specifies the services and interfaces that applications use from that environment.  The OE is comprised of:

- a Core Framework (CF),

- a CORBA middleware, and

- a POSIX-based Operating System (OS) with associated board support packages.

The OE imposes design constraints on waveform and other applications to provide increased portability of those applications from one SCA-compliant radio platform to another.  These design constraints include specified interfaces between the Core Framework and application software, and restrictions on waveform usage of the Operating System.

The SCA also provides a building-block structure (defined in the API Supplement) for defining application programming interfaces (APIs) between application software components.  This building-block structure for API definition facilitates component-level reuse and allows significant flexibility for developers to define waveform-specific APIs.

Core Framework.  The CF is an architectural concept defining the essential, “core” set of open software Interfaces and Profiles that provide for the deployment, management, interconnection, and intercommunication of software application components in embedded, distributed-computing communication systems.  All interfaces defined in section 3.1.3 of the SCA Specification are part of the CF.  Some of them are implemented by Core Application Services developers; some are implemented by non-core Applications (i.e.  waveforms, etc.); and some implemented by hardware device providers.  The CF builds an information base from the collection of profiles, known as the Domain Profile and provided with the hardware and software of the system.

Hardware Structure.  The hardware framework also uses OO concepts to define typical hardware partitions within realizable systems.  The primary purpose of the hardware structure is to require complete and comprehensive publication of interfaces and attributes once systems have been built.  With these published specifications, additional venders can provide modules within a system and software developers can identify hardware modules with capabilities required for a particular waveform application.  Hardware modularity also facilitates technology insertion as future programmable elements increase in capability.

Military Applications.  To maximize the commercial application of the SCA and the resulting benefit, military-unique requirements are provided in SCA supplements.  Currently there are two supplements to the SCA Specification:

- a Security Supplement identifies requirements to insure adequate protection of military secure communications and to facilitate certification of JTRS products by the NSA, and

- an API Supplement identifies structures associated with radio system services at various interfaces such as physical, networking, security, and external interfaces.  These APIs, when fully defined, improve portability of applications within JTRS implementations, and make reuse of functional components of those applications easier.  For example, standardizing APIs for a security module within a JTRS enables reuse of common modules for multiple waveform applications.  Standardizing networking APIs improves portability of networking applications and offers easier internetworking functions such as routing, bridging and providing gateways.

Support and Rationale Document (SRD).  This document provides the rationale behind architectural decisions along with further supporting material.

Future Directions.  The JTRS JPO intends to maintain the SCA Specification and Supplements over the next year.  The goal of the JPO is to transition maintenance of the SCA to a commercial open-standards organization.  Changes to the SCA will be incorporated based upon lessons-learned, industry recommendations, and technology improvements.  Changes to the Supplements will similarly incorporate lessons-learned as well as definitions of additional services such as Quality of Service monitoring and Fault Management.

Feedback.  An open architecture framework is greatly improved through active feedback and recommended changes from a wide audience of potential users.  The JTRS JPO solicits and encourages feedback to this document and provides a form available from http://www.jtrs.saalt.army.mil/docs/documents/sca.html.  Send the completed form to jtrs.sca@saalt.army.mil.  Recommended additions to the SCA must be unencumbered by copyright restrictions or intellectual property rights.  Changes to the SCA are controlled by a JTRS JPO-chaired Configuration Control Board (CCB).

# INTRODUCTION

The Software Communications Architecture (SCA) specification establishes an implementation-independent framework with baseline requirements for the development of Joint Tactical Radio System (JTRS) software configurable radios.  These requirements are comprised of interface specifications, application program interfaces (APIs), behavioral specifications, and rules.  The goal of this specification is to ensure the portability and configurability of the software and hardware and to ensure interoperability of products developed using the SCA.

Companion documents to this specification are Supplements to the SCA and the SCA Support and Rationale Document (SRD).  The Supplements provide specific service and application interface requirements (for Security, networking, other services).  The SRD provides the rationale for the SCA and examples to illustrate the implementation of the architecture for differing domains/platforms and selected waveforms.

## Scope.

This document provides a complete definition of the SCA.  It is an Architecture Framework in that it is precise in areas where reusability is effected and it is general in other areas so that unique requirements of implementations determine the specific application of the architecture.  The SCA defines the hardware and software at different levels of detail to allow the broadest reusability and portability of components.

For hardware, the physical and environmental differences across domains are so diverse that physical commonality cannot be achieved for all implementations.  However, by using an Object-Oriented (OO) description for the hardware, represented as hardware classes, all potential system implementations are included within a single framework.  That framework has attributes (i.e., behavior and interfaces) that are applicable across those different implementations.

The architecture for software makes extensive use of object modeling and its definition is primarily in the Core Framework (CF), an integral part of a system's Operating Environment (OE). Constraints on the software development, imposed by the architecture, are on the interfaces and the structure of the software and not on the implementation of the functions that are performed.  In this way, innovative designs can be put forward with appropriate protection of the developer’s intellectual property and still reap the benefits of wide reuse in other implementations of the architecture.  The SCA permits either hardware or software to be used in implementing a required function.  The approach taken also permits legacy solutions to be incorporated, where appropriate, by encapsulation techniques to provide a “one-sided” standard interface into architecture interfaces.

This architecture specifies rules that further constrain implementations to adhere to open system standards.  Specific implementation requirements may augment the rule-set to increase reusability within and across domains.

Figure 1-1 illustrates the concept of the SCA and its implementation down to specific platforms.  The hardware definition stays at a framework level with rules providing implementation guidance down into domains and platforms.  The software definition can be applied directly down to implementation because of its general independence from hardware implementation.  There are special cases where size, weight, and power requirements limit the direct application of software objects.  However, even in these cases, reusability of designs, captured in software and firmware modeling and simulation tools, reduces the cost of implementation and the development time.

Figure 1-1.  The Architecture Framework and its Relationship to Implementation

## Compliance.

The interfaces, behavior, and rules that define compliance with the SCA are identified in, and are an integral part of this specification.  These elements are selected to maximize portability, interoperability, and configurability of the software and hardware while allowing a procurer the flexibility to address domain requirements and restrictions.  If any requirements stated in this specification are in conflict with existing standards/specifications, this specification takes precedence.

### Joint Technical Architecture Compliance.

The Joint Technical Architecture (JTA) mandates the minimum set of standards and guidelines for all DoD Command, Control, Communications, Computers, and Intelligence (C4I) systems acquisition.  A foremost objective of the JTA is to improve and facilitate the ability of systems to support joint and combined operations in an overall investment strategy.  The SCA Operating Environment fully complies with the JTA and provides a JTA-compliant framework for waveforms and other applications.

## Document conventions, Terminology, and Definitions.

### Conventions and Terminology.

#### Unified Modeling Language.

The Unified Modeling Language (UML), defined by the Object Management Group (OMG), is used to graphically represent SCA interfaces, scenarios, use cases, and collaboration diagrams.

#### Interface Definition Language.

Interface Definition Language (IDL), also defined by the OMG, is used to define the SCA interfaces.  IDL is programming language independent and can be compiled into programming languages such as C++, Ada, and Java.

#### eXtensible Markup Language.

eXtensible Markup Language (XML) is used in a Domain Profile to identify the capabilities, properties, inter-dependencies, and location of the hardware devices and software components that make up an SCA-compliant system

#### Color Coding.

Color-coding is used to differentiate between architecture elements and applications in diagrams as shown in Figure 1-2.

Figure 1-2.  Color Coding Used in Document Figures

#### Requirements Language.

Interfaces, behavior, and rules that are imposed by this specification appear in sections 3 through 5 and are indicated by the word "shall".  Editorial notes are contained within brackets and are italicized ({example}).

#### CF Interface and Operation Identification.

CF interfaces and their operations are presented in italicized text.  Core Framework Base Types (3.1.3.5) are prefixed with “CF” when used in textual descriptions (e.g.  "each item value is a CF Properties type").

### Definitions.

Definitions are included in Appendix A.

## Document Content.

This document provides an overview of the SCA in section 2, followed by the Software, Hardware, and Security architecture requirements in sections 3 – 5.  Section 6 addresses requirements not contained in those functional categories.  Evaluation criteria for product compliance to this specification are addressed in section 7.

Appendices include a glossary, a complete listing of CF IDL, and details of architecture requirements introduced in the main document.

## Applicable Documents.

The following documents are applicable to the SCA either by direct reference or as foundation for the architecture definition.

### Government Documents.

Joint Technical Architecture, Version 2.0, 26 May 1998.

Operational Requirements Document (ORD) for Joint Tactical Radio (JTR), 23 March 1998.

### Commercial Documents.

C Standard:  Programming languages – C, ISO/IEC 9899:1990.

DCE UUID standard (OSF Distributed Computing Environment, DCE 1.1 Remote Procedure Call).

“Design Patterns : Elements of Reusable Object-Oriented Software” (Addison-Wesley Professional Computing) Gamma, Helm, Johnson, and Vlissides.

IEEE 802.1 [Medium Access Control (MAC) addresses] IEEE Standards for Local and Metropolitan Area Networks: LAN/MAN Bridging & Management.

ISO/IEC 10731 Conventions for the Definition of OSI Services,  Annex D Alternative and Additional Time Sequence Diagrams for Two-party Communications.

minimumCORBA:  OMG Document orbos/98-05-13, May 19, 1998.

POSIX 1003.13:  Standardized Application Environment Profile - POSIX Realtime Application Support (AEP), IEEE Std 1003.13-1998.

UML:  OMG (Object Management Group) Unified Modeling Language Specification, Version 1.3, March 2000.

XML: W3C (World Wide Web Consortium) Recommendation: Extensible Markup Language (XML) 1.0, Feb 1998.

# Overview

DELETED

# Software Architecture Definition.

## Operating Environment

This section contains the requirements of the operating system, middleware, and the CF interfaces and operations that comprise the OE.

### Operating System.

The processing environment and the functions performed in the architecture impose differing constraints on the architecture.  An SCA application environment profile (AEP) was defined to support portability of waveforms, scalability of the architecture, and commercial viability.  POSIX specifications are used as a basis for this profile.  The notional relationship of the OE and applications to the SCA AEP is depicted in Figure 3-3.  The OS shall[M&S0010] provide the functions and options designated as mandatory by the AEP defined in Appendix B.  The OS is not limited to providing the functions and options designated as mandatory by the profile.  The CORBA Object Request Broker (ORB), the CF Framework Control Interfaces, Framework Services Interfaces, and hardware device drivers are not limited to using the services designated as mandatory by the profile.

Figure 3-3.  Notional Relationship of OE and Application to the SCA AEP

The OS and related file systems shall[M&S0010] support a minimum file name length of 40 characters and a minimum combined pathname/filename length of 1024 characters.

Applications are limited to using the OS services that are designated as mandatory for the profile.  Applications will perform file access through the CF.  (Application requirements are covered in section 3.2.)

### Middleware & Services.

#### CORBA.

The OE shall[M&S0010] use middleware that, at a minimum, provides the services and capabilities of minimumCORBA as specified by the OMG Document orbos/98-05-13, May 19, 1998.

#### CORBA Extensions.

The following extensions and/or services above and beyond minimumCORBA are allowed.

##### Naming Service.

CORBA Naming Service may be used.  If a CORBA Naming Service is used, the OE should provide an Interoperable Naming Service as specified by the OMG Document orbos/98-10-11, October 19, 1998.

As an alternative, software components will include stringified Interoperable Object References (IORs) in their Software Profile.  If stringified IORs are used, the object reference shall[M&S0010] be generated using the persistent LifeSpan policy per the minimumCORBA specification.

#### Log Service.

##### Use of Log Service.

This section describes the requirements for components that produce log records.  A log producer is a CF component (e.g., DomainManger, Application, ApplicationFactory, DeviceManager, Device) or an application’s CORBA capable component (e.g., Resource, ResourceFactory) that produces log records.  (A component that calls the writeRecords operation of the Log interface.)

A standard record type is defined for all log producers to use when writing log records.  The log producer may be configured via the PropertySet interface to output only specific log levels.

Log producers shall[M&S0010] implement a configure property with an ID of “PRODUCER_LOG_LEVEL”.  The PRODUCER_LOG_LEVEL configure property provides the ability to “filter” the log message output of a log producer.  The type of this property shall[M&S0010] be a LogLevelSequence.  The configure property LogLevelSequence contains all log levels that shall[M&S0010] be enabled.  Only the messages that contain an enabled log level may be sent by a log producer to a Log.  Log levels which are not in the LogLevelSequence shall[M&S0010] be disabled.

Log producers shall[M&S0010] implement a property with an ID of “PRODUCER_LOG_ID”.  The PRODUCER_LOG_ID configure property provides an instance unique identifier for the log producer.  The log producer shall[M&S0010] use the string value of the PRODUCER_LOG_ID property in the ProducerID field of the ProducerLogRecord.

Log producers shall[M&S0010] operate normally in the case where the connections to a Log are nil or an invalid reference.

Log producers shall[M&S0010] output only those log records that correspond to enabled LogLevelType values.

##### LogServiceModule.

The LogService module contains the Log servant interface and the types necessary for a log producer to generate standard SCA log records.  This module also defines the types necessary to control the logging output of a log producer.  Components that produce logs are required to implement configure properties that allow the component to be configured as to what log records it will output.

Implementation of the LogService is optional (i.e.  a log service is not a required component of a JTRS installation).  CF components that are required to write log records are also required to account for the absence of a log service and otherwise operate normally.

###### Types.

            - LogLevelType.

Type LogLevelType is an enumeration that is utilized to identify log levels.

enum LogLevelType {SECURITY_ALARM

, FAILURE_ALARM

, DEGRADED_ALARM

, EXCEPTION_ERROR

, FLOW_CONTROL_ERROR

, RANGE_ERROR

, USAGE_ERROR

, ADMINISTRATIVE_EVENT

, STATISTIC_REPORT

, PROGRAMMER_DEBUG1

, PROGRAMMER_DEBUG2

, PROGRAMMER_DEBUG3

, PROGRAMMER_DEBUG4

, PROGRAMMER_DEBUG5

, PROGRAMMER_DEBUG6

, PROGRAMMER_DEBUG7

, PROGRAMMER_DEBUG8

, PROGRAMMER_DEBUG9

, PROGRAMMER_DEBUG10

, PROGRAMMER_DEBUG11

, PROGRAMMER_DEBUG12

, PROGRAMMER_DEBUG13

, PROGRAMMER_DEBUG14

, PROGRAMMER_DEBUG15

, PROGRAMMER_DEBUG16

}

            - ProducerLogRecordType.

Log producers format log records as defined in the structure ProducerLogRecordType.

struct ProducerLogRecordType {

unsigned long producerID;

string producerName;

LogLevelType level;

String logData;

}

producerID : This field uniquely identifies the source of a log record.  The value is unique within the Domain.  The DomainManager and ApplicationFactory are responsible for assigning this value.

producerName : This field identifies the producer of a log record in textual format.  This field is assigned by the log producer, thus is not unique within the Domain (e.g. multiple instances of an application will assign the same name to the ProducerName field.)

level : This field identifies the type of message being logged as defined by the type LogLevelType.

logData : This field contains the informational message being logged.

            - LogLevelSequence.

The LogLevelSequence type is an unbounded sequence of LogLevelTypes.  The PRODUCER_LOG_LEVEL configure/query property is of the LogLevelSequence type.

typedef sequence <LogLevelType> LogLevelSequence;

##### Log.

###### Description.

A Log is utilized by CF and CORBA capable application components to store informational messages.  These informational messages are referred to as ‘log records’ in this document. The interface provides operations for writing log records to a Log, retrieving log records from a Log, control of a Log, and status of a Log.

###### UML.

Figure 3-4.  Log UML.

###### Types.

            - InvalidParamException.

The InvalidParamException exception indicates that a provided parameter was invalid.

exception InvalidParam {string details};

            - LogEmptyException.

The LogEmptyException exception indicates that the log is empty.

exception LogEmpty ();

            - LogTimeType.

This type provides the time format used when writing log records.  The Log implementation is required to produce time-stamps compatible with the POSIX-defined struct tm.

typedef unsigned long long LogTimeType;

            - OperationalStateType.

The enumeration OperationalStateType defines the Log states of operation.  When the Log is ENABLED it is fully functional and is available for use by log producer and log consumer clients.  A Log that is DISABLED has encountered a runtime problem and is not available for use by log producers or log consumers.  The internal error conditions that cause the Log to set the operational state to ENABLED or DISABLED are implementation specific.

enum OperationalStateType {DISABLED, ENABLED};

            - AdministrativeStateType.

The AdministrativeStateType denotes the active logging state of an operational Log.  When set to UNLOCKED the Log will accept records for storage, per its operational parameters.  When set to LOCKED the Log will not accept new log records and records can be read or deleted only.

enum AdministrativeStateType {LOCKED, UNLOCKED};

            - AvailabilityStatusType.

AvailabilityStatusType denotes whether or not the Log is available for use.  When true, offDuty indicates the Log is LOCKED (administrative state) or DISABLED (operational state).  When true, logFull indicates the Log storage is full.

struct AvailabilityStatusType{

boolean offDuty;

boolean logFull;

};

            - LogFullActionType.

This type specifies the action that the Log should take when its internal buffers become full of data, leaving no room for new records to be written.  Wrap indicates that the Log will overwrite the oldest log records with the newest records, as they are written to the Log.  Halt indicates that the Log will stop logging when full.

enum LogFullActionType (WRAP, HALT);

            - RecordIDType.

This type provides the record ID that is assigned to a log record.

typedef unsigned long long RecordIDType;

            - LogRecordType.

The LogRecordType defines the format of the log records as stored in the Log. The ‘info’ field is the ProducerLogRecord that is written by a client to the Log.

struct LogRecordType {

RecordIDType id;

LogTimeType time;

ProducerLogRecordType info;

};

            - LogRecordSequence.

The LogRecordSequence type defines an unbounded sequence of log records.

typedef sequence<LogRecordType> LogRecordSequence;

            - ProducerLogRecordSequence Type.

The ProducerLogRecordSequence type defines a sequence of ProducerLogRecordTypes.

typedef sequence <ProducerLogRecordType> ProducerLogRecordSequence

###### Attributes.

N/A.

###### Operations.

            - getMaxSize.

              - Brief Rationale.

This operation sets the maximum number of bytes that the Log can store.

              - Synopsis.

unsigned long long getMaxSize();

              - Behavior.

The getMaxSize operation returns the maximum size of the Log measured in number of bytes.

              - Returns.

The getMaxSize operation shall[M&S0010] return the integer number of bytes that the Log is capable of storing.

              - Exceptions/Errors.

This operation does not raise any exceptions.

            - setMaxSize.

              - Brief Rationale.

This operation sets the maximum number of bytes that the Log can store.

              - Synopsis.

void setMaxSize(unsigned long long size) raises (InvalidParam);

              - Behavior.

The setMaxSize operation shall[M&S0010] set the maximum size of the log measured in number of bytes.

              - Returns.

This operation does not return a value.

              - Exceptions/Errors.

The setMaxSize operation shall[M&S0010] raise the InvalidParam exception if the size parameter passed in is less than the current size of the Log.

The setMaxSize operation shall[M&S0010] raise the InvalidParam exception if the input size parameter is greater than the storage space available to the Log.

            - getCurrentSize.

              - Brief Rationale.

The getCurrentSize operation provides the current size of the log storage in bytes.

              - Synopsis.

unsigned long long getCurrentSize ();

              - Behavior.

The getCurrentSize operation returns the current size of the log storage in bytes.

              - Returns.

The getCurrentSize operation shall[M&S0010] return the current size of the log storage in bytes. (i.e.  if the log contains no records, getCurrentSize will return a value of 0 (zero).)

              - Exceptions/Errors.

This operation does not return any exceptions.

            - getNumRecords.

              - Brief Rationale.

The getNumRecords operation provides the number of records present in the Log.

              - Synopsis.

unsigned long long getNumRecords ();

              - Behavior.

The getNumRecords operation returns the current number of records contained in the Log.

              - Returns.

The getNumRecords operation shall[M&S0010] return the current number of log records contained in the Log.

              - Exceptions/Errors.

This operation does not raise any exceptions.

            - getLogFullAction.

              - Brief Rationale.

The getLogFullAction operation provides the action taken when the Log becomes full.

              - Synopsis.

LogFullActionType getLogFullAction();

              - Behavior.

The getLogFullAction operation returns the action that will be taken when the maximum size of the Log has been reached.

              - Returns.

The getLogFullAction operation shall[M&S0010] return the Log’s log full action setting.

              - Exceptions/Errors.

This operation does not return any exceptions.

            - setLogFullAction.

              - Brief Rationale.

The setLogFullAction operation is used to configure the Log to either WRAP or HALT when the Log becomes full.

              - Synopsis.

void setLogFullAction(in LogFullActionType action)

              - Behavior.

The setLogFullAction operation shall[M&S0010] set the action taken when the maximum size of the Log has been reached.

              - Returns.

This operation does not return a value.

              - Exceptions/Errors.

This operation does not return any exceptions.

            - getAvailabilityStatus.

              - Brief Rationale.

The getAvailabilityStatus operation is used to read the availability status of the Log.

              - Synopsis.

AvailabilityStatusType getAvailabilityStatus ();

              - Behavior.

The getAvailabilityStatus operation returns a structure that reflects the availability status of the Log.  See the description of the AvailabilityStatusType in 3.1.2.3.3.3.6.

              - Returns.

The getAvailabilityStatus operation shall[M&S0010] return the current availability status of the Log.

              - Exceptions/Errors.

This operation does not raise any exceptions.

            - getAdministrativeState.

              - Brief Rationale.

The getAdministrativeState is used to read the administrative state of the Log.

              - Synopsis.

AdministrativeStateType getAdministrativeState();

              - Behavior.

The getAdministrativeState operation returns the administrative state of the Log.  See the description of the AdministrativeStateType in 3.1.2.3.3.3.5.

              - Returns.

The getAdministrativeState operation shall[M&S0010] return the current administrative state of the Log.

              - Exceptions/Errors.

This operation does not raise any exceptions.

            - setAdministrativeState.

              - Brief Rationale.

The setAdministrativeState operation provides write access to the administrative state value.

              - Synopsis.

void setAdministrativeState(in AdministrativeStateType state);

              - Behavior.

The setAdministrativeState operation shall[M&S0010] set the administrative state of the Log.

              - Returns.

This operation does not return a value.

              - Exceptions/Errors.

This operation does not raise any exceptions.

            - getOperationalState.

              - Brief Rationale.

The getOperationalState operation returns the operational state of the Log.

              - Synopsis.

OperationalStateType getOperationalState();

              - Behavior.

The getOperationalState operation returns the operational state of the Log.  See the description of OperationalStateType in 3.1.2.3.3.3.4.

              - Returns.

The getOperationalState operation shall[M&S0010] return the current operational state of the Log.

              - Exceptions/Errors.

This operation does not raise any exceptions.

            - writeRecords.

              - Brief Rationale.

The writeRecords operation provides the method for writing log records to the Log. The operation is defined as oneway to minimize client overhead while writing to the Log.

              - Synopsis.

oneway void writeRecords(in ProducerLogRecordSequence records);

              - Behavior.

The writeRecords operation shall[M&S0010] add each log record supplied in the records parameter to the Log.  When there is insufficient storage to add one of the supplied log records to the Log, and theLogFullAction is set to HALT, the writeRecords method shall[M&S0010] set the availability status logFull state to true (e.g.  if 3 records are provided in the records parameter, and while trying to write the second record to the log, the record will not fit, then the log is considered to be full therefore the second and third records will not be stored in the log but the first record would have been successfully stored.).

The writeRecords operation shall[M&S0010] write the current time to the time field of the LogRecord in the format defined by the standard POSIX type struct tm.

The writeRecords operation shall[M&S0010] assign a unique record ID to the id field of the LogRecord.

Log records accepted for storage by the writeRecords shall[M&S0010] be available for retrieval in the order received.

              - Returns.

This operation does not return a value.

              - Exceptions/Errors.

This operation does not raise any exceptions.

            - getRecordIdFromTime.

              - Brief Rationale.

The getRecordIdFromTime operation is used to get the record ID of the first record in the Log with a time-stamp that is greater than or equal to the time specified in the parameter.

              - Synopsis.

RecordIDType getRecordIdFromTime (in LogTimeType fromTime);

raises (LogEmpty)

              - Behavior.

The getRecordIdFromTime operation returns the record ID of the first record in the log with a time-stamp that is greater than, or equal to, the time specified in the fromTime parameter.  If the Log is empty, the LogEmpty exception is raised, and the returned record ID is invalid.

              - Returns.

The getRecordIdFromTime operation shall[M&S0010] return the record ID of the first record in the log with a time-stamp that is greater than, or equal to, the time specified in the fromTime parameter. The getRecordIdFromTime operation shall[M&S0010] return zero if no record exists in the log with a time-stampthat is greater than or equal to the time specified in the fromTime parameter.

              - Exceptions/Errors.

When the Log is empty, the getRecordIdFromTime operation shall[M&S0010] raise the LogEmpty exception.

            - retrieveById.

              - Brief Rationale.

The retrieveById operation is used to get a specified number of records from a Log.

              - Synopsis.

LogRecordSequence retrieveById (inout RecordIDType currentId, in unsigned long howMany);

              - Behavior.

The retrieveById operation returns a list of log records that begins with the record specified bythe currentID parameter and contains less than or equal to the number of records specified in the howMany parameter.

The retrieveById operation shall[M&S0010] set the inout parameter currentId to the LogRecord Id of next record after the last record in the log record sequence returned.

              - Returns.

The retrieveById operation shall[M&S0010] return a log record sequence that begins with the record specified by the currentId parameter.  The number of records in the log record sequence returned by the retrieveById operation shall[M&S0010] be equal to the number of records specified by the howMany parameter, or the number of records available if the number of records specified by the howMany parameter cannot be met.  If the record specified by currentId does not exist, the retrieveById operation shall[M&S0010] return an empty list of log records.  If the Log is empty, the retrieveById operation shall[M&S0010] return an empty list of log records.

              - Exceptions/Errors.

This operation does not raise any exceptions.

            - clearLog.

              - Brief Rationale.

The clearLog operation provides the method for removing all of the log records from the Log.

              - Synopsis.

void clearLog ();

              - Behavior.

The clearLog operation shall[M&S0010] delete all records from the Log.  The clearLog operation shall[M&S0010] set the current size of the Log storage to zero.  The clearLog operation shall[M&S0010] set the current number of records in the Log to zero.  The clearLog operation shall[M&S0010] set the logFull availability status element to false.

              - Returns.

This operation does not return a value.

              - Exceptions/Errors.

This operation does not raise any exceptions.

            - destroy.

              - Brief Rationale.

The destroy operation provides a means by which an instantiated Log may be torn down.

              - Synopsis.

void destroy ();

              - Behavior.

The destroy operation shall[M&S0010] release all internal memory and/or storage allocated by the Log.  The destroy operation shall[M&S0010] tear down the component (i.e.  released from the CORBA environment).

              - Returns.

This operation does not return a value.

              - Exceptions/Errors.

This operation does not raise any exception.

### Core Framework.

The CF specification includes a detailed description of the purpose of each interface, the purpose of each supported operation within the interface, and interface class diagrams to support these descriptions.  The corresponding IDL for the CF can be found in Appendix C.

Figure 3-5 depicts the key elements of the CF and the IDL relationships between these elements. A DomainManager component manages the software Applications, ApplicationFactories, hardware devices (represented by software Devices) and DeviceManagers within the system.  An Application is a type of Resource and consists of one to many software Resources. Some of the software Resources may directly control the system’s internal hardware devices; these Resources are logical Devices which implement the Device, LoadableDevice, or ExecutableDevice interfaces. (For example, a ModemDevice may provide direct control of a modem hardware device such as a Field Programmable Gate Array (FPGA) or an Application Specific Integrated Circuit (ASIC).  An I/ODevice may operate as a device driver to provide external access to the system.)  Other software Resources have no direct relationship with a hardware device, but perform application services for the user.  (For example, a NetworkResource may perform a network layer function.  A WaveformLinkResource may perform a waveform specific link layer service.)  Each Resource can potentially communicate with other Resources.  Devices are allocated to one or more hardware devices by the DomainManager based upon various factors including the hardware devices that the DeviceManager knows about, the current availability of hardware devices, the behavior rules of a Resource, and the loading requirements of the Resource.

Figure 3-5.  Core Framework IDL Relationships

The Resources being managed by the DomainManager are CORBA objects implementing the Resource interface.  Some Resources may be dependent on other Resources. This interface provides a consistent way of creating up and tearing down any Resource within the system. These resources can be created by using a ResourceFactory interface or by the Device interfaces (Device, LoadableDevice, or ExecutableDevice).

The file service interfaces (FileManager, FileSystem, and File) are used for installation and removal of application files within the system, and for loading and unloading application files on the various processors that the Devices execute upon.

#### Base Application Interfaces.

Base Application Interfaces are defined by the Core Framework requirements and implemented by application developers; see 3.2 for Application requirements.

##### Port.

This interface provides operations for managing associations between ports.  The Port interface UML is depicted in Figure 3-6.  An application defines a specific Port type by specifying an interface that inherits the Port interface.  An application establishes the operations for transferring data and control.  The application also establishes the meaning of the data and control values.  Examples of how applications may use ports in different ways include: push or pull, synchronous or asynchronous, mono- or bi-directional, or whether to use flow control (e.g., pause, start, stop).

The nature of Port fan-in, fan-out, or one-to-one is component dependent.

Note 1: The CORBA specification defines only a minimum size for each basic IDL type.  The actual size of the data type is dependent on the language (defined in the language mappings) as well as the Central Processing Unit (CPU) architecture used.  By using these CORBA basic data types, portability is maintained between components implemented in differing CPU architectures and languages.

Note 2: How components' ports are connected is described in the software assembly descriptor (SAD) file of the Domain Profile (Error: Reference source not found).

UML

Figure 3-6.  Port Interface UML

InvalidPort Type

The InvalidPort exception indicates one of the following errors has occurred in the specification of a Port association:

- errorCode 1 means the Port component is invalid (unable to narrow object reference) or illegal object reference,

- errorCode 2 means the Port name is not found (not used by this Port).

exception InvalidPort { unsigned short errorCode, string msg };

OccupiedPort Exception

The OccupiedPort exception indicates the Port is unable to accept any additional connections.

exception OccupiedPort {};

###### connectPort

void connectPort(in Object connection, in string connectionID) raises (InvalidPort, OccupiedPort);

Applications require connectPort to establish associations between Ports.  Ports provide channels through which data and/or control pass.

connectPort provides half of a two-way association; therefore two calls are required to create a two-way association.

Behavior

connectPort shall[RES0010] make a connection to the component identified by connection and connectionID.

A port may support several connections. connectionID is a unique identifier to be used by disconnectPort when breaking this specific connection.

This operation does not return a value.

Exceptions/Errors

connectPort shall[RES0020] raise the InvalidPort exception when the connection is an invalid connection for this Port.

connectPort shall[RES0030] raise the OccupiedPort exception when unable to accept the connections because the Port is already fully occupied.

###### disconnectPort.

void disconnectPort (in string connectionID) raises (InvalidPort);

Applications require disconnectPort in order to allow consumer/producer data components to disassociate themselves from their counterparts (consumer/producer).

Behavior.

disconnectPort shall[RES0040] break the connection to the component identified by connectionID.

This operation does not return a value.

Exceptions/Errors.

disconnectPort shall[RES0050] raise the InvalidPort exception when the name passed to disconnectPort is not connected with the Port component.

##### LifeCycle.

The LifeCycle interface defines the generic operations for initializing or releasing instantiated component-specific data and/or processing elements.  The LifeCycle interface UML is depicted in Figure 3-7.

UML.

Figure 3-7.  LifeCycle Interface UML

InitializeError Exception

The InitializeError exception indicates an error occurred during component initialization.  The message is component-dependent, providing additional information describing the reason why the error occurred.

exception InitializeError { StringSequence errorMessage; };

ReleaseError Exception

The ReleaseError exception indicates an error occurred during component releaseObject. The message is component-dependent, providing additional information describing the reason why the error occurred.

exception ReleaseError { StringSequence errorMessage; };

###### initialize

void initialize() raises (InitializeError);

The purpose of initialize is to provide a mechanism to set a component to a known initial state.  (For example, data structures may be set to initial values, memory may be allocated, hardware devices may be configured to some state, etc.)

Behavior

Initialization behavior is implementation dependent.

This operation does not return a value.

Exceptions/Errors

initialize shall[RES0060] raise an InitializeError exception when an initialization error occurs.

###### releaseObject.

void releaseObject() raises (ReleaseError);

The purpose of releaseObject is to provide a means by which an instantiated component may be torn down.

Behavior

releaseObject shall[RES0070] release all internal memory allocated by the component during the life of the component. releaseObject shall[RES0080] tear down the component (i.e. released from the CORBA environment). releaseObject shall[RES0090] release components from the OE.

This operation does not return a value.

Exceptions/Errors.

releaseObject shall[RES0100] raise a ReleaseError exception when a release error occurs.

##### TestableObject.

The TestableObject interface defines a set of operations that can be used to test component implementations. The TestableObject interface UML is depicted in Figure 3-8.

UML

Figure 3-8.  TestableObject Interface UML

UnknownTest Exception

The UnknownTest exception indicates the requested testid for a test to be performed is not known by the component.

exception UnknownTest {};

###### runTest.

void runTest(in unsigned long testid, inout Properties testValues)raises (UnknownTest, UnknownProperties);

runTest allows components to be “blackbox” tested.  This allows Built-In Test (BIT) to be implemented and this provides a means to isolate faults (both software and hardware) within the system.

Behavior

runTest shall[RES0110] use testid to determine which of its predefined test implementations should be performed. testValues CF Properties (id/value pair(s)) shall[RES0120] be used to provide additional information to the implementation-specific test to be run. runTest shall[RES0130] return the result(s) of the test in testValues.

Tests to be implemented by a component are component-dependent and are specified in the component’s Properties Descriptor.  Valid testid(s) and both input and ouput testValues (properties) for runTest shall[RES0140] at a minimum be test properties defined in the properties test element of the component's Properties Descriptor (refer to Appendix D Domain Profile). testid corresponds to the XML attribute testid of the property element test in a propertyfile.

A CF UnknownProperties exception is raised by runTest all inputValues properties shall[RES0150] be validated (i.e., test properties defined in the propertyfile(s) referenced in the component’s SPD).

runTest shall[RES0160] not execute any testing when the input testid or any of the input testValues are not known by the component or are out of range.

This operation does not return a value.

Exceptions/Errors

runTest shall[RES0170] raise the UnknownTest exception when there is no underlying test implementation that is associated with the input testid given.

runTest shall[RES0180] raise the CF UnknownProperties exception when testValues contains any CF DataTypes that are not known by the component’s test implementation or any values that are out of range for the requested test. invalidProperties shall[RES0190] contain the invalid inputValues properties id(s) that are not known by the component or the value(s) are out of range.

##### PortSupplier.

This interface provides the getPort operation for those components that provide ports.

UML.

Figure 3-9.  PortSupplier Interface UML

UnknownPort Exception

The UnknownPort exception is raised if an undefined port is requested.

exception UnknownPort { };

###### getPort.

Object getPort(in string name) raises ( UnknownPort );

getPort provides a mechanism to obtain a specific consumer or producer Port. A PortSupplier may contain zero-to-many consumer and producer port components.  The exact number is specified in the component’s Software Profile SCD (section Error: Reference source not found).  These Ports can be either push or pull types.  Multiple input and/or output ports provide flexibility for PortSuppliers that must manage varying priority levels and categories of incoming and outgoing messages, provide multi-threaded message handling, or other special message processing.

Behavior

getPort returns the object reference to the named port as stated in the component's SCD.

getPort shall[RES0200] return the CORBA object reference that is associated with the input port name.

Exceptions/Errors

getPort shall[RES0210] raise an UnknownPort exception if the port name is invalid.

##### PropertySet.

The PropertySet interface defines configure and query operations to access component properties/attributes.  The PropertySet interface UML is depicted in Figure 3-10.

UML.

Figure 3-10.  PropertySet Interface UML

InvalidConfiguration Exception

The InvalidConfiguration indicates the configuration of a component has failed (no configuration at all was done).  The message is component-dependent, providing additional information describing the reason why the error occurred..  The invalidProperties returned indicate the properties that were invalid.

exception InvalidConfiguration { string msg; Properties invalidProperties};

PartialConfiguration

The PartialConfiguration exception indicates the configuration of a Component was partiallysuccessful. The invalidProperties returned indicate the properties that were invalid.

exception PartialConfiguration { Properties invalidProperties};

###### configure.

void configure(in Properties configProperties) raises (InvalidConfiguration, PartialConfiguration);

configure allows id/value pair configuration properties to be assigned to components implementing this interface.

Behavior

configure shall[RES0220] assign values to the properties as indicated in the configProperties argument.  Valid properties for configure shall[RES0230] at a minimum be the configure readwrite and writeonly properties referenced in the component’s SPD.

This operation does not return a value.

Exceptions/Errors

configure shall[RES0240] raise a PartialConfiguration exception when some configuration properties were successfully set and some configuration properties were not successfully set.

configure shall[RES0250] raise an InvalidConfiguration exception when a configuration error occurs that prevents any property configuration on the component.

###### query.

void query(inout Properties configProperties) raises (UnknownProperties);

query allows a component to be queried to retrieve its properties.

Behavior

If the configProperties are zero size then, query shall[RES0260] return all component properties.  If the configProperties are not zero size, then query shall[RES0270] return only those id/value pairs specified in the configProperties.  Valid properties for query shall[RES0280] at a minimum be the configure readwrite and readonly properties, and allocation properties that have an action value of “external” as referenced in the component’s SPD.

This operation does not return a value.

Exceptions/Errors

query shall[RES0290] raise the CF UnknownProperties exception when one or more properties being requested are not known by the component.

##### Resource.

The Resource interface provides a common API for the control and configuration of a software component.  The Resource interface UML is depicted in Figure 3-11.

The Resource interface inherits from the LifeCycle, PropertySet, TestableObject, and PortSupplier interfaces.

The inherited LifeCycle, PropertySet, TestableObject, and PortSupplier interface operations are documented in their respective sections of this document.

The Resource interface may also be inherited by other application interfaces as described in the Software Profile's Software Component Descriptor (SCD) file (see Error: Reference source not found).

UML

Figure 3-11.  Resource Interface UML

UnknownPort Exception

The UnknownPort exception is raised if an undefined port is requested.

exception UnknownPort { };

StartError Exception

The StartError exception indicates that an error occurred during an attempt to start the Resource.  The message is component-dependent, providing additional information describing the reason for the error and the severity of the error.

exception StartError { string msg };

StopError Exception

The StopError exception indicates that an error occurred during an attempt to stop the Resource.  The message is component-dependent, providing additional information describing the reason for the error and the severity of the error.

exception StopError { string msg };

###### stop

void stop()raises (StopError);

stop is provided to command a Resource implementing this interface to stop internal processing.

Behavior

stop shall[RES0300] disable all current operations and put the Resource in a non-operating condition. Subsequent configure, query, and start operations are not inhibited by stop.

This operation does not return a value.

stop shall[RES0310] raise the StopError exception if an error occurs while stopping the resource.

###### start

void start()raises (StartError);

start is provided to command a Resource implementing this interface to start internal processing.

Behavior

start puts the Resource in an operating condition.

This operation does not return a value.

start shall[RES0320] raise the StartError exception if an error occurs while starting the resource.

###### getPort.

Object getPort(in string name) raises ( UnknownPort );

getPort provides a mechanism to obtain a specific consumer or producer Port.  A Resource may contain zero-to-many consumer and producer port components.  The exact number is specified in the component’s Software Profile SCD (section Error: Reference source not found).  These Ports can be either push or pull types.  Multiple input and/or output ports provide flexibility for Applications and Resources that must manage varying priority levels and categories of incoming and outgoing messages, provide multi-threaded message handling, or other special message processing.

Behavior.

getPort shall[RES0330] return the object reference to the named port as stated in the Resource's SCD.

getPort returns the CORBA object reference that is associated with the input port name.

getPort shall[RES0340] raise an UnknownPort exception if the port name is invalid.

##### ResourceFactory.

A ResourceFactory is used to create and tear down a Resource.  The ResourceFactory interface is designed after the Factory Design Patterns.  The ResourceFactory interface UML is depicted in Figure 3-12.  The factory mechanism provides client-server isolation among Resources (e.g., Network, Link, Modem, I/O, etc.) and provides an industry standard mechanism of obtaining a Resource without knowing its identity.  An application is not required to use ResourceFactories to obtain, create, or tear down resources.  A Software Profile will determine which application ResourceFactories are to be used by the ApplicationFactory.

UML

Figure 3-12.  ResourceFactory Interface UML

ResourceNumType

This type defines the identity of a Resource created by the ResourceFactory.

typedef unsigned short ResourceNumType.

InvalidResourceNumber Exception

The InvalidResourceNumber exception indicates the resourceNumber does not exist in the Factory.

exception InvalidResourceNumber { };

ShutdownFailure Exception

The ShutdownFailure exception indicates that the shutdown method failed to release the ResourceFactory from the CORBA environment due to the fact the Factory still contains Resources.  The message is component-dependent, providing additional information describing why the shutdown failed.

exception ShutdownFailure { string msg };

###### createResource.

Resource createResource (in ResourceNumType resourceNumber, in Properties qualifiers);

createResource provides the capability to create Resources in the same process space as the ResourceFactory or to return a Resource that has already been created. This behavior is an alternative approach for creating a Resource to the Device execute operations.

The resourceNumber is the identifier for Resource. qualifiers are values used by the ResourceFactory in creation of the Resource.  The ApplicationFactory can determine the values to be supplied for the qualifiers from the description in the ResourceFactory’s Software Profile.  The qualifiers may be used to identify, for example, specific subtypes of Resources created by a ResourceFactory.

              - Behavior.

If no Resource exists for the given resourceNumber, createResource shall[RES0350] create a Resource.  If the Resource already exists, that Resource is returned.  createResource shall[RES0360] assign the given resourceNumber to a new Resource and set a reference count to one or, in the case that the Resource already exists, increment the count by one. The reference count is used to indicate the number of times that a specific Resource reference has been given to requesting clients.  This ensures that the ResourceFactory does not release a Resource that has a reference count greater than 0.  (Multiple clients could request the release of the Resource after obtaining a reference to the Resource).

createResource shall[RES0370] return a reference to the created Resource or the existing Resource. createResource shall[RES0380] return a nil CORBA component reference when the operation is unable to create or find the Resource.

###### releaseResource.

void releaseResource(in ResourceNumType resourceNumber) raises {InvalidResourceNumber);

In CORBA there is client side and server side representation of a Resource. releaseResource provides the mechanism of releasing the Resource in the CORBA environment on the server side when all clients are through with a specific Resource.  The client still has to release its client side reference of the Resource.

Behavior

releaseResource shall[RES0390] decrement the reference count for the specified resource, as indicated by the resourceNumber. releaseResource shall[RES0400] make the Resource nolonger avaliable (i.e.  it is released from the CORBA environment) when the Resource’s referencecount is zero.

This operation does not return a value.

releaseResource shall[RES0410] raise the InvalidResourceNumber exception if an invalid resourceNumber is received.

###### shutdown.

void shutdown()raises {ShutdownFailure);

In CORBA there is client side and server side representation of a ResourceFactory. shutdown provides the mechanism for releasing the ResourceFactory from the CORBA environment on the server side.  The client has the responsibility to release its client side reference of the ResourceFactory.

Behavior

shutdown shall[RES0420] result in the ResourceFactory being unavailable to any subsequent calls to its object reference (i.e. it is released from the CORBA environment).

This operation does not return a value.

 POSIX is a registered trademark of the Institute of Electrical and Electronics Engineers, Inc.