Object Management Group

First Needham Place

250 First Avenue, Suite 201

Needham, MA 02494

Telephone: +1-781-444-0404

Facsimile: +1-781-444-0320

High Assurance ORB
Request For Proposal

OMG Document: realtime/2003-06-01

Letters of Intent due: March 22, 2004

Initial Submissions due: April , 2004

Objective of this RFP

Safety Critical embedded systems with real-time, low-footprint and performance-conservative constraints present a hurdle that inhibits the use of the CORBA Object Request Broker (ORB).  The current implementations of specifications for ORBs address Enterprise or Real-Time systems and are not appropriate for analyzable, high assurance systems because most high assurance systems are typically constrained to analyzable functionality by RTCA/DO-178B or other comparable guidance documents.  A high assurance CORBA ORB implementation must be shown to take into consideration the typical industry guidance for development, analysis and testing.

This RFP solicits proposals for the following:

•	A high assurance ORB that can be analyzed for compliance with specific functional requirements. Requirements must be functionally traceable to the high assurance ORB implementation.

•	An interoperability plan, describing how interoperability is maintained between “high assurance” and other ORBs.

For further details see Chapter 6 of this document.

# Introduction

# Architectural Context

# Adoption Process

# Instructions for Submitters

# General Requirements on Proposals

# Specific Requirements on Proposals

## Problem Statement

Safety-critical systems: This RFP addresses requirements for embedded real-time systems that have safety critical constraints. Development requirements are primarily dictated by RTCA DO-178B, EUROCAE ED-12B, RTCA DO-255, and IEC 61508 relating to several needs:

- Development process and documentation

- Verification and Validation

- Formal Analysis and Modeling

- Restricted language constructs.

Many of today’s high assurance applications are unable to use CORBA ORBs available in the commercial marketplace due to design constraints imposed on them by their operational environments.  These constraints include memory limitations and processing overhead limitations in performance conservative applications.  These high assurance environments suggest ORB functionality should be tailored to create a reduced functionality that is “analyzable” with respect to safety needs.  ORB vendors need to support high assurance ORBs that will enable their use by safety critical embedded system providers.

Standards currently exist for real-time and minimumal ORBs.  This RFP provides an opportunity to create a standard specification suitable for high assurance systems that are real-time and/or minimum while, at the same time, addressing the verification needs of safety critical systems.

This RFP seeks High Assurance features rather than just “safety critical”, “secure”, “real-time” or “minimum” ORB features.  However, it is not possible, in general, to test for high assurance. For any specific ORB feature, it is possible to demonstrate its presence by a requirements-based functional test.  One can also check for the presence of some documentation requirements, but it is not easy to determine objectively that the ORB documentation is “adequate.”  Determination of the documentation quality is part of a certification activity with the respective certification authority, e.g., FAA or NSA.   Submitters shall be expected to provide a functionality table or metamodel defining analyzable functionality for high assurance systems per RTCA/DO-178B levels (or equivalent certification guidance).

Commercial ORB developers should commit to provide a complete certification package including development process and tools, certifiable ORB, with customer support and a certification artifacts package for High Assurance customers.  The artifacts package necessary for certification shall document requirement traceability through design, code, tests and results with the required documents to enable customers to certify applications for RTCA/DO-178B (or equivalent) certified products.

This RFP seeks a High Assurance ORB that addresses the requirements of Safety Critical real-time embedded systems and is composed of analyzable functionality determined to be certifiable per RTCA/DO-178B (or equivalent certification guidance).

## Scope of Proposals Sought

The proposal shall contain specifications (CORBA compliant as far as possible) of all ORB services required.  The specification shall explain the differences between the Safety Critical ORB version and other ORB specifications such as Real-Time and Minimum CORBA specification.  For example, Safety Critical proposals can be done using a subset of the Real-Time IDL and ORB profiles or by refactoring an existing ORB solution.  This explanation should include an explanation of the submitted functionality for a High Assurance (e.g. Safe) ORB and any dependency on other OMG specifications with a description of the proposed functionality including the following:

- The needs driving the proposed High Assurance ORB functionality.

- Verification of a High Assurance ORB should complement design (and vice versa) in the development of a dependable ORB based architecture. Verification is useful for fixing problems in the design of a Safety Critical system.  Note that Verification and Validation is also important for highly reliable systems.   ORB designers must keep verification in mind to facilitate subsequent testing of the resulting High Assurance implementations by the intended users and address this as part of any proposal.

- Supported ORB base interfaces, and mechanisms, with documentation that explains how the ORB is different from the original specification such as the Real-Time or Minimum CORBA counterpart.  Before and after interface definitions might be helpful in achieving this objective.

- Interoperability scenarios need to be included that explain how the High Assurance ORB proposal will support distributed application interoperability in a safety critical environment.  Scenarios can include an explanation of how the ORB could support a secure embedded system.

- A language interface discussion.  The High Assurance ORB will be expected to interface to languages in common usage for safety critical systems.  These languages typically are: Ada, C, and C++.  The safety critical ORB proposal should address any language dependencies.  Proposals including an interface discussion shall include a reference or description of the safety critical language subset used for the design of the High Assurance ORB interfaces.

- A High Assurance ORB implementation should not preclude implementation of Lightweight services.  If there are potential incompatibilities that might preclude implementation of Lightweight services they should be discussed as part of the submission.

- A High Assurance ORB implementation should not preclude implementation of Lightweight CCM.  If there are potential incompatibilities that might preclude implementation of Lightweight CCM they should be discussed as part of the submission.

- An ORB to RTOS security interface discussion is optional.  A High Assurance ORB will be expected to interface to a variety of security devices and mechanisms.  This is considered optional, since not all High Assurance applications will require security.  However, all High Assurance proposals must have consideration for the addition of security interfaces for the Application and for the ORB.

This RFP does not require responders to invent new IDL or ORB interfaces if interfaces defined in existing OMG specifications can be used.  Responders are encouraged to define a core set of functionality and interfaces for the High Assurance ORB that will address safety critical or secure application development for embedded real-time systems, based on the existing OMG specifications.  Extensions that address deficiencies in usage of CORBA in High Assurance environments are encouraged and should be explicitly identified and rationale documenting the need for such extensions included in the proposal.

## Relationship to Existing OMG Specifications

- CORBA Component Model (Formal/2002-06-65)

- Unified Modeling Language (UML) (OMG document formal/2003-03-01)

- Meta Object Facility (OMG document formal/02-04-03)

- Minimum CORBA, V1.0 (formal/2002-08-01)

- Real-time CORBA, V1.1 (formal/2002-08-02)

- Common Secure Interoperability (CSIv2), (orbos/96-06-20)

## Related Activities, Documents and Standards

- Rockwell Collins Response to Safety Critical RFI, (mars/02-03-01)

- UML™ Profile for CORBA Components RFP, (ab/2002-10-01)

- Model-level Testing and Debugging RFP, (realtime/2003-01-12)

- Deployment & Configuration of Components-based Distributed Applications RFP, (orbos/2002-01-19)

- Lightweight Services RFP, (realtime/2002-02-31)

- The CCM RTF.

- The FAA and Industry Guide to Product Certification, January 1999

- SAE/ARP 4761, Guidelines and Methods for Conducting the Safety Assessment Process on Civil Airborne Systems and Equipment, December 1996

- RTCA Document No. DO-178B, “Software Considerations in Airborne Systems and Equipment Certification”

- “Software Considerations in Airborne Systems and Equipment Certification”, EUROCAE ED-12B

- “Guide to the Certification of Systems with Embedded Object-Oriented Software”, Aerospace Vehicle Systems Institute (AVSI)

- Ravenscar “?????????”

- John Joseph Chilenski and Steven P. Miller, "Applicability of Modified Condition/Decision Coverage to Software Testing", Software Engineering Journal, September 1994, Vol. 9, No. 5, pp.193-200.

- Daugherty, G. “A ‘substrate’ for dynamically defined behavior in high assurance, embedded, real-time systems”.

- RTCA Document No. DO-254, Design Assurance Guidance for Airborne Electronic Hardware, April 2000

- IEC 61508 Functional safety of electrical/electronic/programmable electronic safety-related systems.

- Internet RFC 2743 - Language Independent Generic Security Service API

- Internet RFC 2853 – Java-Binding Generic Security Service API

- POSIX.1b.

- The Common Criteria (www.commoncriteria.org)

- SCC: The Safe C compiler (http://www.cs.wisc.edu/~austin/scc.html)

- A Safe Dialect of C (http://www.research.att.com/projects/cyclone/)

- SPARK Ada (http://www.sparkada.com)

## Mandatory Requirements

The following subsections detail the mandatory requirements that must be met by any Safety Critical ORB proposal.

- Each proposal shall be targeted to primarily addresses the RTCA/DO-178B Level-A (or equivalent) requirements for development of high assurance systems.  The proposal shall clearly show the responder understands the software process requirements for high assurance systems with respect to RTCA DO-178B Level-A (or equivalent certification guidance).

- Any High Assurance proposal shall use OMG “Lightweight” or “minimum” specifications in preference to the full versions.

- The High Assurance ORB description shall include PIM, and CORBA PSM mapping to the PIM, and text that describes the changes made to the existing Real-Time definitions to create the High Assurance version.

- A High Assurance ORB shall be upwardly compatible with full CORBA versions. Compatibility shall be a compliant functional subset of the IDL and XML and ORB functionality. As such, any High Assurance application designed to use a lightweight version of a service shall work properly with the full version of that service.

- A High Assurance ORB shall be compatible with Real-Time CORBA specifications.

- The submission shall specify a set of operations based on the OMG minimum CORBA specification (e.g., no dynamic any’s, interface repository) plus selected features (e.g. asynchronous messaging) from other specifications for the Safety Critical ORB base interface that is sufficient for the deployment of High Assurance components. The functionality may be a subset of features primarily from real-time and minimum CORBA that are determined to be analyzable with respect to a high assurance system.

- The submission shall provide rationale for the selection of the features that are supported in the Safety Critical ORB.

- The submission shall specify a programming model for managing system resources.  This programming model shall support static resource allocation and define mechanisms for determining and validating maximum ORB resource usage.  This system resource model may be compile time or run time. This may be done as a pre-allocation of all resources, either statically or during initialization.

- Proposals shall support Extensible Transports and/or transports and protocols considered acceptable for High Assurance systems.  ….ARINC 659, TTP, Switched Ethernet etc….

- Proposals shall support user definable maximum transport packet sizes.

- Proposals shall explain how they will handle ORB exceptions for high integrity systems. Their use ensures that the application detects and handles errors, rather than ignoring them. Their unrestricted use, however, can seriously complicate control flow analysis and testing. A high assurance ORB must restrict exception usage in a manner that facilitates analysis.

- Proposals using Object Oriented programming techniques for the High Assurance ORB shall explain how Object Oriented programming techniques will ensure that class properties are visible to applications when they are inherited from the ORB interface.

- The proposal shall explain how application and ORB dead or unused code is avoided when using the high assurance ORB.  (Note: see what guidance is used for OS certification.)

- The High Assurance ORB shall minimize the need to make data copies as part of interface invocations.

- The High Assurance ORB shall have strongly typed interfaces (no type Any) for performance and analyzability.

- The High Assurance ORB shall support an analyzable asynchronous messaging.

- Proposals shall give examples or senarios showing they will satisfy the code coverage analysis criteria called Modified Condition/Decision Coverage (MC/DC) with respect to an example application.

## Optional Requirements

- Proposals may provide PSM(s) for technologies other than CORBA by mapping to the PIM of section 6.5.

- Proposals may specify subsets of other CORBA specifications to meet the goals of this RFP.

- Proposals may specify language mappings that address high assurance needs of a high assurance ORB.

## Issues to be discussed

These issues will be considered during submission evaluation. They should not be part of the proposed normative specification. (Place them in Part I of the submission.)

- How will a High Assurance CORBA application be deployable?

- How will a High Assurance CORBA application be tested/verified?

- How will a High Assurance CORBA application be analyzed?

- Simplicity is desirable for High Assurance systems since it increases understandability and limits complexity.  How does the High Assurance proposal meet this goal?

- How does the submission achieve high assurance analysis goals?  This is typically achieved by ensuring the high assurance application code does not jump or write to arbitrary locations causing unexpected non-deterministic behavior.  E.g. array bounds checking and no use of goto’s.  Programming language subsets and coding rules typically help the analysis process.

- Does the submission help establish compliance tests and metrics with respect to a High Assurance ORB functionality.

## Evaluation Criteria

Although the OMG adopts interface specifications, the technical viability of implementations will be taken into account during the evaluation process. The following criteria will be used:

- Submitters will be expected to show a functionality table primarily with respect to the RTCA/DO-178B levels and/or secondarily to the Common Criteria EAL levels.  This table must show how the proposed functionality maps to the respective certification guidance.  Proposals must at a minimum address safety related functionality.

- Proposals that provide a generalized solution demonstrating determinism, composability/modularity, strong partitioning (for safety and/or security), with minimal latency and jitter.  With the capability to do high speed data transmittal, babbling idiot avoidance, fast error detection, advanced testability, and known peak load handling even in the presence of faults will be preferred to solutions that only partially address these issues.

- Proposals that provide a generalized solution or approach to creating High Assurance ORB features that can be flexibly applied to most existing low-footprint, performance-conservative, embedded safety critical domains will be preferred.

- Proposals will be evaluated on the ease of testability/verifiability of the proposed High Assurance ORB features.

- Proposals will be evaluated with respect to testing requirements for Safety Critical (RTCA/DO-178B Level-A or equivalent) application software.

- Proposals will be evaluated on their choice of exception handling and it’s flexibility in addressing safety critical certification issues.

- Proposals will be evaluated on their thread model for handling asynchronous messaging.

- Proposals submitting security related proposal items will be evaluated for security concepts based on the needs of secure Multi-Level Security/Multiple Independent Levels of Security MLS/MILS embedded systems.

- Submissions may be defined as a meta-model of the requirements and ORB functionality.

- Proposals will be evaluated with respect to the certification documentation requirements outlined in RTCA/DO-178B (or equivalent guideline).  If an equivalent certification guideline is used the submission shall provide a mapping showing the equivalence to the RTCA/DO-178B documents: Plan for Software Aspects of Certification (PSAC), Software Development Plan (SDP), Software Verification Plan (SVP), Software Configuration Management Plan (SCMP), Software Quality Assurance Plan (SQAP), Software Requirements Standards (SRS), Software Design Standards (SDS), Software Code Standards (SCS), Software Requirements Data (SRD), Software Design Description (SDD), Software Verification Cases and Procedures (SVCP), Software Life Cycle Environment Configuration Index (SECI), Software Configuration Index (SCI) and Software Accomplishment Summary (SAS).   The proposal shall indicate what items in the proposal apply to specific certification documents.

- Proposals may outline how their ORB software development process will meet and apply to the general software design process requirements for High Assurance Systems.

- Proposals that address embedded security issues should answer the following with respect to safety critical data integrity issues: What are the security sensitive interfaces or functionality in the proposal?  Which interfaces or functional units should be subject to a security policy?  Does the proposed security mechanism or policy affect any CORBA service that may be used in a high assurance system?

- Default security policies introduced will be evaluated with respect to data integrity issues in a safety critical system. (e.g., policies for authentication, audit, authorization, message protection etc.)

- Are there data integrity considerations that implementers must be aware of?

## Other information unique to this RFP

None

## RFP Timetable

The timetable for this RFP is given below. Note that the TF or its parent TC may, in certain circumstances, extend deadlines while the RFP is running, or may elect to have more than one Revised Submission step. The latest timetable can always be found within the RFP Template on the Template Downloads page [FORMS] of OMG’s Web site. Note that “<month>” and “<approximate month>” is the name of the month spelled out; e.g., January.

| Duration | Event or Activity | Actual Date |
| --- | --- | --- |
|  | Preparation of RFP by TF | January-July, 2003 |
|  | RFP placed on OMG document server | August 18, 2003 |
|  | Approval of RFP by Architecture Board Review by TC | September 10, 2003 |
| 0 | TC votes to issue RFP | September 12, 2003 |
| 90 | LOI to submit to RFP due | March 22, 2004 |
| 105 | Initial Submissions due and placed on OMG document server (“Three week rule”) | April 5, 2004 |
| 115 | Voter registration closes | April 19, 2004 |
| 120 | Initial Submission presentations | April 26, 2004 |
|  | Preliminary evaluation by TF |  |
| 165 | Revised Submissions due and placed on OMG document server (“Three week rule”) | January, 2005 |
| 180 | Revised Submission presentations | January, 2005 |
|  | Final evaluation and selection by TF Recommendation to AB and TC |  |
|  | Approval by Architecture Board Review by TC |  |
| 210 | TC votes to recommend specification | October, 2005 |
| 240 | BoD votes to adopt specification | November, 2005 |

# Appendix A	References and Glossary