2002-06-11-a1045

## Application.

The Application class provides the mechanism for the control, configuration, and status of an instantiated application in the domain.

The Application class realizes the interface of Resource.  A created application instance.contains a setup Software Radio Components (SWRComponent).  The Application class has many relationships with other classes within the radio. These relationships are depicted in the following Figures.

The Application’s releaseObject operation provides the behavior to release the computing components allocated during the instantiation of the Application, and de-allocate the devices associated with Application instance.

An instance of an Application is returned by the create operation of an instance of the ApplicationFactory class.

UML.

Figure.  Application Class, Definition UML

Figure.  Application Class, System Relationships UML

Types & Exceptions.

ComponentProcessIdType

The ComponentProcessIdType defines a type for associating a component with its process ID. This type can be used to retrieve a process ID for a specific component.

struct ComponentProcessIdType

{

string componentId;

unsigned long processId;

};

ComponentProcessIdSequence

The ComponentProcessIdSequence type defines an unbounded sequence of components’ process IDs.

typedef sequence  <ComponentProcessIdType>  ComponentProcessIdSequence;

ComponentElementType

The ComponentElementType defines a type for associating a component with an element (e.g., naming context, implementation ID).

struct ComponentElementType

{

string componentId;

string elementId;

};

ComponentElementSequence

The ComponentElementSequence defines an unbounded sequence of ComponentElementType.

typedef sequence  <ComponentElementType>  ComponentElementSequence;

Attributes.

profile.

This profile attribute contains the SoftwareAssemblySpecification (SAS).

SCA102 The profile attribute contains either a profile element with a file reference to the SAD profile file or the XML for the SAD profile.    Files referenced within a profile will have to be obtained via FileManager.  The Application will have to be queried for profile information for component files that are referenced by an ID instead of a file name.

String profile;

name.

SCA103 This name attribute contains the name of the created Application.    The ApplicationFactory interface’s create operation name parameter provides the name content.

String name;

componentNamingContexts.

SCA104 The componentNamingContexts attribute contains the list of components’ Naming Service Context within the Application for those components using CORBA Naming Service.

ComponentElementSequence componentNamingContexts;

componentProcessIds.

SCA105 The componentProcessIds attribute contains the list of components’ process IDs within the Application for components that are executing on a device.

ComponentProcessIdSequence componentProcessIds;

componentDevices.

SCA106 The componentDevices attribute contains a list of devices, which each component either uses, is loaded on or is executed on.    Each component (componentinstantiation element in the Application’s software profile) is associated with a device.

DeviceAssignmentSequence componentDevices;

componentImplementations.

SCA107 The componentImplementations attribute contains the list of components’ Software Deployment Specification implementation IDs within the Application for those components deployed.

General Class Behavior.

SCA108 The Application shall delegate the implementation of the realized Resource operations (runTest, start, stop, configure, and query) to the Application’s SWResource component (AssemblyController) identified by the Application’s SAS AssemblyController element.    SCA109 The Application shall propagate exceptions raised by the Application’s Assembly Controller’s operations.    SCA110 The initialize operation shall not be propagated to the Application’s components or its Assembly Controller.

SCA111 The intialize operation shall cause no action within an Application.

### Operations.

releaseObject.

The releaseObject operation terminates execution of the Application, returns all allocated computing resources, and de-allocates the Resources’ capacities in use by the devices associated with Application.  Before terminating, the Application removes the message connectivity with its associated Applications (e.g., Ports, Resources, and Logs) in the domain.

releaseObject(): void

raises (ReleaseError)

Behavior.

The following behavior is in addition to the LifeCycle releaseObject operation behavior.

SCA112 For each Application component not created by a ResourceFactory, the releaseObject operation shall release the component by utilizing the Resources’s releaseObject operation.    SCA113 If the component was created by a ResourceFactory, the releaseObject operation shall release the component by the ResourceFactory releaseResource operation.    SCA114 The releaseObject operation shall shutdown a ResourceFactory when no more Resources are managed by the ResourceFactory.    SCA115 For each allocated device capable of operation execution, the releaseObject operation shall terminate all processes / tasks of the Application’s components utilizing the Device’s terminate operation.

SCA116 For each allocated device capable of memory function, the releaseObject operation shall de-allocate the memory associated with Application’s component instances utilizing the Device’s unload operation.

SCA117 The releaseObject operation shall deallocate the Devices that are associated with the Application being released, based on the Application’s Software Profile.    The actual devices deallocated (Device deallocateCapacity) will reflect changes in capacity based upon component capacity requirements deallocated from them, which may also cause state changes for the Devices.

SCA118 The Application shall release all client component references to the Application components.

SCA119 The releaseObject operation shall disconnect Ports from other Ports that have been connected based upon the software profile.

SCA120 The releaseObject operation shall disconnect consumers and producers from a CORBA Event Service’s event channel based upon the software profile.    The releaseObject operation may destroy a CORBA Event Service’s event channel when no more consumers and producers are connected to it.

For components (e.g., SCA121 Resource, ResourceFactory) that are registered with Naming Service, the releaseObject operation shall unbind those components and destroy the associated naming contexts as necessary from the Naming Service.

SCA122 The releaseObject operation for an application shall disconnect Ports first, then release the Resources and ResourceFactories, then call the terminate operation, and lastly call the unload operation on the devices.

SCA123 The releaseObject operation shall, upon successful Application release, write an Administrative_Event log record.

SCA124 The releaseObject operation shall, upon unsuccessful Application release, write a Failure_Alarm log record.

SCA125 The releaseObject operation shall, upon successful Application release, send an event to the Outgoing Domain Management event channel with event data consisting of a DomainManagementObjectRemovedEventType.   The event data will be populated as follows:

- SCA126 The producerId shall be the identifier attribute of the released Application.

- SCA127 The sourceId shall be the identifier attribute of the released Application.

- SCA128 The sourceName shall be the name attribute of the released Application.

- SCA129 The sourceCategory shall be APPLICATION.

The following steps demonstrate one scenario of the Application’s behavior for the release of an Application that contains ResourceFactory behavior:

- Client invokes releaseObject operation.

- Disconnect Ports.

- Release the ResourceFactory components.

- Shutdown the ResourceFactory components.

- Release the Resource components.

- Terminate the components’ processes.

- Unload the components’ executable images.

- Change the state of the associated device entries in the Domain Profile to be available, along with device(s) memory utilization availability and processor utilization availability based upon the Device Profile and Software Profile.

- Unbind application components from Naming Service.

- Log an Event indicating that the Application was either successfully or unsuccessfully released.

- Remove the Application reference from the applications attribute.

- Generate an event to indicate the Application has been removed from the domain.

The following figure is a collaboration diagram depicting the behavior as described above.

Figure.  ApplicationRelease Behavior

This operation does not return a value.

Exceptions/Errors.

SCA130 The releaseObject operation shall raise a ReleaseError exception when the releaseObject operation unsuccessfully releases the Application components due to internal processing errors.

getPort.

The getPort operation obtains a specific visible Port (e.g., command & control (HCI), data (red_io or black_io), responses of the Application.

getPort(in string name): Object

raises ( UnknownPort )

Behavior.

SCA131 The getPort operation shall return object references only for input port names that match the port names that are in the SAS ExternalPorts element.

Exceptions/Errors.

SCA132 The getPort operation shall raise an UnknownPort exception if the port is invalid.

getProfile.

The getProfile operation returns the application’s SAS.

getProfile(): String

Behavior.

The getProfile operation shall return either a profile element with a file reference to the SAS profile file or the XML for the SAS profile.  Files referenced within a profile will have to be obtained via FileManager.  The Application will have to be queried for profile information for component files that are referenced by an ID instead of a file name.

Exceptions/Errors.

The getProfile operation does not raise any exception.

getName.

The getName operation returns the name of the application.

getName(): String

Behavior.

The getName operation shall return the name of the instantiated Application.

Exceptions/Errors.

The getName operation does not raise any exception.

getIdentifier.

The getIdentifier operation returns the identifier of the application.

getIdentifier(): String

Behavior.

The getIdentifier operation shall return the identifier of the instantiated Application.

Exceptions/Errors.

The getIdentifier operation does not raise any exception.

getComponentNamingContexts.

The getComponentNamingContexts operation returns list of Application’s software radio components’ Naming Service Context within the Application for those components using CORBA Naming Service.

getComponentNamingContexts(): ComponentElementsSequence

Behavior.

The getComponentNamingContexts operation shall return a list of Application’s software radio components’ Naming Service Context within the Application for those components (SWRResources and ResourceFactory) using CORBA Naming Service. The SAS indicates which components are using the Naming Service.

Exceptions/Errors.

The getComponentNamingContexts operation does not raise any exception.

getComponentProcessIds.

The getComponentProcessIds operation returns list of Application’s software radio components’ process IDs within the Application for those components that have executable process IDs. A component’s Software Deployment Specification’s implementation element indicates the type of implementation element.

getComponentProcessIds(): ComponentProcessIdSequence

Behavior.

The getComponentProcessIds operation shall return a list of Application’s software radio components’ process IDs within the Application for those components that have executable process IDs.

Exceptions/Errors.

The getComponentProcessIds operation does not raise any exception.

getComponentDevices.

The getComponentDevices operation returns a list of devices, where each component uses a device, is loaded on a device and/or is executed on a device.  Each component (componentinstantiation element in the Application’s SAS) is associated with one or more devices (more is in the cases of uses relationships, in addition to the load and execute). A component’s Software Deployment Specification’s indicates the types of devices needed.

getComponentDevices(): DeviceAssignmentSequence

Behavior.

The getComponentDevices operation shall return a list of devices for each Application’s deployed component, where each component uses a device, is loaded on a device and/or is executed on a device.

Exceptions/Errors.

The getComponentDevices operation does not raise any exception.

getComponentImplementations.

The getComponentDevices operation returns the list of components’ Software Deployment Specification implementation IDs within the Application for those components deployed.  Each deployed component (SAS componentinstantiation element ) is associated with a Software Deployment Specification’s implementation).

getComponentImplementations(): ComponentElementSequence

Behavior.

The getComponentImplementations operation shall return the list of components’ Software Deployment Specification implementation IDs within the Application for those components deployed.

Exceptions/Errors.

The getComponentImplementations operation does not raise any exception.