### Device Configuration Deployment Specification

This package contains the elements for describing a device configuration specification.  The definition is an extension to the concepts in the Assembly Deployment Specification package. The main elements in this package are:

DCD_ComponentPlacement

DeviceConfigurationDeploymentSpecification

FileSystemName

#### DCD_ComponentPlacement

The DCD_ComponenPlacement class, a type of ComponentPlacement, defines the common attributes and relationships for a component placement for a device configuration.   The extensions made to the ComponentPlacement are the concepts of devices and their relationships. A DeviceManager may deploy components onto various different nodes upon startup.  The components a DeviceManager may deploy are logical devices.  These logical devices can be composite types of other logical devices. The DCD_ComponenPlacement provides the mechanism to define these relationships, thus allowing the DeviceManager to build these relationships during system startup.

UML.

Attributes.

deployOnDevice

The optional deployOnDevice is used to reference a componentinstantiation element on which this componentinstantiation is deployed on.

compositePartOfDevice

The compositePartOfDevice is used when an aggregate relationship exists between logical devices. The association describes the whole Device for which these Devices’ ComponentInstantiations are a part of.

hwSpecification

The optional hwSpecification attribute is the associated hardware specification for the logical device component instantiations.

#### DeviceConfigurationDeploymentSpecification

The DeviceConfigurationDeploymentSpecification class, a type of AssemblyDeploymentSpecification, defines the common attributes and relationships for a device (or node) configuration specification.   The extensions made to the DeviceConfigurationDeploymentSpecification are the concepts of FileSystemName, domainManagerNamingContext, DCD_ComponentPlacement, and deviceManagerSpecification.

UML.

Attributes.

deviceManagerSpecification

The deviceManagerSpecification attribute is the associated DeviceManager’s specification. The association can be used to describe the DeviceManager’s implementation and to specify the usesports for the services (Log(s), etc.) used by the DeviceManager.

domainManagerNamingContext

The domainManagerNamingContext attribute is the naming context string to be used for obtaining the DomainManager’s object reference from a naming service.

fileSystemNames

The optional fileSystemNames attribute is the file system names for the DeviceManager’s mounted files systems.

partitioning

The optional partitioning attribute contains the list of elements to be depoyed by the DeviceManager. The elements to be deployed are services (e.g., component homes, Log, CORBA Services, etc.) and logical Devices.

#### FileSystemName

The FileSystemName class identifies the file system name for a physical file system.

UML.

Attributes.

deviceId

The deviceId attribute is a device identifier unique within a device’s operating system.

mountName

The mountName attribute is the file system name to be used for the physical file system. The mountName is the mounted names of the FileSystems associated with a FileManager.