## 1.1 Domain Management Services

The domain management interfaces and classes are:

- DomainRegistration Interface

- DomainHCI Interface

- DomainEventChannels Interface

- ApplicationInstallation Interface

- DomainManager Class

The relationships among these elements are shown Figure 1.

Figure 1. Domain Management Pkg Overview

### Domain Registration Interface

The DomainRegistration interface is the set of operations that are used to register and unregister objects within a domain. These objects are part of the domain upon start up and when they get created they are registered to the DomainManager.

UML.

Figure 2. DomainRegistration Definition

Types & Exceptions.

DeviceManagerNotRegistered Exception

The DeviceManagerNotRegistered exception indicates the registering Device’s DeviceManager is not registered in the DomainManager.  A Device’s DeviceManager has to be registered prior to a Device registration to the DomainManager.

RegisterError.

The RegisterError exception, a type of SystemException, indicates that an internal error has occurred to prevent DomainManager registration operations from successful completion. The error number indicates an ErrorNumberType value. The message is component-dependent, providing additional information describing the reason for the error.

UnregisterError.

The UnregisterError exception, a type of SystemException, indicates that an internal error has occurred to prevent DomainManager unregister operations from successful completion. The error number indicates an ErrorNumberType value. The message is component-dependent, providing additional information describing the reason for the error.

#### Operations

registerDeviceManager.

The registerDeviceManager operation is used to register a DeviceManager, its Device(s), and its Services.  Software profiles can also be obtained from the DeviceManager's FileSystem.

registerDeviceManager(in deviceMgr : DeviceManager) : void raises (InvalidObjectReference, InvalidProfile, RegisterError )

registerDevice.

The registerDevice operation is used to register a Device for a specific DeviceManager in the DomainManager's Domain Profile.

registerDevice(in registeringDevice : Device, in registeredDeviceMgr : DeviceManager) : void raises (InvalidObjectReference, InvalidProfile, DeviceManagerNotRegistered, RegisterError )

unregisterDeviceManager.

The unregisterDeviceManager operation is used to unregister a DeviceManager component from the DomainManager’s Domain Profile.  A DeviceManager may be unregistered during run-time for dynamic extraction or maintenance of the DeviceManager.

unregisterDeviceManager(in deviceMgr : DeviceManager) : void raises (InvalidObjectReference, UnregisterError)

unregisterDevice.

The unregisterDevice operation is used to remove a device entry from the DomainManager for a specific DeviceManager.

unregisterDevice(in unregisteringDevice : Device) : void  raises (InvalidObjectReference, UnregisterError)

registerService.

The registerService operation is used to register a service for a specific DeviceManager with the DomainManager.

registerService(in registeringService : Object, in registeredDeviceMgr : DeviceManager, in name : String) : void raises (InvalidObjectReference, DeviceManagerNotRegistered, RegisterError)

unregisterService.

The unregisterService operation is used to remove a service entry from the DomainManager for a specific DeviceManager.

unregisterService(in unregisteringService : Object, in name : String) : void raises ( InvalidObjectReference, UnregisterError)

### Domain HCI Interface

The DomainManagerHCI interface is the set of operations that a HCI client would use to retrieve domain information.

UML.

Figure 3. DomainHCI Definition

Types & Exceptions.

DeviceManagerSequence.

This type defines an unbounded sequence of DeviceManager(s).

ApplicationSequence.

This type defines an unbounded sequence of Application(s).

ApplicationFactorySequence.

This type defines an unbounded sequence of ApplicationFactory(s).

#### Operations

getApplications

The getApplications operation returns the list of instantiated Applications in a domain.

getApplications() : ApplicationSequence

getApplicationFactories

The getApplicationFactories operation returns the list of installed ApplicationFactories within the domain.

getIdentifier

The getIdentifier operation returns the identifier of the domain.

getIdentifier() : String

getFileManager

The getFileManager operation returns the domain’s FileManager.

getFileMgr() : FileManager

getDeviceManagers

The getDeviceManagers operation returns the list of registered DeviceManagers within the domain.

getDeviceManagers() : DeviceManagerSequence

getDomainManagerProfile

The getDomainManagerProfile operation returns the Domain’s profile.

getDomainManagerProfile() : String

### Domain Event Channels Interface

The DomainEventChannels interface is the set of operations that a client can used for registering or unregistering with a domain event channel.

UML.

Figure 4. DomainEventChannels Definition

Types & Exceptions.

InvalidEventChannelName.

The InvalidEventChannelName exception indicates that a DomainManager was not able to locate the event channel.

AlreadyConnected.

The AlreadyConnected exception indicates that a registering consumer is already connected to the specified event channel.

NotConnected.

The NotConnected exception indicates that the unregistering consumer was not connected to the specified event channel.

#### Operations

registerWithEventChannel.

The registerWithEventChannel operation is used to connect a consumer to a domain’s event channel.

registerWithEventChannel(in registeringObject : Object, in registeringId : String, in eventChannelName : String) : void raises (InvalidObjectReference, InvalidEventChannelName, AlreadyConnected)

unregisterFromEventChannel.

The unregisterFromEventChannel operation is used to disconnect a consumer from a domain’s event channel.

unregisterFromEventChannel(in unregisteringId : String, in eventChannelName : String) : void raises (InvalidEventChannelName, NotConnected)

### Application Installation Interface

The ApplicationInstallation interface is the set of operations for installing and uninstalling an application from a domain.

UML.

Figure 5. ApplicationInstallation Definition

Types & Exceptions.

ApplicationInstallationError.

The ApplicationInstallationError exception, a type of System Exception, is raised when an Application installation has not completed correctly. The error number indicates an ErrorNumberType value (e.g., EINVAL, ENAMETOOLONG, ENOENT, ENOMEM, ENOSPC, ENOTDIR, ENXIO). The message is component-dependent, providing additional information describing the reason for the error.

InvalidIdentifier.

The InvalidIdentifier exception indicates an application identifier is invalid.

ApplicationUninstallationError.

The ApplicationUninstallationError exception, a type of SystemException, is raised when an Application uninstallation has not completed correctly. The error number indicates an ErrorNumberType value. The message is component-dependent, providing additional information describing the reason for the error.

#### Operations

installApplication.

The installApplication operation is used to install new application software in the domain. Installer application invokes this operation when it has completed the installation of a new application into the domain.

installApplication(in profileFileName : String) : void raises ( InvalidProfile, InvalidFileName, ApplicationInstallationError)

uninstallApplication.

The uninstallApplication operation is used to uninstall an ApplicationFactory in the domain.

An installer application invokes this operation when removing an ApplicationFactory from the domain.

uninstallApplication(in ApplicationId : String) : void raises (InvalidIdentifier, ApplicationUninstallationError)

### DomainManager.

The DomainManager class is for the control and configuration of the system domain.

The DomainManager class can be logically grouped into three categories: Human Computer Interface (HCI), Registration, and administration.

The HCI operations are used to configure the domain, get the domain’s capabilities (Devices, Services, and Applications), and initiate maintenance functions.  Host operations are performed by an HCI client capable of interfacing to the DomainManager.

The registration operations are used to register / unregister DeviceManagers, DeviceManager’s Devices, DeviceManager’s Services, and Applications at startup or during run-time for dynamic device, service, and application extraction and insertion.

The administration operations are used to access the interfaces of registered DeviceManagers and DomainManager's FileManager.

UML.

Figure 6. DomainManager Construction Dependencies

Figure 7. DomainManager System Relationships

Types & Exceptions.

None.

Attributes.

deviceManagers.

The deviceManagers attribute contains a sequence of registered DeviceManagers in the domain.  The deviceManagers attribute contains a list of registered DeviceManagers that have registered with the DomainManager.

deviceManagers : DeviceManagerSequence

applications.

The applications attribute contains a sequence of instantiated Applications in the domain.  The applications attribute contains the list of Applications that have been instantiated.

applications : ApplicationSequence

applicationFactories.

The applicationFactories attribute contains a list with one ApplicationFactory per application successfully installed (i.e. no exception raised).

applicationFactories : ApplicationFactorySequence

fileMgr.

The fileMgr attribute contains the DomainManager’s FileManager.

fileMgr : FileManager

domainManagerProfile.

The domainManagerProfile attribute contains the DomainManager’s profile.  The domainManagerProfile attribute contains either a profile element with a file reference to the DomainManagerConfigurationSpecification profile or the contents of the specification.  Files referenced within the profile will have to be obtained from the DomainManager’s FileManager.

domainManagerProfile  : String

identifier.

The identifier attribute contains a unique identifier for a DomainManager instance. The identifier shall be identical to the domainmanagerconfiguration element id attribute of the DomainManagerConfigurationSpecification.

identifier : String

eventChannels.

The eventChannels attribute contains a list of the EventChannels created by the DomainManager upon registration of elements to the DomainManager.

incomingDomainEC.

The incomingDomainEC attribute is the event channel that can be used by the DomainManager for processing internal domain events such as device’s state changes.

outgoingDomainEC.

The outgoingDomainEC attribute is the event channel that is used by the DomainManager when elements are removed or added to the domain.

#### General Class Behavior.

During component construction the DomainManager shall register itself with the Naming Service.  During Naming Service registration the DomainManager shall create a "naming context" using "/DomainName" as its name.ID component and "" (Null string) as its name.kind component, then create a "name binding" to the "/DomainName" naming context using "/DomainManager" as its name.ID component, "" (Null string) as its name.kind component, and the DomainManager's object reference.

Since a log service is not a required component of an installation, a DomainManager implementation may, or may not have access to a Log. However, if log service(s) are available, a DomainManager implementation may use one or more of them.  The Logs utilized by the DomainManager implementation shall be defined in the DomainManagerConfigurationSpecification.

Once a service specified in the DomainManagerConfigurationSpecification is successfully registered with the DomainManager (via registerDeviceManager or registerService operations), the DomainManager shall begin to use the service (e.g., Log).

The DomainManager shall create its own FileManager component that consists of all registered DeviceManager’s FileSystems.

The DomainManager shall restore ApplicationFactories after startup for applications that were previously installed by the DomainManager installApplication operation.  The DomainManager shall add the restored ApplicationFactories to the DomainManager’s applicationFactories attribute.

The DomainManager shall create the Incoming Domain Management and Outgoing Domain Management event channels.

#### Operations

registerDeviceManager.

The registerDeviceManager operation is used to register a DeviceManager, its Device(s), and its Services.  Software profiles can also be obtained from the DeviceManager's FileSystem.

registerDeviceManager(in deviceMgr : DeviceManager) : void raises (InvalidObjectReference, InvalidProfile, RegisterError )

Behavior.

The registerDeviceManager operation verifies that the input parameter, deviceMgr, is a not a nil component reference.

SCA203 The registerDeviceManager operation shall add the input deviceMgr to the DomainManager’s deviceManagers attribute, if it does not already exist.    SCA204 The registerDeviceManager operation shall add the input deviceMgr’s registeredDevices and each registeredDevice’s attributes  (e.g., identifier, softwareProfile’s allocation properties, etc.) to the DomainManager.  The registerDeviceManager operation associates the input deviceMgr’s with the input deviceMgr’s registeredDevices in the DomainManager in order to support the unregisterDeviceManager operation.

SCA205 The registerDeviceManager operation shall add the input deviceMgr’s registeredServices and each registeredService’s names to the DomainManager.    The registerDeviceManager operation associates the input deviceMgr’s with the input deviceMgr’s registeredServices in the DomainManager in order to support the unregisterDeviceManager operation.

SCA206 The registerDeviceManager operation shall perform the connections specified in the connections element of the deviceMgr’s Device Configuration Descriptor (DCD) file.    SCA207 If the DeviceManager’s DCD describes a connection for a service that has not been registered with the DomainManager, the registerDeviceManager operation shall establish any pending connection when the service registers with the DomainManager by the registerDeviceManager operation.   SCA208 For connections established for an Event Service’s event channel, the registerDeviceManager operation shall connect a CosEventComm PushConsumer or PushSupplier object to the event channel as specified in the DCD’s domainfinder element.   SCA209 If the event channel does not exist, the registerDeviceManager operation shall create the event channel.

SCA210 The registerDeviceManager operation shall obtain all the Software profiles from the registering DeviceManager's FileSystems.

SCA211 The registerDeviceManager operation shall mount the DeviceManager’s FileSystem to the DomainManager’s FileManager.    SCA212 The mounted FileSystem name shall have the format, “/DomainName/HostName”, where DomainName is the name of the domain and HostName is the input deviceMgr’s label attribute.

SCA213 The registerDeviceManager operation shall, upon unsuccessful DeviceManager registration, write a Failure_Alarm log record to a DomainManager’s Log.

SCA214 The registerDeviceManager operation shall, upon successful DeviceManager registration, send an event to the outgoingDomainEvent Channel with event data consisting of a DomainManagementObjectAddedEventType.   The event data will be populated as follows:

- The producerId shall be the identifier attribute of the DomainManager.

- The sourceId shall be the identifier attribute of the registered DeviceManager.

- The sourceName shall be the label attribute of the registered DeviceManager.

- The sourceIOR shall be the registered DeviceManager object reference.

- The sourceCategory shall be DEVICE_MANAGER.

The following UML sequence diagram (Figure) illustrates the DomainManager’s behavior for the registerDeviceManager operation.

Figure.  DomainManager Sequence Diagram for registerDeviceManager Operation

Exceptions/Errors.

SCA220 The registerDeviceManager operation shall raise the InvalidObjectReference exception when the input parameter deviceMgr contains an invalid reference to a DeviceManager interface.

SCA221 The registerDeviceManager operation shall raise the RegisterError exception when an internal error exists which causes an unsuccessful registration.

registerDevice.

The registerDevice operation is used to register a Device for a specific DeviceManager in the DomainManager's Domain Profile.

registerDevice(in registeringDevice : Device, in registeredDeviceMgr : DeviceManager) : void raises (InvalidObjectReference, InvalidProfile, DeviceManagerNotRegistered, RegisterError )

Behavior.

The registerDevice operation verifies that the input parameters, registeringDevice and registeredDeviceMgr, are not nil component references.

SCA222 The registerDevice operation shall add the registeringDevice and the registeringDevice’s attributes (e.g.,   identifier, softwareProfile’s allocation properties, etc.) to the DomainManager, if it does not already exist.

The registerDevice operation associates the input registeringDevice with the input registeredDeviceMgr in the DomainManager when the input registeredDeviceMgr is a valid registered DeviceManager in the DomainManager.

SCA223 When the registering Device’s parent DeviceManager’s DCD describes service connections for the registering Device, the registerDevice operation shall establish the connections.

SCA224 The registerDevice operation shall, upon successful device registration, write an Administrative_Event log record to a DomainManager’s Log, to indicate that the device has successfully registered with the DomainManager.

SCA225 Upon unsuccessful device registration, the registerDevice operation shall write a Failure_Alarm log record to a DomainManager’s Log, when the InvalidProfile exception is raised to indicate that the registeringDevice has an invalid profile.

SCA226 Upon unsuccessful device registration, the registerDevice operation shall write a Failure_Alarm log record to a DomainManager’s Log, indicating that the device could not register because the DeviceManager is not registered with the DomainManager.

SCA227 Upon unsuccessful device registration, the registerDevice operation shall write a Failure_Alarm log record to a DomainManager’s Log, because of an invalid reference input parameter.

SCA228 Upon unsuccessful device registration, the registerDevice operation shall write a FAILURE_ALARM log record to a DomainManager’s Log, because of an internal registration error.

SCA229 The registerDevice operation shall, upon successful Device registration, send an event to the Outgoing Domain Management event channel with event data consisting of a DomainManagementObjectAddedEventType.   The event data will be populated as follows:

- SCA230 The producerId shalll be the identifier attribute of the DomainManager.

- SCA231 The sourceId shall be the identifier attribute of the registered Device.

- SCA232 The sourceName shall be the label attribute of the registered Device.

- SCA233 The sourceIOR shall be the registered Device object reference.

- SCA234 The sourceCategory shall be DEVICE.

The following UML sequence diagram illustrates the DomainManager's behavior for the registerDevice operation.

Figure.  DomainManager Sequence Diagram for registerDevice Operation

Exceptions/Errors.

SCA235 The registerDevice operation shall raise the InvalidProfile exception when:

The Device's  EmbeddedComponentDeploymentSpecification and/or its associated specifications do not exist or cannot be processed due to the specifications being ill formed, or the Device does not associated allocation properties.

SCA236 The registerDevice operation shall raise a DeviceManagerNotRegistered exception when the input registeredDeviceMgr (not nil reference) is not registered with the DomainManager.

SCA237 The registerDevice operation shall raise the InvalidObjectReference exception when input parameters registerDevice or registeredDeviceMgr contains an invalid reference.

SCA238 The registerDevice operation shall raise the RegisterError exception when an internal error exists which causes an unsuccessful registration.

installApplication.

The installApplication operation is used to install new application software in the DomainManager's Domain Profile.  An installer application invokes this operation when it has completed the installation of a new application into the domain.

installApplication(in profileFileName : String) : void raises ( InvalidProfile, InvalidFileName, ApplicationInstallationError)

Behavior.

The profileFileName is the absolute path of the profile filename.

SCA239 The installApplication operation shall verify the application’s SAD file exists in the DomainManager’s FileManager and all the files the application is dependent on are also resident.

SCA240 The installApplication operation shall write an Administrative_Event log record to a DomainManager’s Log, upon successful Application installation.

SCA241 The installApplication operation shall, upon unsuccessful application installation, write a Failure_Alarm log record to a DomainManager’s Log.

SCA242 The installApplication operation shall, upon successful application installation, send an event to the Outgoing Domain Management event channel with event data consisting of a DomainManagementObjectAddedEventType.   The event data will be populated as follows:

- SCA243 The producerId shall be the identifier attribute of the DomainManager.

- SCA244 The sourceId shall be the identifier attribute of the installed ApplicationFactory.

- SCA245 The sourceName shall be the name attribute of the installed ApplicationFactory.

- SCA246 The sourceIOR shall be the installed ApplicationFactory object reference.

- SCA247 The sourceCategory shall be APPLICATION_FACTORY.

Exceptions/Errors.

SCA248 The installApplication operation shall raise the ApplicationInstallationError exception when the installation of the Application file(s) was not successfully completed.

SCA249 The installApplication operation shall raise the InvalidFileName exception when the input SAD file or any referenced file name does not exist in the file system as defined in the absolute path of the input profileFileName.    SCA250 When the InvalidFileName exception occurs, the installApplication operation shall log a FAILURE_ALARM log record to a DomainManager’s Log with a message consisting of “installApplication::invalid file is xxx”, where “xxx” is the input or referenced file name that is bad.

SCA251 The installApplication operation shall raise the InvalidProfile exception when the input EmbeddedCADSpecification or any associated specification is not syntactically or semantically correct.    SCA252 When the InvalidProfile exception occurs, the installApplication operation shall log a FAILURE_ALARM log record to a DomainManager’s Log with a message consisting of “installApplication::invalid Profile is yyy,” where “yyy” is the input or referenced file name that is bad along with the element or position within the profile that is bad.

unregisterDeviceManager.

The unregisterDeviceManager operation is used to unregister a DeviceManager component from the DomainManager’s Domain Profile.  A DeviceManager may be unregistered during run-time for dynamic extraction or maintenance of the DeviceManager.

unregisterDeviceManager(in deviceMgr : DeviceManager) : void raises (InvalidObjectReference, UnregisterError)

Behavior.

SCA253 The unregisterDeviceManager operation shall unregister a DeviceManager component from the DomainManager.

SCA254 The unregisterDeviceManager operation shall release all device(s) and service(s) associated with the DeviceManager that is being unregistered.

SCA255 The unregisterDeviceManager operation shall disconnect consumers and producers (e.g.,   Devices, Log, DeviceManager, etc.) from an Event Service event channel based upon the software profile.  The unregisterDeviceManager operation may destroy the Event Service event channel when no more consumers and producers are connected to it.

SCA256 The unregisterDeviceManager operation shall unmount all DeviceManager’s FileSystems from its File Manager.

SCA257 The unregisterDeviceManager operation shall, upon the successful unregistration of a DeviceManager, write an Administrative_Event log record to a DomainManager’s Log.

SCA258 The unregisterDeviceManager operation shall, upon unsuccessful unregistration of a DeviceManager, write a Failure_Alarm log record to a DomainManager’s Log.

SCA259 The unregisterDeviceManager operation shall, upon successful unregistration, send an event to the Outgoing Domain Management event channel with event data consisting of a DomainManagementObjectRemovedEventType.   The event data will be populated as follows::

- The producerId shall be the identifier attribute of the DomainManager.

- The sourceId shall be the identifier attribute of the unregistered DeviceManager.

- The sourceName shall be the label attribute of the unregistered DeviceManager.

- The sourceCategory shall be DEVICE_MANAGER.

Exceptions/Errors.

SCA264 The unregisterDeviceManager operation shall raise the InvalidObjectReference when the input parameter DeviceManager contains an invalid reference to a DeviceManager interface.

SCA265 The unregisterDeviceManager operation shall raise the UnregisterError exception when an internal error exists which causes an unsuccessful unregistration.

unregisterDevice.

The unregisterDevice operation is used to remove a device entry from the DomainManager for a specific DeviceManager.

unregisterDevice(in unregisteringDevice : Device) : raises (InvalidObjectReference, UnregisterError)

Behavior.

SCA266 The unregisterDevice operation shall remove a device entry from the DomainManager.

SCA267 The unregisterDevice operation shall release (client-side release) the unregisteringDevice from the Domain Manager.

SCA268 The unregisterDevice operation shall disconnect the Device’s consumers and producers from an Event Service event channel based upon the software profile.   The unregisterDevice operation may destroy the Event Service event channel when no more consumers and producers are connected to it.

SCA269 The unregisterDevice operation shall, upon the successful unregistration of a Device, write an Administrative_Event log record to a DomainManager’s Log.

SCA270 The unregisterDevice operation shall, upon unsuccessful unregistration of a Device, write a  Failure_Alarm log record to a DomainManager’s Log.

SCA271 The unregisterDevice operation shall, upon successful Device unregistration, send an event to the Outgoing Domain Management event channel with event data consisting of a DomainManagementObjectRemovedEventType.   The event data will be populated as follows:

- The producerId shall be the identifier attribute of the DomainManager.

- The sourceId shall be the identifier attribute of the unregistered Device.

- The sourceName shall be the label attribute of the unregistered Device.

- The sourceCategory shall be DEVICE.

This operation does not return a value.

Exceptions/Errors.

SCA276 The unregisterDevice operation shall raise the InvalidObjectReference exception when the input parameter contains an invalid reference to a Device interface.

SCA277 The unregisterDevice operation shall raise the UnregisterError exception when an internal error exists which causes an unsuccessful unregistration.

uninstallApplication.

The uninstallApplication operation is used to uninstall an ApplicationFactory in the DomainManager’s Domain Profile.

An installer application invokes this operation when removing an ApplicationFactory from the domain.

void uninstallApplication(in ApplicationId : String) : void raises (InvalidIdentifier, ApplicationUninstallationError)

Behavior.

The ApplicationId parameter is the softwareassembly element id attribute of the ApplicationFactory’s Software Assembly Descriptor file.

SCA278 The uninstallApplication operation shall remove all files associated with the Application.

SCA279 The uninstallApplication operation shall make the ApplicationFactory unavailable from the DomainManager (i.e.   its services no longer provided for the Application).

SCA280 The uninstallApplication operation shall, upon successful uninstall of an Application, write an AdministraTive_Event log record to a DomainManager’s Log.

SCA281 The uninstallApplication operation shall, upon unsuccessful uninstall of an Application, write a Failure_Alarm log record to a DomainManager’s Log.

SCA282 The uninstallApplication operation shall, upon unsuccessful uninstall of an Application, log a FAILURE_ALARM log record to a DomainManager’s Log.

SCA283 The uninstallApplication operation shall, upon successful uninstall of an application, send an event to the Outgoing Domain Management event channel with event data consisting of a DomainManagementObjectRemovedEventType.   The event data will be populated as follows::

- The producerId shall be the identifier attribute of the DomainManager.

- The sourceId shall be the identifier attribute of the uninstalled ApplicationFactory.

- The sourceName shall be the name attribute of the uninstalled ApplicationFactory.

- The sourceCategory shall be APPLICATION_FACTORY.

Exceptions/Errors.

SCA288 The uninstallApplication operation shall raise the InvalidIdentifier exception when the ApplicationId is invalid.

SCA289 The uninstallApplication operation shall raise the ApplicationUninstallationError exception when an internal error causes unsuccessful uninstall of the application.

registerService.

The registerService operation is used to register a service for a specific DeviceManager with the DomainManager.

registerService(in registeringService : Object, in registeredDeviceMgr : DeviceManager, in name : String) : void raises (InvalidObjectReference, DeviceManagerNotRegistered, RegisterError)

Behavior.

SCA290 The registerService operation shall verify the input registeringService and registeredDeviceMgr are valid object references.

SCA291 The registerService operation shall verify the input registeredDeviceMgr has been previously registered with the DomainManager.

SCA292 The registerService operation shall add the registeringService’s object reference and the registeringService’s name to the DomainManager, if the name for the type of  service being registered does not exist within the DomainManager.    SCA293 However, if the name of the registering service is a duplicate of a registered service of the same type, then the new service shall not be registered with the DomainManager.

SCA294 The registerService operation shall associate the input registeringService parameter with the input registeredDeviceMgr parameter in the DomainManager’s, when the registeredDeviceMgr parameter indicates a DeviceManager registered with the DomainManager.

SCA295 The registerService operation shall, upon successful service registration, establish any pending connection requests for the registeringService.   SCA296 The registerService operation shall, upon successful service registration, write an Administrative_Event log record to a DomainManager’s Log.

SCA297 The registerService operation shall, upon unsuccessful service registration, write a Failure_Alarm log record to a DomainManager’s Log.

SCA298 The registerService operation shall, upon successful service registration, send an event to the Outgoing Domain Management event channel with event data consisting of a DomainManagementObjectAddedEventType.   The event data will be populated as follows::

- The producerId shall be the identifier attribute of the DomainManager.

- The sourceId shall be the identifier attribute from the componentinstantiation element associated with the registered service.

- The sourceName shall be the input name parameter for the registering service.

- The sourceIOR shall be the registered service object reference.

- The sourceCategory shall be SERVICE.

The following UML sequence diagram illustrates the DomainManager's behavior for the registerService operation.

Figure. DomainManager Sequence Diagram for registerService Operation

Exceptions/Errors.

SCA304 The registerService operation shall raise a DeviceManagerNotRegistered exception when the input registeredDeviceMgr parameter is not a nil reference and is not registered with the DomainManager.

SCA305 The registerService operation shall raise the InvalidObjectReference exception when input parameters registeringService or registeredDeviceMgr contains an invalid reference.

SCA306 The registerService operation shall raise the RegisterError exception when an internal error exists which causes an unsuccessful registration.

unregisterService.

The unregisterService operation is used to remove a service entry from the DomainManager for a specific DeviceManager.

unregisterService(in unregisteringService : Object, in name : String) : void raises ( InvalidObjectReference, UnregisterError)

Behavior.

SCA307 The unregisterService operation shall remove the unregisteringService entry specified by the input name parameter from the DomainManager.

SCA308 The unregisterService operation shall release (client-side release) the unregisteringService from the DomainManager.

SCA309 The unregisterService operation shall, upon the successful unregistration of a Service, write an ADMINISTRATIVE_EVENT log record to a DomainManager’s Log.

SCA310 The unregisterService operation shall, upon unsuccessful unregistration of a Service, write a Failure_Alarm log record to a DomainManager’s Log.

SCA311 The unregisterService operation shall, upon successful service unregistration, send an event to the Outgoing Domain Management event channel with event data consisting of a DomainManagementObjectRemovedEventType.   The event data will be populated as follows:

- The producerId shall be the identifier attribute of the DomainManager.

- The sourceId shall be the ID attribute from the componentinstantiation element associated with the unregistered service.

- The sourceName shall be the input name parameter for the unregistering service.

- The sourceCategory shall be SERVICE.

Exceptions/Errors.

SCA316 The unregisterService operation shall raise the InvalidObjectReference exception when the input parameter contains an invalid reference to a Service interface.

SCA317 The unregisterService operation shall raise the UnregisterError exception when an internal error exists which causes an unsuccessful unregistration.

registerWithEventChannel.

The registerWithEventChannel operation is used to connect a consumer to a domain’s event channel.

registerWithEventChannel(in registeringObject : Object, in registeringId : String, in eventChannelName : String) : void raises (InvalidObjectReference, InvalidEventChannelName, AlreadyConnected)

Behavior.

The registerWithEventChannel operation shall connect the input registeringObject to an event channel as specified by the input eventChannelName.

Returns.

This operation does not return a value.

Exceptions/Errors.

SCA319 The registerWithEventChannel operation shall raise the InvalidObjectReference exception when the input registeringObject parameter contains an invalid reference to a CosEventComm PushConsumer interface.

SCA320 The registerWithEventChannel operation shall raise the InvalidEventChannelName exception when the input eventChannelName parameter contains an invalid event channel name (e.g, "ODM_Channel").

SCA321 The registerWithEventChannel operation shall raise AlreadyConnected exception when the input parameter contains a connection to the event channel for the input registeringId parameter.

unregisterFromEventChannel.

The unregisterFromEventChannel operation is used to disconnect a consumer from a domain’s event channel.

unregisterFromEventChannel(in unregisteringId : String, in eventChannelName : String) : void raises (InvalidEventChannelName, NotConnected)

Behavior.

The unregisterFromEventChannel operation shall disconnect a registered component from the event channel as identified by the input parameters.

Exceptions/Errors.

SCA323 The unregisterFromEventChannel operation shall raise the InvalidEventChannelName exception when the input eventChannelName parameter contains an invalid reference to an event channel (e.g.,   "ODM_Channel").

SCA324 The unregisterFromEventChannel operation shall raise the NotConnected exception when the input parameter unregisteringId parameter is not connected to specified input event channel.

getDeviceManagers.

The getDeviceManagers operation returns the list of registered DeviceManagers.

getDeviceManagers() : DeviceManagerSequence

Behavior.

The getDeviceManagers operation shall return the list of registered DeviceManagers that have registered with the DomainManager. The getDeviceManagers shall write an Administrative_Event log to a DomainManager’s Log, when the deviceManagers attribute is obtained by a client.

Exceptions/Errors.

The getDeviceManagers operation does not return any exceptions.

getApplications.

The getApplications operation returns the list of instantiated Applications.

getApplications() : ApplicationSequence

Behavior.

The getApplications operation shall return the list of instantiated Applications. The getApplications shall write an Administrative_Event log to a DomainManager’s Log, when the applications attribute is obtained by a client.

Exceptions/Errors.

The getApplications operation does not return any exceptions.

getApplicationFactories.

The getApplicationFactories operation returns the list of installed ApplicationFactories.

getApplicationFactories() : ApplicationFactorySequence

Behavior.

The getApplicationFactories operation shall return the list of installed ApplicationFactories. The getApplicationFactories shall write an Administrative_Event log to a DomainManager’s Log, when the applicationFactories attribute is obtained by a client.

Exceptions/Errors.

The getApplicationFactories operation does not return any exceptions.

getFileManager.

The getFileManager operation returns the domain’s FileManager.

getFileMgr() : FileManager

Behavior.

The getFileManager operation shall return the DomainManager’s fileMgr atribute. The getFileManager shall write an Administrative_Event log to a DomainManager’s Log, when the fileMgr attribute is obtained by a client.

Exceptions/Errors.

The getFileManager operation does not return any exceptions.

getDomainManagerProfile.

The getDomainManagerProfile operation returns the DomainManager’s profile.

getDomainManagerProfile() : String

Behavior.

The domainManagerProfile attribute contains the DomainManager’s profile.

The getDomainManagerProfile attribute shall return a string that contains either a Profile element with a file reference to the DomainManager Configuration Descriptor (DMD) profile or a DomainManager’s (DMD) profile.  Files referenced within the profile will have to be obtained from the DomainManager’s FileManager.

Exceptions/Errors.

The getDomainManagerProfile operation does not return any exceptions.

getIdentifier.

The getIdentifier operation returns the identifier of the DomainManager.

getIdentifier() : String

Behavior.

The getIdentifier operation shall return identifier of the DomainManager. The getIdentifier shall write an Administrative_Event log to a DomainManager’s Log, when the identifier attribute is obtained by a client.

Exceptions/Errors.

The getIdentifier operation does not return any exceptions.