### File Services.

The FileServices consist of interfaces and classes that are used to manage and access a distributed file system. The File Services are used for installation and removal of application files within the system, and for loading and unloading application files on the various processors that the Devices execute upon. The File Services, as shown in Figure 1, consist of the following elements:

- FileSystem Class

- FileManager Class

- FileSystemInt Interface

- File Class

Figure 1.  File Services Overview UML

#### File.

The File class provides the ability to read and write files residing within a distributed FileSystem.  A file can be thought of conceptually as a sequence of octets with a current filePointer describing where the next read or write will occur.  This filePointer points to the beginning of the file upon construction of the file object.  The File class is modeled after the POSIX/C file interface.

UML.

Figure 2.  File Construction Dependencies UML

Types & Exceptions.

IOException.

The IOException exception indicates an error occurred during a read or write operation to a File. SCA444 The error number shall indicate an ErrorNumberType value (e.g.,   EFBIG, ENOSPC, EROFS). The message is component-dependent, providing additional information describing the reason for the error.

Figure 3.  IOException Construction Dependencies UML

InvalidFilePointer.

The InvalidFilePointer exception indicates the file pointer is out of range based upon the current file size.

fileName.

SCA445 The fileName  private attribute contains the file name given to the FileSystem open/create operation.    The syntax for a filename is based upon the UNIX operating system.  That is, a sequence of directory names separated by forward slashes (/) followed by the base filename.  The fileName attribute will contain the filename given to the FileSystem open operation.

filename : String

filePointer.

SCA446 The filePointer private attribute contains the file position where the next read or write will occur.

filePointer : UnsignedLong

##### Operations

read.

Applications require the read operation in order to retrieve data from remote files.

read(out OctetSequence : data, in length : UnsignedLong) : void raises ( IOException)

Behavior.

SCA447 The read operation shall read, from the referenced file, the number of octets specified by the input length parameter and advance  the value of the filePointer attribute by the number of octets actually read.    SCA448 The read operation shall read less than the number of octets specified in the input-length parameter, when an end of file is encountered.

SCA449 The read operation shall return via the out Message parameter a OctetSequence that equals the number of octets actually read from the File.    SCA450 If the filePointer attribute value reflects the end of the File, the read operation shall return a 0-length OctetSequence.

Exceptions/Errors.

SCA451 The read operation shall raise the IOException when a read error occurs.

write.

Applications require the write operation in order to write data to remote files.

write(in data : OctetSequence) : void raises ( IOException);

Behavior.

SCA452 The write operation shall write data to the file referenced.    SCA453 If the write is successful, the write operation shall increment the filePointer attribute to reflect the number of octets written.    SCA454 If the write is unsuccessful, the filePointer attribute value shall maintain or be restored to its value prior to the write operation call.

This operation does not return any value.

Exceptions/Errors.

SCA455 The write operation shall raise the IOException when a write error occurs.

sizeOf.

An application may need to know the size of a file in order to determine memory allocation requirements.

sizeOf() : UnsignedLong raises ( FileException );

Behavior.

There is no significant behavior beyond the behavior described by the following section.

SCA456 The sizeOf operation shall return the number of octets stored in the file.

Exceptions/Errors.

SCA457 The sizeOf operation shall raise the FileException when a file-related error occurs (e.g.,   file does not exist anymore).

close.

The close operation is needed in order to release file resources once they are no longer needed.

close() : void raises ( FileException );

Behavior.

SCA458 The close operation shall release any OE file resources associated with the component.    SCA459 The close operation shall make the file unavailable to the component.

This operation does not return any value.

Exceptions/Errors.

SCA460 The close operation shall raise the FileException when it cannot successfully close the file.

setFilePointer.

The setFilePointer operation positions the file pointer where the next read or write will occur.

setFilePointer(in filePointer : UnsignedLong) : void raises ( InvalidFilePointer, FileException );

Behavior.

SCA461 The setFilePointer operation shall set the filePointer attribute value to the input filePointer.

This operation does not return any value.

Exceptions/Errors.

SCA462 The setFilePointer operation shall raise the CP FileException when the file pointer for the referenced file cannot be set to the value of the input filePointer parameter.    SCA463 The setFilePointer operation shall raise the InvalidFilePointer exception when the value of the filePointer parameter exceeds the file size.

getFilePointer.

The getFilePointer operation returns the value of the current file pointer which contains the location where the next read or write will occur.

getFilePointer() : UnsignedLong

Behavior.

SCA461 The getFilePointer operation shall return the current position of the filePointer attribute.

getFileName.

The getFileName operation returns the value of the current file name.

getFileName() : String

Behavior.

SCA461 The getFileName operation shall return the current value of the fileName attribute.

#### FileSystemInt.

The FileSystemInt interface defines common operations that enable remote access to a physical file system.

UML.

Figure 4.  FileSystem Construction Dependencies UML

Types & Exceptions.

UnknownFileSystemProperties.

The UnknownFileSystemProperties exception indicates a set of properties unknown by the component.

Figure 5.  UnknownFileSystemProperties Construction Dependencies UML

fileSystemProperties Query Constants.

Constants are defined to be used for the query operation (see section ).

SIZE : constant String= “SIZE”

AVAILABLE_SPACE : constant String = “AVAILABLE_SPACE”

FileInformationType.

The FileInformationType indicates the information returned for a file. Not all the fields in the FileInformationType are applicable for all file systems. SCA464 At a minimum, the FileSystem shall support name, kind, and size information for a file.   Examples of other file properties that can be specified are created time, modified time, and last access time.

Figure 6.  FileInformationType Construction Dependencies UML

name: This field indicates the simple name of the file.

kind: This field indicates the type of the file entry.

size: This field indicates the size in octets.

FileInformationSequence.

The FileInformationSequence type defines an unbounded sequence of FileInformationTypes.

Figure 7.  FileInformationSequence Construction Dependencies UML

FileType.

The FileType indicates the type of file entry. A file system can have PLAIN or DIRECTORY files and mounted file systems contained in a FileSystem.

Figure 8.  FileType Construction Dependencies UML

CREATED_TIME_ID.

The CREATED_TIME_ID is the identifier for the created time file property. A created time property indicates the time the file was created. SCA465 The value for created time shall be unsigned long long and measured in seconds since 00:00:00 UTC, Jan.   1, 1970.

CREATED_TIME_ID : constant String = “CREATED_TIME”;

MODIFIED_TIME_ID.

The MODIFIED_TIME_ID is the identifier for the modified time file property. The modified time property is the time the file data was last modified. SCA466 The value for modified time property shall be unsigned long long and measured in seconds since 00:00:00 UTC, Jan.   1, 1970.

MODIFIED_TIME_ID : constant String =”MODIFIED_TIME”;

LAST_ACCESS_TIME_ID.

The LAST_ACCESS_TIME_ID is the identifier for the last access time file property. The last access time property is the time the file was last access (e.g. read). SCA467 The value for last access time property shall be unsigned long long and measured in seconds since 00:00:00 UTC, Jan.   1, 1970.

LAST_ACCESS_TIME_ID : constant String = ”LAST_ACCESS_TIME”;

##### Operations

remove.

The remove operation provides the ability to remove a file from a file system.

remove(in fileName : String fileName) : void raises( FileException, InvalidFileName );

copy.

The copy operation provides the ability to copy a file to another file.

copy(in sourceFileName : String, in destinationFileName : String) : void raises( InvalidFileName, FileException );

exists.

The exists operation provides the ability to verify the existence of a file within a FileSystem.

exists(in fileName : String) : Boolean raises( InvalidFileName );

list.

The list operation provides the ability to obtain a list of files along with their information in the FileSystem according to a given search pattern. The list operation can be used to return information for one file or for a set of files.

list(in pattern : String) : FileInformationSequence raises (FileException, InvalidFileName);

create.

The create operation provides the ability to create a new file on the FileSystem.

create(in filename : String) : File raises( InvalidFileName, FileException );

open.

The open operation provides the ability to open a file for read or write.

open(in filename : String, in read_Only : Boolean) : File raises( InvalidFileName,  FileException );

mkdir.

The mkdir operation provides the ability to create a directory on the file system.

mkdir(in directoryName : String) : void raises( InvalidFileName, FileException );

rmdir.

The mkdir operation provides the ability to remove a directory from the file system.

rmdir(in directoryName : String) : void raises( InvalidFileName, FileException );

query.

The query operation provides the ability to retrieve information about a file system.

query(inout fileSystemProperties : Properties) : void raises( UnknownFileSystemProperties );

#### FileSystem.

The FileSystem class realizes the FileSystemInt interface that defines common operations and behavior that enable remote access to a physical file system.

UML.

Figure 9.  FileSystem Construction Dependencies UML

##### Operations

remove.

The remove operation provides the ability to remove a file from a file system.

remove(in filename : String) : void raises( FileException, InvalidFileName );

Behavior.

SCA468 The remove operation shall remove the file with the given filename.

This operation does not return any value.

Exceptions/Errors.

SCA469 The remove operation shall raise the InvalidFileName exception when the filename is not a valid filename or not an absolute pathname.

SCA470 The remove operation shall raise the FileException when a file-related error occurs.

copy.

The copy operation provides the ability to copy a file to another file.

copy(in sourceFileName : String, in destinationFileName : String) : void raises( InvalidFileName, FileException );

Behavior.

SCA471 The copy operation shall copy the source file with the specified sourceFileName to the destination file with the specified destinationFileName.

This operation does not return any value.

Exceptions/Errors.

SCA472 The copy operation shall raise the FileException when a file-related error occurs.

SCA473 The copy operation shall raise the InvalidFileName exception when the filename is not a valid file name or not an absolute pathname.

exists.

The exists operation provides the ability to verify the existence of a file within a FileSystem.

exists(in fileName : String) : Boolean raises( InvalidFileName );

Behavior.

SCA474 The exists operation shall check to see if a file exists based on the fileName parameter.

SCA475 The exists operation shall return True if the file exists, or False if it does not.

Exceptions/Errors.

SCA476 The exists operation shall raise the InvalidFileName exception when fileName is not a valid file name or not an absolute pathname.

list.

The list operation provides the ability to obtain a list of files along with their information in the FileSystem according to a given search pattern. The list operation can be used to return information for one file or for a set of files.

list(in pattern : String) : FileInformationSequence raises (FileException, InvalidFileName);

Behavior.

SCA477 The list operation shall return a list of file information based upon the search pattern given.    SCA478 The list operation shall support the following wildcard characters for base file names (i.e.,   the part after the right-most slash):

(1) * used to match any sequence of characters (including null).

(2) ? used to match any single character.

These wildcards may only be applied to the base filename in the search pattern given.  For example, the following are valid search patterns:

/tmp/files/*.*	Returns all files and directories within the /tmp/files directory.  Directory names indicated with a “/” at the end of the name.

/tmp/files/foo*	Returns all files beginning with the letters “foo” in the /tmp/files directory.

/tmp/files/f??	Returns all 3 letter files beginning with the letter f in the /tmp/files directory.

SCA479 The list operation shall return a FileInformationSequence for files that match the wildcard specification as specified in the input pattern parameter.   The list operation will return a zero length sequence when no file matching occurred for the input pattern parameter.

Exceptions/Errors.

SCA480 The list operation shall raise the InvalidFileName exception when the input pattern does not start with a slash "/" or cannot be interpreted due to unexpected characters.

SCA481 The list operation shall raise the FileException when a file-related error occurs.

create.

The create operation provides the ability to create a new file on the FileSystem.

create(in filename : String) : File raises( InvalidFileName, FileException );

Behavior.

SCA482 The create operation shall create a new File based upon the provided fileName attribute.

SCA483 The create operation shall return a File component reference to the opened file.    SCA484 The create operation shall return a null file component reference if an error occurs.

Exceptions/Errors.

SCA485 The create operation shall raise the FileException if the file already exists or another file error occurred.

SCA486 The create operation shall raise the InvalidFileName exception when a fileName is not a valid file name or not an absolute pathname.

open.

The open operation provides the ability to open a file for read or write.

open(in filename : String, in read_Only : Boolean) : File raises( InvalidFileName,  FileException );

Behavior.

SCA487 The open operation shall open a file based upon the input fileName.    The read_Only parameter indicates if the file should be opened for read access only.  SCA488 The open operation shall open the file for write access when the read_Only parameter is false.

SCA489 The open operation shall return a File component parameter on successful completion.    SCA490 The open operation shall return a null file component reference if the open operation is unsuccessful.    If the file is opened with the read_Only flag set to true, then writes to the file will be considered an error.

Exceptions/Errors.

SCA491 The open operation shall raise the FileException if the file does not exist or another file error occurred.

SCA492 The v operation shall raise the InvalidFileName exception when the filename is not a valid file name or not an absolute pathname.

mkdir.

The mkdir operation provides the ability to create a directory on the file system.

mkdir(in directoryName : String) : void raises( InvalidFileName, FileException );

Behavior.

SCA493 The mkdir operation shall create a FileSystem directory based on the directoryName given.    SCA494 The mkdir operation shall create all parent directories required to create the directoryName path given.

This operation does not return any value.

Exceptions/Errors.

SCA495 The mkdir operation shall raise the FileException if a file-related error occurred during the operation.

SCA496 The mkdir operation shall raise the InvalidFileName exception when the directoryName is not a valid directory name.

rmdir.

The rmdir operation provides the ability to remove a directory from the file system.

rmdir(in directoryName : String) : void raises( InvalidFileName, FileException );

Behavior.

SCA497 The mkdir operation shall remove a FileSystem directory, based on the directoryName given, only if the directory is empty (no files exist in directory).

This operation does not return any value.

Exceptions/Errors.

SCA498 The mkdir operation shall raise the FileException when the directory does not exist, if the directory is not empty, or another file-related error occurred.

SCA499 The mkdir operation shall raise the InvalidFileName exception when the directoryName is not a valid directory name.

query.

The query operation provides the ability to retrieve information about a file system.

query(inout fileSystemProperties : Properties) : void raises( UnknownFileSystemProperties );

Behavior.

SCA500 The query operation shall return file system information to the calling client based upon the given fileSystemProperties' ID.

SCA501 As a minimum, the FileSystem query operation shall support the following fileSystemProperties:

- SIZE – an ID value of “SIZE causes query to return an unsigned long long containing the file system size (in octets).

- AVAILABLE SPACE – an ID value of “AVAILABLE SPACE” causes the query operation to return an unsigned long long containing the available space on the file system (in octets),

See section  for the constants for the fileSystemProperties.

This operation does not return any value.

Exceptions/Errors.

SCA502 The query operation shall raise the UnknownFileSystemProperties exception when the given file system property is not recognized.

#### FileManager.

Multiple, distributed FileSystems may be accessed through a FileManager.  The FileManager class appears to be a single FileSystem although the actual file storage may span multiple physical file systems.

This is called a federated file system.  A federated file system is created using the mount and unmount operations.  Typically, the DomainManager or system initialization software will invoke these operations.

The FileManager realizes the FileSystemInt interface.  Based upon the pathname of a directory or file and the set of mounted FileSystems, the FileManager will delegate the FileSystem operations to the appropriate FileSystem.  For example, if a FileSystem is mounted at /ppc2, an open operation for a file called /ppc2/profile.xml would be delegated to the mounted FileSystem.  The mounted FileSystem will be given the filename relative to it.  In this example the FileSystem’s open operation would receive /profile.xml as the fileName argument.

Another example of this concept can be shown using the copy operation.  When a client invokes the copy operation, the FileManager will delegate operations to the appropriate FileSystems (based upon supplied pathnames) thereby allowing copy of files between FileSystems.

If a client does not need to mount and unmount FileSystems, it can treat the FileManager as a FileSystem by widening a FileManager reference to a FileSystem reference.  One can always widen a FileManager to a FileSystem since the FileManager is derived from a FileSystem.

UML.

Figure-10  FileManager Construction Dependencies UML

MountType.

SCA503 The MountType structure shall identify the FileSystems mounted within the FileManager.

Figure-11.  MountType Construction Dependencies UML

MountSequence.

The MountSequence is an unbounded sequence of MountTypes.

Figure-12.  MountSequence Construction Dependencies UML

NonExistentMount.

The NonExistentMount exception indicates a mount point does not exist within the FileManager.

MountPointAlreadyExists.

The MountPointAlreadyExists exception indicates the mount point is already in use in the FileManager.

InvalidFileSystem.

The InvalidFileSystem exception indicates the FileSystem is a null (nil) object reference.

##### Operations

mount.

The FileManager supports the notion of a federated file system.  To create a federated file system, the mount operation associated a FileSystem with a mount point (a directory name).

mount(in string mountPoint, in FileSystem file_System) raises( InvalidFileName, InvalidFileSystem, MountPointAlreadyExists );

Behavior.

SCA504 The mount operation shall associate the specified FileSystem with the given mountPoint.    SCA505 A mountPoint name shall begin with a “/”.    A mountPoint name is a logical directory name for a FileSystem.

This operation does not return any value.

Exceptions/Errors.

SCA506 The mount operation shall raise the InvalidFileName exception when the input file name is invalid.

SCA507 The mount operation shall raise the MountPointAlreadyExists exception when the mountPoint already exists in the file manager.

SCA508 The mount operation shall raise the InvalidFileSystem exception when the input FileSystem is a null object reference.

unmount.

Mounted FileSystems may need to be removed from a FileManager.

void unmount(in string mountPoint) raises( NonExistentMount );

Behavior.

SCA509 The unmount operation shall remove a mounted FileSystem from the FileManager whose mounted name matches the input mountPoint name.

This operation does not return any value.

Exceptions/Errors.

SCA510 The unmount operation shall raise the NonExistentMount exception when the mountPoint does not exist.

getMounts.

File management user interfaces may need to list a FileManager’s mounted FileSystems.

MountSequence getMounts();

Behavior.

SCA511 The getMounts operation shall return a sequence of Mount structures that describe the mounted FileSystems.

The getMounts operation returns a sequence of Mount structures.

Exceptions/Errors.

This operation does not raise any exceptions.

File Manager’s FileSystem Operations.

SCA513 The FileManager’s inherited FileSystem operations behavior shall implement the requirements of the FileSystem operations against the mounted file systems.    The FileSystem operations ensure that the filename/directory arguments given are absolute pathnames relative to a mounted FileSystem.  SCA514 The FileManager’s FileSystem operations shall remove the FileSystem mounted name from the input fileName before passing the fileName to an operation on a mounted FileSystem.

SCA515 The FileManager shall use the mounted FileSystem for FileSystem operations based upon the mounted FileSystem name that exactly matches the input fileName to the lowest matching subdirectory.

query.

The inherited query operation provides the ability to retrieve the same information for a set of file systems.

query(inout fileSystemProperties : Properties) : void raises( UnknownFileSystemProperties );

Behavior.

SCA516 The query operation shall return the combined mounted file systems information to the calling client based upon the given input fileSystemProperties’ IDs.    SCA517 As a minimum, the query operation shall support the following input fileSystemProperties IDs:

- SIZE - a property item ID value of "SIZE" will cause the query operation to return the combined total size of all the mounted file system as an unsigned long long property value.

- AVAILABLE_SPACE - a property item ID value of "AVAILABLE_SPACE" will cause the query operation to return the combined total available space (in octets) of all the mounted file system as unsigned long long property value.

This operation does not return any value.

Exceptions/Errors.

SCA518 The query operation shall raise the UnknownFileSystemProperties exception when the input fileSystemProperties parameter contains an invalid property ID