### Hardware Device Specification

This package contains the specification for a hardware device's specification that contains hardware device Registration attributes, which are typically used by a Human Computer Interface application to display information about the device(s) resident in a Software Radio system.  The information is intended to provide hardware configuration and revision information to a radio operator or to radio maintenance personnel.  The elements that make up this specification and contained within this section are:

- Author

- HW_DeviceRegistration

- HW_DeviceSpecification

#### Author

An Author type is used to indicate the name of the person, the company, and the web page of the developer of the component deployment specification. The type consists of three attributes, zero to multiple name(s), and optional a company and/or webPage.

#### HW_DeviceRegistration

The HW_DeviceRegistration element provides device-specific information for a hardware device.  At a minimum, the HW_DeviceRegistration element must include a description, the manufacturer, the model number and the device’s hardware class(es). The HW_DeviceRegistraction definition can describe a simple hardware device or a composition hardware device made up different hardware devices.

UML.

Attributes.

childHwDevices

The optional childHwDevices attribute The childHwDevices element indicates additional device-specific information for hardware devices that make up the root or parent hardware device registration.  An example of childHwDevices would be a radio's RF module that has receiver and exciter functions within it.  The parent / child relationship indicates that when the RF module is removed from the system, the receiver and exciter devices are also removed.

description

The description attribute is text describing this hardware device registration.

deviceClasses

The deviceClasses element is used to identify one or more hardware classes that make up the device being installed (e.g., RF, Receiver, Exciter, Antenna, Power Amplifier, Modem, Processor, DSP, FPGA, GPP, I/O).

deviceProperties

The deviceProperties attribute specifies characteristics of this hardware device.

id

The id attribute is the unique identifier for this hardware device registration.  The id format is a DCE UUID.

manufacture

The manufacture attribute is is used to convey the name of manufacturer of the device being installed.

modelNumber

The modelnumber attribute is used to indicate the manufacture's model number, for the device being installed.

name

The name attribute is a user-friendly label for the hardware device being registered.

title

The title attribute is an optional title for this specification.

version

The version attribute is an optional version information for the hardware device registration. The format of the version string is numerical major and minor version numbers separated by commas (e.g., "1,0,0,0").

#### HW_DeviceSpecification

The HW_DeviceSpecification contains documentation and descriptive information on the definition of a hardware device.  Only one Specification may be referenced.

UML.

Attributes.

authors

The authors attribute is list of authors for this hardware device specification.

description

The description attribute is optional text describing this specification.

id

The id attribute is the unique identifier for this hardware device specification.  The id format is a DCE UUID.

hwDeviceRegistration

The hwDeviceRegistration attribute defines the characteristics of the hardware device.

name

The name attribute is a user-friendly label for this hardware device specification.

title

The title attribute is an optional title for this specification.

version

The optional version attribute is version information for this registration.